## Hardware Telnet / VTY Lab – High Level

Devices (example):

- 1 Cisco router (ISR 4xxx, 29xx, etc.)

- 1 Cisco Catalyst switch (2960/3560 etc.)

- 1–2 PCs/laptops with terminal emulator (PuTTY/TeraTerm) and NICs

Goal:

- Cable the devices, configure management IP on switch/ router, set up local user accounts, configure VTY lines for Telnet, and test Telnet from a PC.IP scheme (same as Packet Tracer version):

- Router LAN interface (e.g., G0/0): 192.168.1.1 /24

- Switch VLAN 1: 192.168.1.2 /24

- PC: 192.168.1.10 /24, default gateway 192.168.1.1

## Step-by-Step Procedure (Real Hardware)

## Part A – Physical setup

- Place the router, switch, and PC on the lab bench and power them on.

- Connect the PC NIC to a switch access port (e.g., Fa0/1) with a straight-through Ethernet cable.

- Connect the router LAN interface (e.g., G0/0) to another switch access port (e.g., Fa0/2) with a straight-through cable.

- Connect a console cable (RJ45–DB9 or USB-serial) from the PC to the router’s console port.

- Optionally, connect another console cable from PC to the switch, or move the same cable once the router is configured.

## Part B – Console access and basic prep

- On the PC, open PuTTY/TeraTerm and start a serial/COM session to the router console.

- Press Enter until you see the router prompt, e.g., Router>.

- If needed, perform a basic initial setup: set hostname, disable DNS lookup, etc. (you can script this for students).

## Part C – Configure IP addressing (router)

- Enter privileged EXEC mode on the router:

  - enable

- Enter global configuration mode:

  - configure terminal

- Configure the LAN interface (example: G0/0):

  - interface gigabitEthernet 0/0

  - ip address 192.168.1.1 255.255.255.0

  - no shutdown

- Exit to privileged mode:

  - end

- Verify:

  - show ip interface brief

  - Confirm G0/0 is up/up with IP 192.168.1.1

## Part D – Configure IP addressing (switch)

- Move the console cable to the switch, open a serial session, and reach the Switch> prompt.

- Enter privileged EXEC:

  - enable

- Enter global configuration:

  - configure terminal

- Configure management SVI on VLAN 1:

  - interface vlan 1

  - ip address 192.168.1.2 255.255.255.0

  - no shutdown

- Exit to privileged mode:

  - end

- Verify:

  - show ip interface brief

  - Confirm VLAN 1 is up/up with IP 192.168.1.2.

If students already have VLANs in the lab, you can require them to place the PC port into the correct access VLAN matching VLAN 1 (or whichever SVI you choose).

## Part E – Configure PC IP

- On the PC, open Network Adapter settings (Windows/Linux) and statically assign:

  - IP Address: 192.168.1.10

  - Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.1.1

- Save and close; optionally verify with ipconfig or ifconfig.

## Part F – Create local user accounts

## On router

- On the router (enable → configure terminal), create a user:

  - username cisco privilege 15 secret CCNA

- (Optional but recommended) set an enable secret:

  - enable secret class

- Exit to privileged mode:

  - end

## On switch

- On the switch (enable → configure terminal), create the same user:

  - username cisco privilege 15 secret CCNA

- (Optional) set enable secret:

  - enable secret class

- end

## Part G – Configure VTY lines for Telnet

## On router

- On the router, go to global config:

  - configure terminal

- Enter VTY line config:

  - line vty 0 4 (or 0 15 depending on platform)

- Configure:

  - login local

  - transport input telnet

- Optional hardening (advanced students):

  - exec-timeout 10 0 (idle timeout)

  - access-class <ACL-number> in (restrict source IPs)

- Exit:

  - end

- Verify with:

  - show running-config

  - Confirm VTY lines show login local and transport input telnet.

## On switch

- On the switch:

  - configure terminal

- VTY line config:

  - line vty 0 15

- Configure:

  - login local

  - transport input telnet

- (Optional) same timeout/access-class hardening as router.

- end and verify with show running-config.

## Part H – Test connectivity and Telnet

- From the PC, open Command Prompt/terminal and ping router:

  - ping 192.168.1.1

  - Expect replies.

- Ping switch:

  - ping 192.168.1.2

- Test Telnet to router:

  - telnet 192.168.1.1

- When prompted, enter:

  - Username: cisco

  - Password: CCNA

- Confirm you get router prompt (R1>), then go to privileged mode with enable and enter the enable secret.

- Exit Telnet session with exit.

- Test Telnet to switch:

  - telnet 192.168.1.2

- Log in with the same credentials and verify you reach the switch CLI.

- Exit.