Bank Network Design Lab: Network Topology Design Using Cisco Packet Tracer

Course: Enterprise Networking / CompTIA Network+ Preparation
Lab Number: Project 5 - Part 2
Student Name: __________________________________
Date: ________________________
Instructor: Piyali Ganguly

Lab Overview

This lab is Part 2 of an enterprise networking project focused on designing a bank network topology for a four-story building in Nairobi, Kenya. You will use Cisco Packet Tracer to build a hierarchical network design with core, distribution, and access layers supporting 12 departments across 4 floors[1].

Prerequisites: Completion of Part 1 (Microsoft Visio network diagram design)

Estimated Time: 90-120 minutes

Learning Objectives

By the end of this lab, you will be able to:

- Design and implement a hierarchical three-layer network topology (Core, Distribution, Access)

- Configure redundant connections between network layers for high availability

- Implement proper device naming conventions following enterprise standards

- Organize network topology by floors and departments using visual separation techniques

- Prepare router interfaces and add hardware modules for serial connectivity

- Understand full-mesh routing topology for redundancy

- Apply proper cabling standards between different network devices

- Document network design with color coding and labeling

Network Requirements

Building Structure

| Floor | Departments | VLAN Range |
| --- | --- | --- |
| First Floor | Management (VLAN 10), Research (VLAN 20), Human Resource (VLAN 30) | 10-30 |
| Second Floor | Marketing (VLAN 40), Accounting (VLAN 50), Finance (VLAN 60) | 40-60 |
| Third Floor | Logistics (VLAN 70), Customer Care (VLAN 80), Guest (VLAN 90) | 70-90 |
| Fourth Floor | Administration (VLAN 100), ICT (VLAN 110), Server Room (VLAN 120) | 100-120 |

Table 1: Floor and Department Organization

Network Hierarchy

- Core Layer: 4 routers (Cisco 2911) in full-mesh topology

- Distribution Layer: 4 Layer 3 switches (Cisco 3650) - one per floor

- Access Layer: 12 Layer 2 switches (Cisco 2960) - one per department

- End Devices: 1 PC, 1 Printer, 1 Wireless Access Point per department (except Server Room has 3 servers)

Required Equipment

Cisco Packet Tracer Devices

| Device Type | Quantity | Model |
| --- | --- | --- |
| Routers | 4 | Cisco 2911 |
| Layer 3 Switches | 4 | Cisco 3650-24PS |
| Layer 2 Switches | 12 | Cisco 2960 (or similar) |
| PCs | 33 | Generic PC |
| Printers | 11 | Generic Printer |
| Wireless Access Points | 11 | AccessPoint-PT |
| Servers | 3 | Generic Server |

Table 2: Network Equipment List

Hardware Modules

- HWIC-2T (High-Speed WAN Interface Card - 2 Serial Ports) - 4 modules for routers

- Power Supply modules for Cisco 3650 switches (if not pre-installed)

Lab Topology Diagram

The network follows a hierarchical three-layer design:

- Core Layer (Center): Four routers interconnected in full-mesh topology for redundancy

- Distribution Layer (Middle): Four Layer 3 switches, each connected to two core routers for redundancy

- Access Layer (Bottom): Twelve Layer 2 switches, each connected to two distribution switches for redundancy

- End Devices: Connected to access layer switches in star topology

Part 1: Initial Packet Tracer Setup

Step 1: Create New Project File

- Open Cisco Packet Tracer

- Click File → New to create a new project

- Click Options → Preferences → Interface and set background color to your preference (instructor used light blue)

- Save the file as: Enterprise_Networking_Project5.pkt

Question 1: Why is it important to use descriptive file names for network projects?

________________________________________________________________________

Part 2: Building the Core Layer

Step 2: Place Core Routers

- From the device panel at bottom, click on Routers (orange icon)

- Select 2911 router model

- Place 4 routers in the center of the workspace in a diamond/square formation

- Position them with adequate spacing for cable connections

Step 3: Add Serial Interface Modules

Important: Cisco 2911 routers do not have serial interfaces by default. You must add HWIC-2T modules.

- Click on Router0 (any router in core layer)

- Click the Physical tab

- Locate the power button (shown as 1/0 with a circle) in the interface

- Click the power button to turn OFF the router (wait for shutdown)

- In the Modules panel on left, locate HWIC-2T module

- Drag and drop HWIC-2T module into an empty slot (right side of router)

- Click the power button to turn ON the router

- Click the Config tab, then click each interface on the left:

  - GigabitEthernet0/0 - Check the Port Status: On checkbox

  - GigabitEthernet0/1 - Check the Port Status: On checkbox

  - Serial0/0/0 - Check the Port Status: On checkbox (newly added)

  - Serial0/0/1 - Check the Port Status: On checkbox (newly added)

- Close the router configuration window

- Repeat Steps 1-9 for the remaining 3 routers

Question 2: Why must you power off a router before adding or removing hardware modules?

________________________________________________________________________

Step 4: Connect Routers with Serial Cables (Partial Mesh)

- Click the Connections icon (lightning bolt) at bottom

- Select Serial DCE cable (solid black line)

- Click on Router0, select Serial0/0/0

- Click on Router1 (adjacent router), select Serial0/0/0

- Observe the connection - both ends should show orange initially, then turn green

- Repeat to create the following serial connections:

  - Router1 Serial0/0/1 ↔ Router2 Serial0/0/0

  - Router2 Serial0/0/1 ↔ Router3 Serial0/0/0

  - Router3 Serial0/0/1 ↔ Router0 Serial0/0/1

This creates a ring topology between the four routers.

Step 5: Add Full-Mesh Redundancy (Crossover Cables)

To achieve true redundancy, add diagonal connections using Ethernet crossover cables:

- Select Copper Cross-Over cable (red dashed line)

- Connect Router0 GigabitEthernet0/0 ↔ Router2 GigabitEthernet0/0

- Connect Router1 GigabitEthernet0/0 ↔ Router3 GigabitEthernet0/0

Question 3: What is the purpose of full-mesh topology in the core layer? What happens if one link fails?

________________________________________________________________________

________________________________________________________________________

Part 3: Building the Distribution Layer

Step 6: Place Layer 3 Switches

- Click on Switches icon (blue icon) at bottom

- Select 3650-24PS switch model

- Place 4 Layer 3 switches - two on the left side and two on the right side of the core routers

- Arrange them vertically with adequate spacing

Step 7: Add Power Supplies to Layer 3 Switches

Important: Cisco 3650 switches may require power supply modules.

- Click on any Layer 3 switch

- Go to Physical tab

- Check if power supply slots are empty

- If empty, drag a power supply module from the Modules panel to an empty power supply slot

- Close the window

- Repeat for all 4 Layer 3 switches

Step 8: Connect Distribution Layer to Core Layer

Connect each Layer 3 switch to two routers for redundancy:

- Select Copper Straight-Through cable (solid black line) or click Automatically Choose Connection Type

- Connect switches to routers using the following pattern:

  - Floor1-L3-Switch (left top) → Router0 GigabitEthernet0/1

  - Floor1-L3-Switch → Router1 GigabitEthernet0/1

  - Floor2-L3-Switch (left bottom) → Router1 (available GigabitEthernet)

  - Floor2-L3-Switch → Router2 (available GigabitEthernet)

  - Floor3-L3-Switch (right top) → Router2 (available GigabitEthernet)

  - Floor3-L3-Switch → Router3 (available GigabitEthernet)

  - Floor4-L3-Switch (right bottom) → Router3 (available GigabitEthernet)

  - Floor4-L3-Switch → Router0 (available GigabitEthernet)

Note: Each Layer 3 switch should have exactly 2 connections to the core layer.

Question 4: Why is each distribution switch connected to two different core routers?

________________________________________________________________________

Part 4: Building the Access Layer

Step 9: Place Layer 2 Switches

- Select Switches → 2960 (or any Layer 2 switch)

- Place 3 switches below the left-side distribution switches (Floor 1 and Floor 2)

- Place 3 switches below the left-side distribution switches (continue for Floor 2)

- Place 3 switches below the right-side distribution switches (Floor 3)

- Place 3 switches below the right-side distribution switches (Floor 4)

- Total: 12 Layer 2 switches (3 per floor)

- Arrange them in groups of 3 for better organization

Step 10: Connect Access Layer to Distribution Layer

Connect each access switch to two distribution switches for redundancy:

- Use Copper Straight-Through cable or Auto connection

- Connect the 3 switches for Floor 1 to both:

  - Floor1-L3-Switch (primary)

  - Floor2-L3-Switch (secondary backup)

- Connect the 3 switches for Floor 2 to both:

  - Floor2-L3-Switch (primary)

  - Floor1-L3-Switch (secondary backup)

- Connect the 3 switches for Floor 3 to both:

  - Floor3-L3-Switch (primary)

  - Floor4-L3-Switch (secondary backup)

- Connect the 3 switches for Floor 4 to both:

  - Floor4-L3-Switch (primary)

  - Floor3-L3-Switch (secondary backup)

Question 5: Some links will show orange (blocked) due to Spanning Tree Protocol. Explain why this happens and what STP prevents.

________________________________________________________________________

________________________________________________________________________

Part 5: Adding End Devices

Step 11: Connect End Devices to Access Switches

For each of the 11 departments (all except Server Room):

- Select End Devices icon

- Place the following devices near each access switch:

  - 1 PC (Generic PC)

  - 1 Printer (Generic Printer)

  - 1 Wireless Access Point (from Wireless Devices → AccessPoint-PT)

- Use Copper Straight-Through cable to connect each device to the access switch

- Connect to FastEthernet ports on the switch

Step 12: Connect Server Room (Floor 4)

For the Server Room department on Floor 4:

- Remove the PC and Printer that were placed

- Select End Devices → Server

- Place 3 servers near the Server Room switch

- Connect all 3 servers to the Server Room access switch using Copper Straight-Through cables

- Leave the wireless access point connected

Question 6: Why does the Server Room have 3 servers instead of PCs and a printer?

________________________________________________________________________

Part 6: Device Naming Convention

Step 13: Name Core Layer Devices

- Click on each router in the core layer

- Click the Config tab

- In the Display Name field, enter the appropriate name:

  - Floor1-Router

  - Floor2-Router

  - Floor3-Router

  - Floor4-Router

- Press Enter to apply the name change

Step 14: Name Distribution Layer Devices

- Click on each Layer 3 switch

- In the Display Name field, enter:

  - Floor1-Layer3-Switch

  - Floor2-Layer3-Switch

  - Floor3-Layer3-Switch

  - Floor4-Layer3-Switch

Step 15: Name Access Layer Devices by Department

Use the following naming convention for Layer 2 switches:

| Floor | Switch Names |
| --- | --- |
| Floor 1 | Floor1-Management-Switch, Floor1-Research-Switch, Floor1-HR-Switch |
| Floor 2 | Floor2-Marketing-Switch, Floor2-Accounting-Switch, Floor2-Finance-Switch |
| Floor 3 | Floor3-Logistics-Switch, Floor3-CustomerCare-Switch, Floor3-Guest-Switch |
| Floor 4 | Floor4-Administration-Switch, Floor4-ICT-Switch, Floor4-ServerRoom-Switch |

Table 3: Access Layer Switch Naming Convention

Question 7: What are the benefits of using a consistent naming convention in enterprise networks?

________________________________________________________________________

________________________________________________________________________

Step 16: Name End Devices

Use department abbreviations for end device naming:

| Department | PC Name | Printer Name | Access Point Name |
| --- | --- | --- | --- |
| Management | Mgmt-PC | Mgmt-Printer | Mgmt-AP |
| Research | Res-PC | Res-Printer | Res-AP |
| Human Resource | HR-PC | HR-Printer | HR-AP |
| Marketing | Mkt-PC | Mkt-Printer | Mkt-AP |
| Accounting | Acct-PC | Acct-Printer | Acct-AP |
| Finance | Fin-PC | Fin-Printer | Fin-AP |
| Logistics | Log-PC | Log-Printer | Log-AP |
| Customer Care | CC-PC | CC-Printer | CC-AP |
| Guest | Guest-PC | Guest-Printer | Guest-AP |
| Administration | Admin-PC | Admin-Printer | Admin-AP |
| ICT | ICT-PC | ICT-Printer | ICT-AP |
| Server Room | Server1, Server2, Server3 | (none) | SR-AP |

Table 4: End Device Naming Convention

Part 7: Visual Organization and Documentation

Step 17: Add Background Color Coding by Floor

- Click Add Simple PDU tool, then click Select tool to deselect

- Click the rectangle/shape tool at the top toolbar

- Select Fill Color and choose a color for Floor 1

- Draw a rectangle around all Floor 1 devices (Layer 3 switch, 3 access switches, and end devices)

- Right-click the rectangle → To Back (to move it behind devices)

- Repeat for Floors 2, 3, and 4 using different colors:

  - Floor 1: Light color (e.g., light blue)

  - Floor 2: Different shade (e.g., light green)

  - Floor 3: Another shade (e.g., light yellow)

  - Floor 4: Another shade (e.g., light pink)

Step 18: Add Department Boundaries

- Use the rectangle tool with white fill color

- Draw smaller rectangles around each department's devices (access switch + end devices)

- Move rectangles To Back

- This creates visual separation of departments within each floor

Step 19: Add Text Labels

- Click the text (ABC) tool at top

- Click in the workspace and type floor labels:

  - "First Floor" (above Floor 1 section)

  - "Second Floor" (above Floor 2 section)

  - "Third Floor" (above Floor 3 section)

  - "Fourth Floor" (above Floor 4 section)

- Add department labels with VLAN information:

  - "Management - VLAN 10"

  - "Research - VLAN 20"

  - (Continue for all 12 departments with their VLAN IDs)

Step 20: Add Border Lines

- Use the line tool at the top

- Select a bright color (e.g., yellow)

- Draw boundary lines between floors for clear visual separation

Part 8: Verify Connectivity and Interface Status

Step 21: Check Interface Status

- Click on any router

- Verify that all connected interfaces show green (up/up state)

- If any interfaces are red, click Config tab and enable Port Status: On for those interfaces

- Repeat verification for all routers and switches

- Some links between access and distribution switches may show orange - this is normal (Spanning Tree Protocol blocking state)

Question 8: What do the following link colors indicate in Cisco Packet Tracer?

- Green: __________________________________

- Orange: __________________________________

- Red: __________________________________

Step 22: Export Topology Diagram

- Click File → Print

- Select Print to File

- Choose a location and filename: Project5_Bank_Network_Topology.png

- Click Save

- Verify the exported image clearly shows all devices, connections, and labels

Part 9: Documentation and Submission

Step 23: Complete Network Inventory

Fill out the following inventory table based on your completed topology:

| Device Category | Quantity | Total Ports Used |
| --- | --- | --- |
| Routers | ____ | ____ |
| Layer 3 Switches | ____ | ____ |
| Layer 2 Switches | ____ | ____ |
| PCs | ____ | N/A |
| Printers | ____ | N/A |
| Access Points | ____ | N/A |
| Servers | ____ | N/A |
| Total Devices | ____ |  |

Table 5: Network Device Inventory

Step 24: Answer Review Questions

Question 9: Explain the three-layer hierarchical network design and the role of each layer.

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

Question 10: How many redundant paths exist between a PC in the Management department and a PC in the Finance department? Trace the possible paths.

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

Question 11: Calculate the total number of network connections (cables) in your topology.

________________________________________________________________________

Question 12: Why are servers placed in a dedicated Server Room rather than distributed across departments?

________________________________________________________________________

________________________________________________________________________

Lab Submission Checklist

Before submitting your lab, ensure you have completed:

- All 4 routers placed and configured with serial modules

- Full-mesh connectivity established in core layer

- All 4 Layer 3 switches placed with power supplies

- Redundant connections between distribution and core layers

- All 12 Layer 2 switches placed (one per department)

- Redundant connections between access and distribution layers

- All end devices connected (33 PCs, 11 printers, 11 APs, 3 servers)

- All devices named according to naming convention

- All router and switch interfaces enabled (showing green)

- Background color coding applied to all floors

- Department boundaries marked with white rectangles

- Text labels added for floors and departments with VLANs

- Border lines drawn between floors

- Topology diagram exported as PNG/JPG file

- All review questions answered completely

- Network inventory table completed

- Packet Tracer .pkt file saved with correct filename

Troubleshooting Guide

Problem: Serial interfaces not appearing on routers

Solution: Power off router → Add HWIC-2T module → Power on router → Verify in Config tab

Problem: Layer 3 switch ports showing red

Solution: Add power supply module to empty power supply slot on the switch

Problem: Links between devices show red

Solution:

- Check both devices are powered on

- Verify correct cable type used

- Enable Port Status in Config tab for both interfaces

- Wait 30-60 seconds for interface to come up

Problem: Some links showing orange instead of green

Solution: This is normal - Spanning Tree Protocol blocks redundant links to prevent loops. Orange indicates blocked/standby state.

Problem: Cannot connect devices after many connections made

Solution: You may have used all available ports on a switch. Verify port availability or use a switch with more ports.

Next Steps: Part 3 Configuration

In Part 3 of this project, you will configure the following technologies on this topology:

- VLANs: Create and assign VLANs 10-120 for all departments

- IP Addressing: Implement subnetting scheme for all networks

- DHCP Server: Configure dynamic IP address assignment

- Inter-VLAN Routing: Enable Layer 3 switching on distribution layer

- SSH: Secure remote access to network devices

- Port Security: MAC address filtering on access layer

- Basic Security: Passwords, banners, and access control

- Routing Protocols: Implement OSPF or EIGRP in core layer

- Testing: End-to-end connectivity verification

References

[1] Gurutech Cloud. (2025, May 6). Bank Network Design Part 2: Network Topology Design Using Cisco Packet Tracer [Video]. YouTube. https://youtu.be/rVsWMUcflOA

Additional Resources

- Cisco Packet Tracer Help Documentation (Help → Contents)

- CompTIA Network+ Objectives 1.0 (Networking Concepts) and 2.0 (Network Implementations)

- Cisco Three-Layer Hierarchical Network Design Model

- Spanning Tree Protocol (802.1D) Overview

- Cisco Device Naming Best Practices

Instructor Notes:

________________________________________________________________________

________________________________________________________________________

Grade: ________ / 100

Instructor Signature: ______________________________ Date: ______________