# Building a complete, realistic enterprise-style network using a Catalyst 3560 switch, a Cisco 1911 router, a server, and a firewall.

┌──────────────────────────┐

│        Internet / ISP     │

└───────────────┬───────────┘

│

Public IP │

│

┌───────────────▼──────────────┐

│         FIREWALL              │

│  Outside: Public/WAN          │

│  Inside: 10.0.0.1/30          │

│  NAT + ACLs                   │

└───────────────┬──────────────┘

│ 10.0.0.2/30

│

┌───────────────▼──────────────┐

│        CISCO 1911 ROUTER      │

│  G0/0: 802.1Q trunk to switch │

│  G0/1: Inside to firewall     │

│  VLAN10 GW: 192.168.10.1      │

│  VLAN20 GW: 192.168.20.1      │

│  VLAN30 GW: 192.168.30.1      │

└───────────────┬──────────────┘

│ dot1q trunk

│

┌───────────────▼──────────────┐

│      CATALYST 3560 SWITCH     │

│  VLAN10 – Users               │

│  VLAN20 – Servers             │

│  VLAN30 – Management          │

│  SVI: 192.168.30.2            │

└───────┬─────────┬────────────┘

│         │

┌─────────────┘         └──────────────┐

│                                        │

VLAN 10 (Users)                               VLAN 20 (Servers)

192.168.10.0/24                                   192.168.20.0/24

GW: 192.168.10.1                                  GW: 192.168.20.1

Example PC:                                       Example Server:

192.168.10.50                                      192.168.20.10

─────────────────────────────────────────────────────────────────────

VLAN 30 (Management)

192.168.30.0/24

GW: 192.168.30.1

Switch SVI: 192.168.30.2

# Those four devices are used to model a production-grade network with routing, switching, security, and services.

Design Topology:

- VLAN 10 (Users): 192.168.10.0/24

- VLAN 20 (Servers): 192.168.20.0/24

- VLAN 30 (Management): 192.168.30.0/24

- Router–Switch link: G0/0 on router ↔ Gi0/1 on switch (802.1Q trunk)

- Router inside interface to firewall: G0/1 → 10.0.0.2/30

- Firewall inside: 10.0.0.1/30

- Firewall outside: public/WAN (from ISP or simulated)

### 1. Catalyst 3560 configuration (VLANs + trunk to router)

! Create VLANs

conf t

vlan 10

name USERS

vlan 20

name SERVERS

vlan 30

name MGMT

exit

! Trunk to router on Gi0/1

interface GigabitEthernet0/1

description TRUNK_TO_ROUTER_1911

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20,30

no shutdown

! Example access ports

interface GigabitEthernet0/2

description USER_PC_VLAN10

switchport mode access

switchport access vlan 10

no shutdown

interface GigabitEthernet0/3

description SERVER_VLAN20

switchport mode access

switchport access vlan 20

no shutdown

interface Vlan30

description SWITCH_MANAGEMENT

ip address 192.168.30.2 255.255.255.0

no shutdown

exit

! Default gateway for the switch itself (management)

ip default-gateway 192.168.30.1

end

wr mem

### 2. Cisco 1911 router configuration (router-on-a-stick + default route)

Assume:

- G0/0 connects to the switch (trunk)

- G0/1 connects to firewall (inside)

- Router is default gateway for all VLANs

conf t

hostname R1

! Physical interface toward switch

interface GigabitEthernet0/0

description TRUNK_TO_3560

no shutdown

! Subinterface for VLAN 10

interface GigabitEthernet0/0.10

description VLAN10_USERS

encapsulation dot1q 10

ip address 192.168.10.1 255.255.255.0

! Subinterface for VLAN 20

interface GigabitEthernet0/0.20

description VLAN20_SERVERS

encapsulation dot1q 20

ip address 192.168.20.1 255.255.255.0

! Subinterface for VLAN 30

interface GigabitEthernet0/0.30

description VLAN30_MGMT

encapsulation dot1q 30

ip address 192.168.30.1 255.255.255.0

! Interface toward firewall (inside)

interface GigabitEthernet0/1

description TO_FIREWALL_INSIDE

ip address 10.0.0.2 255.255.255.252

no shutdown

! Default route to firewall

ip route 0.0.0.0 0.0.0.0 10.0.0.1

! (Optional) basic inter-VLAN ACL example – allow all for now

! You can later add ACLs on subinterfaces if you want filtering.

end

wr mem

### 3. Firewall high-level setup

- Inside interface: 10.0.0.1/30, security level high

- Outside interface: public IP / ISP side

- Default route: to ISP gateway on outside

- NAT: inside 10.0.0.0/8 or specific 192.168.10.0/24, 192.168.20.0/24, 192.168.30.0/24 to outside

- ACLs:

  - Allow outbound from inside to outside

  - Allow return traffic

  - Optionally allow specific inbound services (e.g., if you later put a server in a DMZ)

Traffic path:

- PC in VLAN 10 → default gateway 192.168.10.1 (router)

- Router routes to 0.0.0.0/0 via 10.0.0.1 (firewall)

- Firewall NATs and sends to Internet

### 4. Server configuration idea

Put the server on VLAN 20:

- IP: 192.168.20.10

- Mask: 255.255.255.0

- Gateway: 192.168.20.1 (router subinterface)

If you want the server to provide DHCP for all VLANs:

- On the router, add ip helper-address on each subinterface pointing to the server:

conf t

interface GigabitEthernet0/0.10

ip helper-address 192.168.20.10

interface GigabitEthernet0/0.30

ip helper-address 192.168.20.10

end

wr mem

- On the server, create scopes for:

  - 192.168.10.0/24 (gateway 192.168.10.1)

  - 192.168.20.0/24 (gateway 192.168.20.1)

  - 192.168.30.0/24 (gateway 192.168.30.1)

### 5. Basic verification steps

On the switch:

show vlan brief

show interfaces trunk

On the router:

show ip interface brief

show ip route

ping 192.168.10.1

ping 192.168.20.10

ping 10.0.0.1

From a PC in VLAN 10:

- Ping 192.168.10.1 (gateway)

- Ping 192.168.20.10 (server)

- Ping a public IP (once firewall/ISP side is set)

Classic Enterprise Edge Model (Fully Built Out)

This model uses the four devices exactly as in your document and mirrors how many small–medium enterprises design their networks. It keeps routing on the router, switching on the switch, and security on the firewall.

Core idea

A Layer 2 switch, a router-on-a-stick, and a firewall at the WAN edge.
Your document already reflects this with lines like:

“Router–Switch link: G0/0 on router ↔ Gi0/1 on switch (802.1Q trunk)”
“Firewall inside: 10.0.0.1/30”

Logical Architecture

VLANs

- VLAN 10 – Users: 192.168.10.0/24

- VLAN 20 – Servers: 192.168.20.0/24

- VLAN 30 – Management: 192.168.30.0/24

Routing

- Router performs inter-VLAN routing using subinterfaces:

“interface GigabitEthernet0/0.10 … encapsulation dot1q 10”

Security

- Firewall protects the inside network and performs NAT:

“NAT: inside 10.0.0.0/8 or specific 192.168.x.x to outside”

Physical Topology Model

Internet / ISP

|

[ Firewall Outside ]

|

[ Firewall Inside ]

| 10.0.0.1/30

|

[ Cisco 1911 Router ]

| 10.0.0.2/30

|

trunk (dot1q)

|

[ Catalyst 3560 Switch ]

|             |               |

VLAN10         VLAN20          VLAN30

Users          Servers         Management

This is the cleanest, most realistic model for your hardware.

Functional Model (What each device does)

1. Catalyst 3560 (Access + VLAN segmentation)

- Creates VLANs

- Assigns ports to VLANs

- Provides management SVI (VLAN 30)

- Trunks to router

- Does not route between VLANs

Your document shows this clearly:

“switchport trunk allowed vlan 10,20,30”
“interface Vlan30 … ip address 192.168.30.2”

2. Cisco 1911 Router (Inter-VLAN routing + WAN handoff)

- Router-on-a-stick

- Subinterfaces for VLAN 10/20/30

- Default gateway for all VLANs

- Static default route to firewall

From your document:

“ip route 0.0.0.0 0.0.0.0 10.0.0.1”

3. Firewall (Security boundary + NAT)

- Inside: 10.0.0.1/30

- Outside: public IP

- NAT for all internal networks

- ACLs for outbound/inbound traffic

- Optional DMZ later

Your document:

“Allow outbound from inside to outside”
“Optionally allow specific inbound services”

4. Server (Services + DHCP if needed)

- Lives in VLAN 20

- Can provide DHCP to all VLANs using helper addresses

Your document:

“ip helper-address 192.168.20.10”

Why this model is ideal for your lab

- Matches real enterprise edge networks

- Uses all four devices in meaningful roles

- Demonstrates VLANs, trunking, routing, NAT, ACLs, DHCP relay

- Perfect for Network+ and CCNA teaching

- Easy to expand into DMZ, VPN, HA firewall, or L3 switching later