Lab Worksheet: Network Design & Implementation for Campus Area Network (CAN)

Case Study: Albon University Network Requirements

Background

Albon University is a large educational institution with two campuses situated 20 miles apart. The university requires a comprehensive network infrastructure to support students and staff across four faculties.

Main Campus Structure

| Building | Departments/Faculties |
| --- | --- |
| Building A | Administrative Staff, HR, Finance, Faculty of Business |
| Building B | Faculty of Engineering & Computing, Faculty of Arts & Design |
| Building C | Student Labs, IT Department (hosting web/FTP servers) |

Table 1: Main Campus Building Layout

Branch Campus Structure

- Faculty of Health and Sciences

  - Staff Department (separate floor)

  - Student Labs (separate floor)

Network Requirements

- Each department must be on a separate IP network using VLANs

- Switches must be configured with appropriate VLANs and security settings

- RIP version 2 for internal routing between campuses

- Static routing for external server connectivity

- Building A devices must acquire dynamic IP addresses from router-based DHCP

- External email server hosted on the cloud

- Internal web and FTP servers in IT Department

- End-to-end connectivity between all departments and servers

Equipment and Resources Required

Cisco Packet Tracer Devices

| Device Type | Quantity | Purpose |
| --- | --- | --- |
| 2901 ISR Router | 3 | Main Campus, Branch Campus, Cloud Router |
| 3650-24PS Layer 3 Switch | 2 | Distribution layer switches (Main & Branch) |
| 2960-24TT Layer 2 Switch | 10 | Access layer switches (per department) |
| Generic PC | 10+ | End user devices (2 per department) |
| Generic Server | 4 | Web, FTP, Email, DNS servers |
| Printer | 10+ | Department printers (1-2 per dept) |
| HWIC-2T Serial Module | 6 | For router serial interfaces |
| AC Power Supply | 2 | For Layer 3 switches |

Table 2: Required Network Equipment

Cables Required

- Serial DCE/DTE cables (for WAN connections)

- Copper straight-through cables (switch-to-device)

- Copper crossover cables (if needed for device-to-device)

IP Addressing Scheme

Main Campus Networks

| Department/Faculty | VLAN ID | Network Address |
| --- | --- | --- |
| Administrative Staff | 10 | 192.168.1.0/24 |
| HR | 20 | 192.168.2.0/24 |
| Finance | 30 | 192.168.3.0/24 |
| Business | 40 | 192.168.4.0/24 |
| Engineering & Computing | 50 | 192.168.5.0/24 |
| Arts & Design | 60 | 192.168.6.0/24 |
| Student Labs | 70 | 192.168.7.0/24 |
| IT Department | 80 | 192.168.8.0/24 |

Table 3: Main Campus VLAN and IP Assignment

Branch Campus Networks

| Department | VLAN ID | Network Address |
| --- | --- | --- |
| Staff Department | 100 | 192.168.10.0/24 |
| Student Labs | 90 | 192.168.9.0/24 |

Table 4: Branch Campus VLAN and IP Assignment

WAN Link Addressing

| Link Description | Network Address |
| --- | --- |
| Main Campus Router to Branch Campus Router | 10.10.10.0/30 |
| Main Campus Router to Cloud Router | 10.10.10.4/30 |
| Cloud Router to Email Server | 20.0.0.0/30 |

Table 5: WAN Serial Link Addressing

Lab Procedures

Part 1: Network Topology Design and Prototyping

Step 1.1: Create Hierarchical Network Model

Core Layer:

- Place three 2901 ISR routers in the workspace

- Label routers as:

  - Main Campus Router

  - Branch Campus Router

  - Cloud Router

Distribution Layer:

- Place two 3650-24PS Layer 3 switches

- Label as:

  - Main Campus L3 Switch

  - Branch Campus L3 Switch

Access Layer:

- Place 2960-24TT Layer 2 switches for each department

- Main Campus: 8 switches (Admin, HR, Finance, Business, Eng/Comp, Arts/Design, Student Labs, IT)

- Branch Campus: 2 switches (Staff, Student Labs)

Step 1.2: Add End Devices

- Add at least 2 PCs per department

- Add 1 printer per department

- Add Web Server and FTP Server in IT Department

- Add Email Server connected to Cloud Router

Step 1.3: Physical Device Preparation

Install Serial Modules on Routers:

- Click on each router (Main Campus, Branch Campus, Cloud)

- Click the power button to turn OFF the router

- Go to Physical tab

- Drag HWIC-2T module to an empty slot

- Turn router back ON

- Close the physical window

Power Layer 3 Switches:

- Click on Layer 3 switch

- Go to Physical tab

- Drag AC Power Supply to power slot

- Observe switch booting up

Step 1.4: Cable Connections

WAN Connections (Serial DCE/DTE):

- Connect Main Campus Router Serial 0/1/1 to Branch Campus Router Serial 0/1/0 (Serial DCE cable)

- Connect Main Campus Router Serial 0/1/0 to Cloud Router Serial 0/2/0 (Serial DCE cable)

- Note which end shows the clock symbol (DCE interface)

Distribution to Core:

- Connect Main Campus L3 Switch Gig1/0/1 to Main Campus Router Gig0/0

- Connect Branch Campus L3 Switch Gig1/0/1 to Branch Campus Router Gig0/0

Access to Distribution:

- Connect each departmental switch to corresponding L3 switch port:

  - Admin Switch FastEthernet0/1 to L3 Switch Gig1/0/2

  - HR Switch FastEthernet0/1 to L3 Switch Gig1/0/3

  - Finance Switch FastEthernet0/1 to L3 Switch Gig1/0/4

  - (Continue for remaining departments)

End Devices to Access Switches:

- Connect PCs and printers to access layer switches using copper straight-through cables

- Connect servers to IT Department switch

Step 1.5: Topology Organization and Documentation

- Use notes/rectangles to visually separate:

  - Main Campus (Buildings A, B, C)

  - Branch Campus

  - Cloud section

- Add text labels for VLAN IDs and network addresses

- Use background colors to distinguish departments

- Save topology as "Albon_University_CAN.pkt"

Checkpoint 1: Have your instructor verify your topology design before proceeding.

Part 2: Basic Device Configuration

Step 2.1: Configure Router Interfaces

Main Campus Router Configuration:

Router> enable
Router# configure terminal
Router(config)# hostname Main-Campus-Router
Main-Campus-Router(config)# interface GigabitEthernet0/0
Main-Campus-Router(config-if)# no shutdown
Main-Campus-Router(config-if)# exit
Main-Campus-Router(config)# interface Serial0/1/0
Main-Campus-Router(config-if)# no shutdown
Main-Campus-Router(config-if)# exit
Main-Campus-Router(config)# interface Serial0/1/1
Main-Campus-Router(config-if)# no shutdown
Main-Campus-Router(config-if)# exit
Main-Campus-Router(config)# do write

Branch Campus Router Configuration:

Router> enable
Router# configure terminal
Router(config)# hostname Branch-Campus-Router
Branch-Campus-Router(config)# interface GigabitEthernet0/0
Branch-Campus-Router(config-if)# no shutdown
Branch-Campus-Router(config-if)# exit
Branch-Campus-Router(config)# interface Serial0/1/0
Branch-Campus-Router(config-if)# no shutdown
Branch-Campus-Router(config-if)# exit
Branch-Campus-Router(config)# do write

Cloud Router Configuration:

Router> enable
Router# configure terminal
Router(config)# hostname Cloud-Router
Cloud-Router(config)# interface GigabitEthernet0/0
Cloud-Router(config-if)# no shutdown
Cloud-Router(config-if)# exit
Cloud-Router(config)# interface Serial0/2/0
Cloud-Router(config-if)# no shutdown
Cloud-Router(config-if)# exit
Cloud-Router(config)# do write

Step 2.2: Configure Clock Rate on Serial DCE Interfaces

Identify DCE interfaces (hover over serial cable to see clock symbol on one end).

On Main Campus Router:

Main-Campus-Router(config)# interface Serial0/1/0
Main-Campus-Router(config-if)# clock rate 64000
Main-Campus-Router(config-if)# exit
Main-Campus-Router(config)# interface Serial0/1/1
Main-Campus-Router(config-if)# clock rate 64000
Main-Campus-Router(config-if)# exit
Main-Campus-Router(config)# do write

Note: Only configure clock rate on DCE side of serial connection.

Part 3: VLAN Configuration on Access Layer Switches

Step 3.1: Configure VLANs on Departmental Switches

General Command Template:

Switch> enable
Switch# configure terminal
Switch(config)# hostname [Department-Name]-Switch
Switch(config)# interface range FastEthernet0/1-24
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan [VLAN-ID]
Switch(config-if-range)# exit
Switch(config)# do write

Admin Switch (VLAN 10):

Switch> enable
Switch# configure terminal
Switch(config)# hostname Admin-Switch
Admin-Switch(config)# interface range FastEthernet0/1-24
Admin-Switch(config-if-range)# switchport mode access
Admin-Switch(config-if-range)# switchport access vlan 10
Admin-Switch(config-if-range)# exit
Admin-Switch(config)# do write

HR Switch (VLAN 20):

Switch> enable
Switch# configure terminal
Switch(config)# hostname HR-Switch
HR-Switch(config)# interface range FastEthernet0/1-24
HR-Switch(config-if-range)# switchport mode access
HR-Switch(config-if-range)# switchport access vlan 20
HR-Switch(config-if-range)# exit
HR-Switch(config)# do write

Repeat for remaining switches:

- Finance Switch: VLAN 30

- Business Switch: VLAN 40

- Engineering & Computing Switch: VLAN 50

- Arts & Design Switch: VLAN 60

- Student Labs Switch: VLAN 70

- IT Department Switch: VLAN 80

- Branch Staff Switch: VLAN 100

- Branch Student Labs Switch: VLAN 90

Checkpoint 2: Verify VLAN configuration using show vlan brief on each switch.

Part 4: Layer 3 Switch Configuration

Step 4.1: Configure Access Ports on Main Campus L3 Switch

Each departmental uplink must be configured as access port with corresponding VLAN.

Switch> enable
Switch# configure terminal
Switch(config)# hostname Main-L3-Switch
Main-L3-Switch(config)# interface GigabitEthernet1/0/2
Main-L3-Switch(config-if)# switchport mode access
Main-L3-Switch(config-if)# switchport access vlan 10
Main-L3-Switch(config-if)# exit

Main-L3-Switch(config)# interface GigabitEthernet1/0/3
Main-L3-Switch(config-if)# switchport mode access
Main-L3-Switch(config-if)# switchport access vlan 20
Main-L3-Switch(config-if)# exit

Main-L3-Switch(config)# interface GigabitEthernet1/0/4
Main-L3-Switch(config-if)# switchport mode access
Main-L3-Switch(config-if)# switchport access vlan 30
Main-L3-Switch(config-if)# exit

Main-L3-Switch(config)# interface GigabitEthernet1/0/5
Main-L3-Switch(config-if)# switchport mode access
Main-L3-Switch(config-if)# switchport access vlan 40
Main-L3-Switch(config-if)# exit

Main-L3-Switch(config)# interface GigabitEthernet1/0/6
Main-L3-Switch(config-if)# switchport mode access
Main-L3-Switch(config-if)# switchport access vlan 50
Main-L3-Switch(config-if)# exit

Main-L3-Switch(config)# interface GigabitEthernet1/0/7
Main-L3-Switch(config-if)# switchport mode access
Main-L3-Switch(config-if)# switchport access vlan 60
Main-L3-Switch(config-if)# exit

Main-L3-Switch(config)# interface GigabitEthernet1/0/8
Main-L3-Switch(config-if)# switchport mode access
Main-L3-Switch(config-if)# switchport access vlan 70
Main-L3-Switch(config-if)# exit

Main-L3-Switch(config)# interface GigabitEthernet1/0/9
Main-L3-Switch(config-if)# switchport mode access
Main-L3-Switch(config-if)# switchport access vlan 80
Main-L3-Switch(config-if)# exit
Main-L3-Switch(config)# do write

Step 4.2: Configure Trunk Port to Router

Main Campus L3 Switch:

Main-L3-Switch(config)# interface GigabitEthernet1/0/1
Main-L3-Switch(config-if)# switchport trunk encapsulation dot1q
Main-L3-Switch(config-if)# switchport mode trunk
Main-L3-Switch(config-if)# exit
Main-L3-Switch(config)# do write

Step 4.3: Configure Branch Campus L3 Switch

Access Ports:

Switch> enable
Switch# configure terminal
Switch(config)# hostname Branch-L3-Switch
Branch-L3-Switch(config)# interface GigabitEthernet1/0/2
Branch-L3-Switch(config-if)# switchport mode access
Branch-L3-Switch(config-if)# switchport access vlan 90
Branch-L3-Switch(config-if)# exit

Branch-L3-Switch(config)# interface GigabitEthernet1/0/3
Branch-L3-Switch(config-if)# switchport mode access
Branch-L3-Switch(config-if)# switchport access vlan 100
Branch-L3-Switch(config-if)# exit

Trunk Port:

Branch-L3-Switch(config)# interface GigabitEthernet1/0/1
Branch-L3-Switch(config-if)# switchport trunk encapsulation dot1q
Branch-L3-Switch(config-if)# switchport mode trunk
Branch-L3-Switch(config-if)# exit
Branch-L3-Switch(config)# do write

Checkpoint 3: Verify trunk configuration with show interfaces trunk command.

Part 5: Inter-VLAN Routing Configuration

Step 5.1: Configure Subinterfaces on Main Campus Router

Router-on-a-Stick Configuration:

Main-Campus-Router(config)# interface GigabitEthernet0/0.10
Main-Campus-Router(config-subif)# encapsulation dot1Q 10
Main-Campus-Router(config-subif)# ip address 192.168.1.1 255.255.255.0
Main-Campus-Router(config-subif)# exit

Main-Campus-Router(config)# interface GigabitEthernet0/0.20
Main-Campus-Router(config-subif)# encapsulation dot1Q 20
Main-Campus-Router(config-subif)# ip address 192.168.2.1 255.255.255.0
Main-Campus-Router(config-subif)# exit

Main-Campus-Router(config)# interface GigabitEthernet0/0.30
Main-Campus-Router(config-subif)# encapsulation dot1Q 30
Main-Campus-Router(config-subif)# ip address 192.168.3.1 255.255.255.0
Main-Campus-Router(config-subif)# exit

Main-Campus-Router(config)# interface GigabitEthernet0/0.40
Main-Campus-Router(config-subif)# encapsulation dot1Q 40
Main-Campus-Router(config-subif)# ip address 192.168.4.1 255.255.255.0
Main-Campus-Router(config-subif)# exit

Main-Campus-Router(config)# interface GigabitEthernet0/0.50
Main-Campus-Router(config-subif)# encapsulation dot1Q 50
Main-Campus-Router(config-subif)# ip address 192.168.5.1 255.255.255.0
Main-Campus-Router(config-subif)# exit

Main-Campus-Router(config)# interface GigabitEthernet0/0.60
Main-Campus-Router(config-subif)# encapsulation dot1Q 60
Main-Campus-Router(config-subif)# ip address 192.168.6.1 255.255.255.0
Main-Campus-Router(config-subif)# exit

Main-Campus-Router(config)# interface GigabitEthernet0/0.70
Main-Campus-Router(config-subif)# encapsulation dot1Q 70
Main-Campus-Router(config-subif)# ip address 192.168.7.1 255.255.255.0
Main-Campus-Router(config-subif)# exit

Main-Campus-Router(config)# interface GigabitEthernet0/0.80
Main-Campus-Router(config-subif)# encapsulation dot1Q 80
Main-Campus-Router(config-subif)# ip address 192.168.8.1 255.255.255.0
Main-Campus-Router(config-subif)# exit
Main-Campus-Router(config)# do write

Step 5.2: Configure Subinterfaces on Branch Campus Router

Branch-Campus-Router(config)# interface GigabitEthernet0/0.90
Branch-Campus-Router(config-subif)# encapsulation dot1Q 90
Branch-Campus-Router(config-subif)# ip address 192.168.9.1 255.255.255.0
Branch-Campus-Router(config-subif)# exit

Branch-Campus-Router(config)# interface GigabitEthernet0/0.100
Branch-Campus-Router(config-subif)# encapsulation dot1Q 100
Branch-Campus-Router(config-subif)# ip address 192.168.10.1 255.255.255.0
Branch-Campus-Router(config-subif)# exit
Branch-Campus-Router(config)# do write

Part 6: WAN Interface IP Addressing

Step 6.1: Configure Serial Interfaces on Main Campus Router

Main-Campus-Router(config)# interface Serial0/1/0
Main-Campus-Router(config-if)# ip address 10.10.10.5 255.255.255.252
Main-Campus-Router(config-if)# exit

Main-Campus-Router(config)# interface Serial0/1/1
Main-Campus-Router(config-if)# ip address 10.10.10.1 255.255.255.252
Main-Campus-Router(config-if)# exit
Main-Campus-Router(config)# do write

Step 6.2: Configure Serial Interface on Branch Campus Router

Branch-Campus-Router(config)# interface Serial0/1/0
Branch-Campus-Router(config-if)# ip address 10.10.10.2 255.255.255.252
Branch-Campus-Router(config-if)# exit
Branch-Campus-Router(config)# do write

Step 6.3: Configure Serial Interface on Cloud Router

Cloud-Router(config)# interface Serial0/2/0
Cloud-Router(config-if)# ip address 10.10.10.6 255.255.255.252
Cloud-Router(config-if)# exit

Cloud-Router(config)# interface GigabitEthernet0/0
Cloud-Router(config-if)# ip address 20.0.0.1 255.255.255.252
Cloud-Router(config-if)# exit
Cloud-Router(config)# do write

Step 6.4: Configure Email Server Static IP

- Click on Email Server

- Go to Desktop tab

- Select IP Configuration

- Configure:

  - IP Address: 20.0.0.2

  - Subnet Mask: 255.255.255.252

  - Default Gateway: 20.0.0.1

- Close configuration window

Checkpoint 4: Test connectivity - Ping from Cloud Router (20.0.0.1) to Email Server (20.0.0.2).

Part 7: DHCP Server Configuration

Step 7.1: Enable DHCP Service

Main Campus Router:

Main-Campus-Router(config)# service dhcp

Step 7.2: Create DHCP Pools for Each Department

Admin Department Pool (VLAN 10):

Main-Campus-Router(config)# ip dhcp pool Admin-Pool
Main-Campus-Router(dhcp-config)# network 192.168.1.0 255.255.255.0
Main-Campus-Router(dhcp-config)# default-router 192.168.1.1
Main-Campus-Router(dhcp-config)# dns-server 192.168.1.1
Main-Campus-Router(dhcp-config)# exit

HR Department Pool (VLAN 20):

Main-Campus-Router(config)# ip dhcp pool HR-Pool
Main-Campus-Router(dhcp-config)# network 192.168.2.0 255.255.255.0
Main-Campus-Router(dhcp-config)# default-router 192.168.2.1
Main-Campus-Router(dhcp-config)# dns-server 192.168.2.1
Main-Campus-Router(dhcp-config)# exit

Finance Department Pool (VLAN 30):

Main-Campus-Router(config)# ip dhcp pool Finance-Pool
Main-Campus-Router(dhcp-config)# network 192.168.3.0 255.255.255.0
Main-Campus-Router(dhcp-config)# default-router 192.168.3.1
Main-Campus-Router(dhcp-config)# dns-server 192.168.3.1
Main-Campus-Router(dhcp-config)# exit

Business Department Pool (VLAN 40):

Main-Campus-Router(config)# ip dhcp pool Business-Pool
Main-Campus-Router(dhcp-config)# network 192.168.4.0 255.255.255.0
Main-Campus-Router(dhcp-config)# default-router 192.168.4.1
Main-Campus-Router(dhcp-config)# dns-server 192.168.4.1
Main-Campus-Router(dhcp-config)# exit

Engineering & Computing Pool (VLAN 50):

Main-Campus-Router(config)# ip dhcp pool Engineering-Pool
Main-Campus-Router(dhcp-config)# network 192.168.5.0 255.255.255.0
Main-Campus-Router(dhcp-config)# default-router 192.168.5.1
Main-Campus-Router(dhcp-config)# dns-server 192.168.5.1
Main-Campus-Router(dhcp-config)# exit

Arts & Design Pool (VLAN 60):

Main-Campus-Router(config)# ip dhcp pool Arts-Pool
Main-Campus-Router(dhcp-config)# network 192.168.6.0 255.255.255.0
Main-Campus-Router(dhcp-config)# default-router 192.168.6.1
Main-Campus-Router(dhcp-config)# dns-server 192.168.6.1
Main-Campus-Router(dhcp-config)# exit

Student Labs Pool (VLAN 70):

Main-Campus-Router(config)# ip dhcp pool StudentLabs-Pool
Main-Campus-Router(dhcp-config)# network 192.168.7.0 255.255.255.0
Main-Campus-Router(dhcp-config)# default-router 192.168.7.1
Main-Campus-Router(dhcp-config)# dns-server 192.168.7.1
Main-Campus-Router(dhcp-config)# exit

IT Department Pool (VLAN 80):

Main-Campus-Router(config)# ip dhcp pool IT-Pool
Main-Campus-Router(dhcp-config)# network 192.168.8.0 255.255.255.0
Main-Campus-Router(dhcp-config)# default-router 192.168.8.1
Main-Campus-Router(dhcp-config)# dns-server 192.168.8.1
Main-Campus-Router(dhcp-config)# exit
Main-Campus-Router(config)# do write

Step 7.3: Configure DHCP on Branch Campus Router

Staff Pool (VLAN 100):

Branch-Campus-Router(config)# service dhcp
Branch-Campus-Router(config)# ip dhcp pool Staff-Pool
Branch-Campus-Router(dhcp-config)# network 192.168.10.0 255.255.255.0
Branch-Campus-Router(dhcp-config)# default-router 192.168.10.1
Branch-Campus-Router(dhcp-config)# dns-server 192.168.10.1
Branch-Campus-Router(dhcp-config)# exit

Student Labs Pool (VLAN 90):

Branch-Campus-Router(config)# ip dhcp pool BranchStudentLabs-Pool
Branch-Campus-Router(dhcp-config)# network 192.168.9.0 255.255.255.0
Branch-Campus-Router(dhcp-config)# default-router 192.168.9.1
Branch-Campus-Router(dhcp-config)# dns-server 192.168.9.1
Branch-Campus-Router(dhcp-config)# exit
Branch-Campus-Router(config)# do write

Step 7.4: Test DHCP Assignment

- Click on a PC in Admin department

- Go to Desktop > IP Configuration

- Select DHCP option

- Verify PC receives IP address in 192.168.1.0/24 range

- Note default gateway should be 192.168.1.1

- Repeat for PCs in other departments

Checkpoint 5: Verify all PCs receive appropriate DHCP addresses from their respective pools.

Part 8: Dynamic Routing Configuration (RIP v2)

Step 8.1: Configure RIP v2 on Main Campus Router

Main-Campus-Router(config)# router rip
Main-Campus-Router(config-router)# version 2
Main-Campus-Router(config-router)# network 192.168.1.0
Main-Campus-Router(config-router)# network 192.168.2.0
Main-Campus-Router(config-router)# network 192.168.3.0
Main-Campus-Router(config-router)# network 192.168.4.0
Main-Campus-Router(config-router)# network 192.168.5.0
Main-Campus-Router(config-router)# network 192.168.6.0
Main-Campus-Router(config-router)# network 192.168.7.0
Main-Campus-Router(config-router)# network 192.168.8.0
Main-Campus-Router(config-router)# network 10.10.10.0
Main-Campus-Router(config-router)# network 10.10.10.4
Main-Campus-Router(config-router)# no auto-summary
Main-Campus-Router(config-router)# exit
Main-Campus-Router(config)# do write

Step 8.2: Configure RIP v2 on Branch Campus Router

Branch-Campus-Router(config)# router rip
Branch-Campus-Router(config-router)# version 2
Branch-Campus-Router(config-router)# network 192.168.9.0
Branch-Campus-Router(config-router)# network 192.168.10.0
Branch-Campus-Router(config-router)# network 10.10.10.0
Branch-Campus-Router(config-router)# no auto-summary
Branch-Campus-Router(config-router)# exit
Branch-Campus-Router(config)# do write

Step 8.3: Configure Static Routing on Cloud Router

Cloud-Router(config)# ip route 192.168.0.0 255.255.0.0 10.10.10.5
Cloud-Router(config)# ip route 10.10.10.0 255.255.255.252 10.10.10.5
Cloud-Router(config)# do write

Step 8.4: Configure Static Route on Main Campus Router for Cloud

Main-Campus-Router(config)# ip route 20.0.0.0 255.255.255.252 10.10.10.6
Main-Campus-Router(config)# do write

Checkpoint 6: Verify routing tables using show ip route on all routers.

Part 9: Server Configuration

Step 9.1: Configure Web Server in IT Department

- Click on Web Server

- Desktop > IP Configuration

- Configure static IP:

  - IP Address: 192.168.8.10

  - Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.8.1

- Go to Services > HTTP

- Ensure HTTP service is ON

- Edit index.html with custom content: "Welcome to Albon University Web Server"

Step 9.2: Configure FTP Server in IT Department

- Click on FTP Server

- Desktop > IP Configuration

- Configure static IP:

  - IP Address: 192.168.8.11

  - Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.8.1

- Go to Services > FTP

- Ensure FTP service is ON

- Create user account: username "student", password "cisco123"

Step 9.3: Configure Email Server (Cloud)

- Email Server already configured with IP: 20.0.0.2

- Go to Services > EMAIL

- Ensure email service is ON

- Set domain name: albon.edu

- Create email accounts for testing:

  - admin@albon.edu

  - student@albon.edu

Part 10: Verification and Testing

Step 10.1: Test Local Connectivity

Within Same VLAN:

- From Admin PC1, ping Admin PC2

- Command: ping 192.168.1.[other-PC-IP]

- Expected result: Successful ping

Step 10.2: Test Inter-VLAN Routing

Between Different VLANs:

- From Admin PC (VLAN 10), ping HR PC (VLAN 20)

- Command: ping 192.168.2.[HR-PC-IP]

- Expected result: Successful ping

- Repeat between multiple VLAN pairs

Step 10.3: Test WAN Connectivity

Main Campus to Branch Campus:

- From Admin PC, ping Branch Staff PC

- Command: ping 192.168.10.[Staff-PC-IP]

- Expected result: Successful ping

Step 10.4: Test Server Access

Web Server Access:

- From any PC, open Web Browser

- Navigate to: http://192.168.8.10

- Expected result: Display "Welcome to Albon University Web Server"

FTP Server Access:

- From any PC, use Command Prompt or PC configuration

- Command: ftp 192.168.8.11

- Login with username: student, password: cisco123

- Expected result: Successful FTP connection

Email Server Access:

- From any PC, open Email client

- Configure email with server: 20.0.0.2

- Send test email from admin@albon.edu to student@albon.edu

- Expected result: Email successfully sent and received

Step 10.5: Verify DHCP Assignments

Main-Campus-Router# show ip dhcp binding

Expected output: List of all IP addresses assigned to clients with MAC addresses.

Step 10.6: Verify Routing Protocol

Main-Campus-Router# show ip route
Branch-Campus-Router# show ip route

Expected output: RIP routes (marked with "R") showing learned networks.

Step 10.7: Verify VLAN Configuration

Admin-Switch# show vlan brief
Main-L3-Switch# show vlan brief

Expected output: VLAN assignments matching configuration plan.

Step 10.8: Verify Trunk Links

Main-L3-Switch# show interfaces trunk

Expected output: Gig1/0/1 showing as trunk with VLANs 10-80 allowed.

Troubleshooting Scenarios

Scenario 1: PCs in the Admin department cannot obtain IP addresses automatically.

Questions:

- What commands would you use to troubleshoot this issue?

- How can you verify if the DHCP pool is configured correctly?

- What interface configuration might be missing?

Scenario 2: Admin PC can ping HR PC, but cannot ping Branch Campus Staff PC.

Questions:

- What layer of the OSI model is the issue likely occurring?

- What routing configuration would you check?

- How would you verify if the WAN link is operational?

Scenario 3: The trunk link between Main-L3-Switch and Main-Campus-Router shows as down.

Questions:

- What commands would you use to verify trunk status?

- What encapsulation must match on both ends of the trunk?

- How can you verify which VLANs are allowed on the trunk?