Enterprise Network Lab Manual

Building a Complete Realistic Network Using Switch, Router, Server, and Firewall

Table of Contents

- Lab Objectives

- Lab Overview and Network Design

- Required Equipment and Software

- Network Topology and Addressing Scheme

- Part 1: Initial Setup and Device Placement

- Part 2: Catalyst 3560 Switch Configuration

- Part 3: Cisco 1911 Router Configuration

- Part 4: Firewall Configuration

- Part 5: Server Configuration

- Part 6: End Device Configuration

- Part 7: Verification and Testing

- Part 8: Troubleshooting Common Issues

- Deliverables and Assessment

Lab Objectives

Upon completion of this lab, students will be able to:

- Design and implement a multi-VLAN enterprise network topology

- Configure VLANs and 802.1Q trunking on a Catalyst switch

- Implement router-on-a-stick for inter-VLAN routing

- Configure a firewall with NAT and access control lists

- Set up DHCP services with IP helper addresses

- Verify network connectivity across all network segments

- Troubleshoot common networking issues in enterprise environments

- Understand the security boundary between internal and external networks

Lab Overview and Network Design

This lab simulates a production-grade small-to-medium enterprise network with proper network segmentation, routing, security, and services. The design follows industry best practices by separating users, servers, and management traffic into distinct VLANs while implementing security at the network edge.

Network Design Philosophy

Classic Enterprise Edge Model:

- Layer 2 Switch - Provides VLAN segmentation and access layer connectivity

- Router-on-a-Stick - Performs inter-VLAN routing using subinterfaces

- Firewall - Creates security boundary between internal network and Internet

- Server - Provides network services (DHCP, DNS, etc.)

This four-device model mirrors how many real-world enterprises design their networks, keeping routing, switching, and security functions separate and manageable.

Required Equipment and Software

Cisco Packet Tracer Devices

| Device Type | Model | Quantity |
| --- | --- | --- |
| Router | Cisco 1911 | 1 |
| Switch | Catalyst 3560-24PS | 1 |
| Firewall | ASA 5506-X (or generic firewall) | 1 |
| Server | Server-PT | 1 |
| PC | PC-PT | 2-3 |
| Cloud | Cloud-PT (for Internet simulation) | 1 |

Table 1: Required Packet Tracer devices

Software Requirements

- Cisco Packet Tracer 8.0 or higher

- Basic understanding of CLI command-line interface

Network Topology and Addressing Scheme

Logical Network Diagram

Internet (ISP)
                     |
              [Public IP]
                     |
                FIREWALL
        Outside: Public WAN IP
        Inside: 10.0.0.1/30
                     |
               10.0.0.2/30
                     |
              CISCO 1911 ROUTER
        G0/0: Trunk (802.1Q)
        G0/1: 10.0.0.2/30
                     |
              [802.1Q Trunk]
                     |
            CATALYST 3560 SWITCH
                     |
      +--------------+--------------+
      |              |              |
   VLAN 10        VLAN 20       VLAN 30
   Users          Servers       Management

IP Addressing Scheme

| VLAN | Name | Network | Gateway |
| --- | --- | --- | --- |
| VLAN 10 | USERS | 192.168.10.0/24 | 192.168.10.1 |
| VLAN 20 | SERVERS | 192.168.20.0/24 | 192.168.20.1 |
| VLAN 30 | MGMT | 192.168.30.0/24 | 192.168.30.1 |

Table 2: VLAN addressing scheme

| Interface | IP Address/Mask |
| --- | --- |
| Router G0/0 | Physical interface (no IP) |
| Router G0/0.10 | 192.168.10.1/24 |
| Router G0/0.20 | 192.168.20.1/24 |
| Router G0/0.30 | 192.168.30.1/24 |
| Router G0/1 | 10.0.0.2/30 |
| Switch VLAN 30 SVI | 192.168.30.2/24 |
| Firewall Inside | 10.0.0.1/30 |
| Firewall Outside | Public IP (simulated) |
| Server (VLAN 20) | 192.168.20.10/24 |
| Example PC (VLAN 10) | 192.168.10.50/24 |

Table 3: Device interface addressing

Connection Table

| Device A | Interface | Device B Interface |
| --- | --- | --- |
| Router G0/0 | ↔ | Switch Gi0/1 (Trunk) |
| Router G0/1 | ↔ | Firewall Inside |
| Firewall Outside | ↔ | Cloud/ISP |
| Switch Gi0/2 | ↔ | PC1 (VLAN 10) |
| Switch Gi0/3 | ↔ | Server (VLAN 20) |
| Switch Gi0/4 | ↔ | PC2 (VLAN 10) |

Table 4: Physical connections

Part 1: Initial Setup and Device Placement

Step 1.1: Open Cisco Packet Tracer

- Launch Cisco Packet Tracer

- Create a new blank project

- Save the project as "Enterprise_Network_Lab.pkt"

Step 1.2: Place Devices on Workspace

- From the Network Devices section:

  - Select Routers → Add 1941 router (or 1911 if available)

  - Select Switches → Add 3560-24PS switch

  - Select Security → Add ASA 5506-X firewall

- From the End Devices section:

  - Add 1 Server-PT

  - Add 2-3 PC-PT devices

  - Add 1 Cloud-PT (for Internet simulation)

- Arrange devices logically: Cloud at top, Firewall below, Router in center, Switch below router, end devices at bottom

Step 1.3: Label Devices

Rename devices for clarity:

- Router: "R1-Edge"

- Switch: "SW1-Access"

- Firewall: "FW1-ASA"

- Server: "Server1"

- PCs: "PC-User1", "PC-User2"

- Cloud: "Internet-ISP"

To rename: Click device → Config tab → Display Name field

Step 1.4: Cable the Network

Use Copper Straight-Through cables for all connections:

- Cloud to Firewall Outside: Cloud Ethernet port to Firewall GigabitEthernet0/0

- Firewall Inside to Router: Firewall GigabitEthernet0/1 to Router GigabitEthernet0/1

- Router to Switch (Trunk): Router GigabitEthernet0/0 to Switch GigabitEthernet0/1

- Switch to Server: Switch GigabitEthernet0/3 to Server FastEthernet0

- Switch to PC-User1: Switch GigabitEthernet0/2 to PC-User1 FastEthernet0

- Switch to PC-User2: Switch GigabitEthernet0/4 to PC-User2 FastEthernet0

Note: Wait for interface indicators to turn green (up/up). Some may remain orange/red until configuration is complete.

Part 2: Catalyst 3560 Switch Configuration

Step 2.1: Access Switch CLI

- Click on SW1-Access

- Select CLI tab

- Press Enter to start

Step 2.2: Enter Privileged EXEC Mode

Switch> enable
Switch#

Step 2.3: Enter Global Configuration Mode

Switch# configure terminal
Switch(config)#

Step 2.4: Create VLANs

Create three VLANs for Users, Servers, and Management:

Switch(config)# vlan 10
Switch(config-vlan)# name USERS
Switch(config-vlan)# exit

Switch(config)# vlan 20
Switch(config-vlan)# name SERVERS
Switch(config-vlan)# exit

Switch(config)# vlan 30
Switch(config-vlan)# name MGMT
Switch(config-vlan)# exit

Verification:

Switch(config)# do show vlan brief

You should see VLANs 10, 20, and 30 listed with their names.

Step 2.5: Configure Trunk Port to Router

Configure GigabitEthernet0/1 as an 802.1Q trunk:

Switch(config)# interface GigabitEthernet0/1
Switch(config-if)# description TRUNK-TO-ROUTER-R1
Switch(config-if)# switchport trunk encapsulation dot1q
Switch(config-if)# switchport mode trunk
Switch(config-if)# switchport trunk allowed vlan 10,20,30
Switch(config-if)# no shutdown
Switch(config-if)# exit

Note: The switchport trunk encapsulation dot1q command may not be required on some Packet Tracer switch models. If you receive an error, skip that command.

Step 2.6: Configure Access Ports for End Devices

GigabitEthernet0/2 - PC-User1 (VLAN 10):

Switch(config)# interface GigabitEthernet0/2
Switch(config-if)# description USER-PC-VLAN10
Switch(config-if)# switchport mode access
Switch(config-if)# switchport access vlan 10
Switch(config-if)# no shutdown
Switch(config-if)# exit

GigabitEthernet0/3 - Server (VLAN 20):

Switch(config)# interface GigabitEthernet0/3
Switch(config-if)# description SERVER-VLAN20
Switch(config-if)# switchport mode access
Switch(config-if)# switchport access vlan 20
Switch(config-if)# no shutdown
Switch(config-if)# exit

GigabitEthernet0/4 - PC-User2 (VLAN 10):

Switch(config)# interface GigabitEthernet0/4
Switch(config-if)# description USER-PC2-VLAN10
Switch(config-if)# switchport mode access
Switch(config-if)# switchport access vlan 10
Switch(config-if)# no shutdown
Switch(config-if)# exit

Step 2.7: Configure Switch Management Interface (SVI)

Create a management interface in VLAN 30:

Switch(config)# interface vlan 30
Switch(config-if)# description SWITCH-MANAGEMENT
Switch(config-if)# ip address 192.168.30.2 255.255.255.0
Switch(config-if)# no shutdown
Switch(config-if)# exit

Step 2.8: Configure Default Gateway for Switch

Switch(config)# ip default-gateway 192.168.30.1

Step 2.9: Save Configuration

Switch(config)# end
Switch# write memory

Or use the shorter version:

Switch# wr

Step 2.10: Verify Switch Configuration

Run these verification commands:

Switch# show vlan brief
Switch# show interfaces trunk
Switch# show ip interface brief
Switch# show running-config

Expected Results:

- VLANs 10, 20, 30 should be active

- Gi0/1 should show as trunk with allowed VLANs 10,20,30

- Gi0/2 and Gi0/4 should be in VLAN 10

- Gi0/3 should be in VLAN 20

- VLAN 30 interface should have IP 192.168.30.2

Part 3: Cisco 1911 Router Configuration

The router will perform router-on-a-stick inter-VLAN routing using subinterfaces.

Step 3.1: Access Router CLI

- Click on R1-Edge

- Select CLI tab

- Press Enter to start

Step 3.2: Enter Privileged EXEC and Configuration Mode

Router> enable
Router# configure terminal
Router(config)#

Step 3.3: Set Hostname

Router(config)# hostname R1
R1(config)#

Step 3.4: Configure Physical Interface Toward Switch

GigabitEthernet0/0 will be the trunk port - do NOT assign an IP address:

R1(config)# interface GigabitEthernet0/0
R1(config-if)# description TRUNK-TO-SWITCH-SW1
R1(config-if)# no shutdown
R1(config-if)# exit

Important: The physical interface must be "no shutdown" for subinterfaces to work.

Step 3.5: Configure Subinterface for VLAN 10 (Users)

R1(config)# interface GigabitEthernet0/0.10
R1(config-subif)# description VLAN10-USERS
R1(config-subif)# encapsulation dot1Q 10
R1(config-subif)# ip address 192.168.10.1 255.255.255.0
R1(config-subif)# exit

Step 3.6: Configure Subinterface for VLAN 20 (Servers)

R1(config)# interface GigabitEthernet0/0.20
R1(config-subif)# description VLAN20-SERVERS
R1(config-subif)# encapsulation dot1Q 20
R1(config-subif)# ip address 192.168.20.1 255.255.255.0
R1(config-subif)# exit

Step 3.7: Configure Subinterface for VLAN 30 (Management)

R1(config)# interface GigabitEthernet0/0.30
R1(config-subif)# description VLAN30-MGMT
R1(config-subif)# encapsulation dot1Q 30
R1(config-subif)# ip address 192.168.30.1 255.255.255.0
R1(config-subif)# exit

Step 3.8: Configure Interface Toward Firewall

R1(config)# interface GigabitEthernet0/1
R1(config-if)# description TO-FIREWALL-INSIDE
R1(config-if)# ip address 10.0.0.2 255.255.255.252
R1(config-if)# no shutdown
R1(config-if)# exit

Step 3.9: Configure Default Route to Firewall

R1(config)# ip route 0.0.0.0 0.0.0.0 10.0.0.1

This sends all Internet-bound traffic to the firewall.

Step 3.10: Configure DHCP Relay (IP Helper-Address)

If you want the server to provide DHCP to all VLANs, configure IP helper addresses:

R1(config)# interface GigabitEthernet0/0.10
R1(config-subif)# ip helper-address 192.168.20.10
R1(config-subif)# exit

R1(config)# interface GigabitEthernet0/0.30
R1(config-subif)# ip helper-address 192.168.20.10
R1(config-subif)# exit

Note: VLAN 20 server interface doesn't need helper-address since the server is in that VLAN.

Step 3.11: Save Configuration

R1(config)# end
R1# write memory

Step 3.12: Verify Router Configuration

R1# show ip interface brief
R1# show ip route
R1# show running-config

Expected Results:

- G0/0 should be up/up with no IP address

- G0/0.10, G0/0.20, G0/0.30 should be up/up with respective IPs

- G0/1 should be up/up with IP 10.0.0.2

- Routing table should show connected networks for all VLANs

- Default route 0.0.0.0/0 pointing to 10.0.0.1

Part 4: Firewall Configuration

Note: Cisco Packet Tracer firewall configuration varies by version. This section provides ASA 5506-X configuration. Adapt commands as needed for your Packet Tracer version.

Step 4.1: Access Firewall CLI

- Click on FW1-ASA

- Select CLI tab

- Press Enter to start

Step 4.2: Enter Privileged EXEC and Configuration Mode

ciscoasa> enable
ciscoasa# configure terminal
ciscoasa(config)#

Step 4.3: Set Hostname

ciscoasa(config)# hostname FW1
FW1(config)#

Step 4.4: Configure Inside Interface

FW1(config)# interface GigabitEthernet0/1
FW1(config-if)# nameif inside
FW1(config-if)# security-level 100
FW1(config-if)# ip address 10.0.0.1 255.255.255.252
FW1(config-if)# no shutdown
FW1(config-if)# exit

Step 4.5: Configure Outside Interface

FW1(config)# interface GigabitEthernet0/0
FW1(config-if)# nameif outside
FW1(config-if)# security-level 0
FW1(config-if)# ip address dhcp
FW1(config-if)# no shutdown
FW1(config-if)# exit

Note: For simulation, you can use ip address dhcp or assign a static public IP like 209.165.200.225 255.255.255.224 if your Cloud is configured with that network.

Step 4.6: Configure Default Route

FW1(config)# route outside 0.0.0.0 0.0.0.0 dhcp

Or if using static IP:

FW1(config)# route outside 0.0.0.0 0.0.0.0 209.165.200.226

Step 4.7: Configure NAT (Network Address Translation)

Configure dynamic PAT (Port Address Translation) for all inside networks:

FW1(config)# object network INSIDE-NET
FW1(config-network-object)# subnet 10.0.0.0 255.255.255.0
FW1(config-network-object)# nat (inside,outside) dynamic interface
FW1(config-network-object)# exit

FW1(config)# object network VLAN10-NET
FW1(config-network-object)# subnet 192.168.10.0 255.255.255.0
FW1(config-network-object)# nat (inside,outside) dynamic interface
FW1(config-network-object)# exit

FW1(config)# object network VLAN20-NET
FW1(config-network-object)# subnet 192.168.20.0 255.255.255.0
FW1(config-network-object)# nat (inside,outside) dynamic interface
FW1(config-network-object)# exit

FW1(config)# object network VLAN30-NET
FW1(config-network-object)# subnet 192.168.30.0 255.255.255.0
FW1(config-network-object)# nat (inside,outside) dynamic interface
FW1(config-network-object)# exit

Step 4.8: Configure Access Control Lists (ACLs)

Allow inside networks to access outside:

FW1(config)# access-list OUTSIDE-IN extended permit icmp any any
FW1(config)# access-list OUTSIDE-IN extended permit ip any any
FW1(config)# access-group OUTSIDE-IN in interface outside

Note: These are permissive rules for lab purposes. Production environments require more restrictive ACLs.

Step 4.9: Save Configuration

FW1(config)# end
FW1# write memory

Step 4.10: Verify Firewall Configuration

FW1# show interface ip brief
FW1# show route
FW1# show nat
FW1# show running-config

Part 5: Server Configuration

Step 5.1: Configure Server IP Addressing

- Click on Server1

- Select Desktop tab

- Click IP Configuration

- Select Static IP configuration

- Enter the following:

  - IP Address: 192.168.20.10

  - Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.20.1

  - DNS Server: 192.168.20.10 (points to itself)

- Close the IP Configuration window

Step 5.2: Configure DHCP Service

- On Server1, go to Services tab

- Select DHCP from the left menu

- Click On to enable DHCP service

Create DHCP Pool for VLAN 10 (Users):

- Pool Name: VLAN10-POOL

- Default Gateway: 192.168.10.1

- DNS Server: 192.168.20.10

- Start IP Address: 192.168.10.100

- Subnet Mask: 255.255.255.0

- Maximum Number of Users: 50

- Click Add

Create DHCP Pool for VLAN 30 (Management):

- Pool Name: VLAN30-POOL

- Default Gateway: 192.168.30.1

- DNS Server: 192.168.20.10

- Start IP Address: 192.168.30.100

- Subnet Mask: 255.255.255.0

- Maximum Number of Users: 20

- Click Add

Note: VLAN 20 doesn't need DHCP since it's for servers with static IPs.

Step 5.3: Configure DNS Service (Optional)

- On Server1, go to Services tab

- Select DNS from the left menu

- Click On to enable DNS service

- Add sample DNS records:

  - Name: server1.local, Address: 192.168.20.10

  - Name: router.local, Address: 192.168.10.1

- Click Add for each record

Part 6: End Device Configuration

Step 6.1: Configure PC-User1 (Static IP Example)

- Click on PC-User1

- Select Desktop tab

- Click IP Configuration

- Select Static IP configuration

- Enter the following:

  - IP Address: 192.168.10.50

  - Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.10.1

  - DNS Server: 192.168.20.10

Step 6.2: Configure PC-User2 (DHCP Example)

- Click on PC-User2

- Select Desktop tab

- Click IP Configuration

- Select DHCP

- Wait for DHCP to assign an address (should receive 192.168.10.100 or next available)

- Verify that IP address, subnet mask, gateway, and DNS are populated

Troubleshooting: If DHCP doesn't work:

- Verify router has ip helper-address 192.168.20.10 on G0/0.10

- Verify DHCP service is enabled on Server1

- Verify PC is in VLAN 10 on the switch

Part 7: Verification and Testing

Step 7.1: Verify Switch Status

From SW1-Access CLI:

Switch# show vlan brief
Switch# show interfaces trunk
Switch# show interfaces status

Expected Results:

- VLANs 10, 20, 30 are active

- Gi0/1 is trunking with VLANs 10, 20, 30 allowed

- Gi0/2 and Gi0/4 are in VLAN 10, status connected

- Gi0/3 is in VLAN 20, status connected

Step 7.2: Verify Router Status

From R1 CLI:

R1# show ip interface brief
R1# show ip route
R1# show interfaces trunk

Expected Results:

- All subinterfaces (G0/0.10, G0/0.20, G0/0.30) are up/up

- G0/1 is up/up with IP 10.0.0.2

- Routing table shows:

  - Connected routes for 192.168.10.0/24, 192.168.20.0/24, 192.168.30.0/24

  - Connected route for 10.0.0.0/30

  - Default route 0.0.0.0/0 via 10.0.0.1

Step 7.3: Test Connectivity from Router

From R1 CLI:

R1# ping 192.168.10.1
R1# ping 192.168.20.1
R1# ping 192.168.30.1
R1# ping 192.168.30.2
R1# ping 192.168.20.10
R1# ping 10.0.0.1

Expected Results: All pings should succeed.

Step 7.4: Test Connectivity from PC-User1

From PC-User1 Desktop, open Command Prompt:

C:> ipconfig
C:> ping 192.168.10.1
C:> ping 192.168.20.10
C:> ping 192.168.30.2
C:> ping 10.0.0.1

Expected Results:

- ipconfig shows IP 192.168.10.50, gateway 192.168.10.1

- Ping to gateway (192.168.10.1) succeeds - tests local VLAN

- Ping to server (192.168.20.10) succeeds - tests inter-VLAN routing

- Ping to switch (192.168.30.2) succeeds - tests VLAN 30 connectivity

- Ping to firewall (10.0.0.1) may succeed - tests router-to-firewall link

Step 7.5: Test Inter-VLAN Routing

From PC-User1, ping a device in different VLAN:

C:> ping 192.168.20.10

From Server1 Desktop, open Command Prompt:

C:> ping 192.168.10.50

Expected Results: Both pings should succeed, demonstrating inter-VLAN routing.

Step 7.6: Test DHCP Functionality

From PC-User2 (configured for DHCP):

C:> ipconfig /release
C:> ipconfig /renew
C:> ipconfig

Expected Results:

- IP address in range 192.168.10.100-150

- Subnet mask 255.255.255.0

- Default gateway 192.168.10.1

- DNS server 192.168.20.10

Step 7.7: Test Internet Connectivity (Simulated)

Note: This depends on your Cloud configuration. If Cloud is configured as ISP:

From PC-User1 Command Prompt:

C:> ping 8.8.8.8
C:> tracert 8.8.8.8

If successful, packet traverses: PC → Switch → Router → Firewall (NAT) → Internet

Step 7.8: Verify NAT on Firewall

From FW1 CLI:

FW1# show xlate
FW1# show nat

This displays active NAT translations.

Step 7.9: Test Security Segmentation

From PC-User1, attempt to ping the firewall's inside interface:

C:> ping 10.0.0.1

This may succeed or fail depending on firewall ACL configuration. Document the behavior.

Part 8: Troubleshooting Common Issues

Issue 1: PCs Cannot Ping Gateway

Symptoms: PC cannot ping its default gateway (192.168.10.1)

Possible Causes:

- PC is not in correct VLAN

- Switch port not configured as access port

- Router subinterface not configured or down

- Physical interface G0/0 on router is shutdown

Resolution Steps:

- Verify PC is in correct VLAN:
Switch# show vlan brief

- Verify switch port configuration:
Switch# show running-config interface GigabitEthernet0/2

- Verify router subinterface status:
R1# show ip interface brief

- Ensure physical interface is up:
R1# show interfaces GigabitEthernet0/0
R1# configure terminal
R1(config)# interface GigabitEthernet0/0
R1(config-if)# no shutdown

Issue 2: Inter-VLAN Routing Not Working

Symptoms: PC can ping its gateway but cannot reach devices in other VLANs

Possible Causes:

- 802.1Q encapsulation not configured on subinterfaces

- VLAN numbers mismatch between switch and router

- Trunk port not allowing all VLANs

Resolution Steps:

- Verify trunk configuration:
Switch# show interfaces trunk

- Verify subinterface encapsulation:
R1# show running-config interface GigabitEthernet0/0.10
Should show: encapsulation dot1Q 10

- Verify routing table:
R1# show ip route
Should show connected routes for all VLAN subnets

Issue 3: DHCP Not Working

Symptoms: PC configured for DHCP doesn't receive IP address

Possible Causes:

- DHCP service not enabled on server

- IP helper-address not configured on router

- DHCP pool not configured correctly

- PC not in correct VLAN

Resolution Steps:

- Verify DHCP service enabled on Server1

- Verify IP helper-address on router:
R1# show running-config interface GigabitEthernet0/0.10
Should show: ip helper-address 192.168.20.10

- Verify DHCP pools on Server1 (Services → DHCP)

- Test with static IP first to verify connectivity

Issue 4: No Internet Connectivity

Symptoms: PC can reach internal networks but not Internet

Possible Causes:

- Default route not configured on router

- Firewall NAT not configured

- Firewall outside interface not configured

- Cloud/ISP not configured properly

Resolution Steps:

- Verify default route on router:
R1# show ip route
Should show: S* 0.0.0.0/0 [1/0] via 10.0.0.1

- Verify router can reach firewall:
R1# ping 10.0.0.1

- Verify NAT configuration on firewall:
FW1# show nat
FW1# show xlate

- Test connectivity from router to outside:
R1# ping 8.8.8.8

Issue 5: Trunk Not Forming

Symptoms: Switch-to-router trunk not operational

Possible Causes:

- Trunk mode not configured on both ends

- Native VLAN mismatch

- Allowed VLANs restricted incorrectly

Resolution Steps:

- Verify trunk status:
Switch# show interfaces trunk

- Verify switchport mode:
Switch# show interfaces GigabitEthernet0/1 switchport
Should show: Administrative Mode: trunk

- Reset interface if needed:
Switch(config)# interface GigabitEthernet0/1
Switch(config-if)# shutdown
Switch(config-if)# no shutdown

Issue 6: Switch Cannot Be Managed Remotely

Symptoms: Cannot ping switch management IP (192.168.30.2)

Possible Causes:

- VLAN 30 interface not configured

- Default gateway not configured on switch

- VLAN 30 subinterface not configured on router

Resolution Steps:

- Verify VLAN 30 interface on switch:
Switch# show ip interface brief
Should show VLAN 30 with IP 192.168.30.2

- Verify default gateway:
Switch# show running-config | include default-gateway
Should show: ip default-gateway 192.168.30.1

- Test from switch itself:
Switch# ping 192.168.30.1
Switch# ping 192.168.20.10

Deliverables and Assessment

Required Deliverables

Students must submit the following:

- Packet Tracer File (.pkt)

  - Complete working network topology

  - All devices properly configured

  - File named: LastName_FirstName_Enterprise_Network.pkt

- Configuration Documentation

  - Copy of running-config from each device (Switch, Router, Firewall)

  - Save as text file or Word document

- Verification Screenshots

  - Switch: show vlan brief, show interfaces trunk

  - Router: show ip interface brief, show ip route

  - Firewall: show interface ip brief, show nat

  - PC: ipconfig, successful ping tests

- Lab Report (2-3 pages)

  - Network diagram with all IP addresses labeled

  - Description of network design and VLAN segmentation

  - Explanation of router-on-a-stick operation

  - Discussion of firewall role and NAT operation

  - Troubleshooting challenges encountered and resolutions

  - Reflection on enterprise network design principles

Assessment Rubric

| Component | Points | Criteria |
| --- | --- | --- |
| Switch Configuration | 20 | VLANs created, trunk configured, |
|  |  | access ports assigned correctly |
| Router Configuration | 25 | Subinterfaces configured, |
|  |  | inter-VLAN routing works, |
|  |  | default route configured |
| Firewall Configuration | 15 | Inside/outside interfaces configured, |
|  |  | NAT operational, basic ACLs present |
| Server Configuration | 10 | DHCP configured and functional, |
|  |  | proper IP addressing |
| Connectivity Tests | 15 | All verification tests pass, |
|  |  | screenshots provided |
| Documentation | 10 | Complete configs saved, |
|  |  | clear network diagram included |
| Lab Report | 5 | Professional writeup, |
|  |  | demonstrates understanding |
| Total | 100 |  |

Table 5: Lab assessment rubric

Bonus Challenges (Optional - Extra Credit)

For students who complete the lab early or want additional challenges:

- ACL Implementation: Configure extended ACLs to restrict VLAN 10 users from accessing VLAN 30 management network

- Port Security: Implement port security on switch access ports with MAC address limits

- SSH Access: Configure SSH on router and switch for secure remote management

- HSRP: Add a second router and implement Hot Standby Router Protocol for gateway redundancy

- DMZ: Create VLAN 40 as DMZ for public-facing servers with specific firewall rules

- QoS: Implement Quality of Service to prioritize voice/video traffic

- SNMP: Configure SNMP for network monitoring

- Syslog: Configure devices to send logs to syslog server

Appendix A: Quick Reference Commands

Switch Commands

! VLAN Creation
vlan 10
name USERS

! Trunk Configuration
interface GigabitEthernet0/1
switchport trunk encapsulation dot1q
switchport mode trunk
switchport trunk allowed vlan 10,20,30

! Access Port Configuration
interface GigabitEthernet0/2
switchport mode access
switchport access vlan 10

! Management Interface
interface vlan 30
ip address 192.168.30.2 255.255.255.0
ip default-gateway 192.168.30.1

! Verification
show vlan brief
show interfaces trunk
show ip interface brief

Router Commands

! Physical Interface
interface GigabitEthernet0/0
no shutdown

! Subinterface Configuration
interface GigabitEthernet0/0.10
encapsulation dot1Q 10
ip address 192.168.10.1 255.255.255.0

! Default Route
ip route 0.0.0.0 0.0.0.0 10.0.0.1

! DHCP Relay
interface GigabitEthernet0/0.10
ip helper-address 192.168.20.10

! Verification
show ip interface brief
show ip route
show interfaces trunk
ping 192.168.10.50

Firewall Commands

! Interface Configuration
interface GigabitEthernet0/1
nameif inside
security-level 100
ip address 10.0.0.1 255.255.255.252

! NAT Configuration
object network VLAN10-NET
subnet 192.168.10.0 255.255.255.0
nat (inside,outside) dynamic interface

! Default Route
route outside 0.0.0.0 0.0.0.0 dhcp

! Verification
show interface ip brief
show nat
show xlate
show route

Appendix B: Network Design Best Practices

VLAN Design Principles

- Separate by Function: Users, servers, and management should be in different VLANs

- Security Boundaries: VLANs create logical security zones

- Broadcast Domain Control: Smaller broadcast domains improve performance

- Scalability: Plan VLAN scheme to accommodate growth

IP Addressing Best Practices

- Consistent Scheme: Use logical, predictable addressing (e.g., VLAN 10 = 192.168.10.0/24)

- Reserve Addresses: First/last IPs for network equipment, middle ranges for DHCP

- Documentation: Maintain accurate IP address management spreadsheet

- Summarization: Use contiguous subnets for route summarization when possible

Security Best Practices

- Defense in Depth: Multiple layers of security (firewall, ACLs, port security)

- Principle of Least Privilege: Only allow necessary traffic

- Management VLAN: Separate management traffic from user/server traffic

- Disable Unused Ports: Shutdown unused switch ports, assign to dummy VLAN

- Strong Passwords: Use complex passwords on all devices (enable secret)

Troubleshooting Methodology

- Define the Problem: What is not working? What should be working?

- Gather Information: Use show commands, ping, traceroute

- Analyze Information: Compare expected vs actual state

- Eliminate Possibilities: Work through OSI layers (physical → datalink → network)

- Implement Solution: Make one change at a time

- Test Results: Verify problem is resolved

- Document: Record problem and solution for future reference

Conclusion

This lab provides hands-on experience with a realistic enterprise network design incorporating VLANs, routing, security, and network services. The skills developed here—VLAN configuration, router-on-a-stick implementation, firewall security, and systematic troubleshooting—are fundamental to real-world network engineering.

By completing this lab, students gain practical knowledge of how modern enterprise networks are architected with proper segmentation, security boundaries, and service integration. These principles scale from small office networks to large enterprise environments.

Remember: Enterprise networking is about understanding not just how to configure devices, but why we architect networks this way—separating concerns, implementing defense in depth, and designing for scalability and manageability.

End of Lab Manual

Instructor: Piyali Ganguly
Last Updated: March 2026