Hotel Network Design Lab: Multi-Floor Network Implementation Using Cisco Packet Tracer

Learning Objectives

By the end of this lab, you will be able to:

- Design and implement a multi-floor hierarchical network for an enterprise environment

- Configure serial DCE connections between routers with proper clock rates

- Implement VLAN segmentation across multiple departments and floors

- Configure router-on-a-stick (inter-VLAN routing) using subinterfaces

- Set up DHCP servers on routers for dynamic IP address assignment

- Configure OSPF routing protocol for multi-router connectivity

- Implement wireless networking with access points for laptops and mobile devices

- Configure SSH for secure remote access to network devices

- Apply port security with MAC address sticky learning and violation policies

- Verify end-to-end connectivity across VLANs, floors, and departments

- Troubleshoot common network configuration issues

Building Structure

Vic Modern Hotel is a three-floor building with the following department layout:

| Floor | Departments |
| --- | --- |
| First Floor | Reception, Store, Logistics |
| Second Floor | Finance, Human Resources (HR), Sales & Marketing |
| Third Floor | IT, Administration |

Table 1: Hotel Department Distribution by Floor

Network Requirements

- Three routers - One router per floor, interconnected via serial DCE cables

- Serial networks - Routers connected using the following networks:

  - Floor 1 ↔ Floor 2: 10.10.10.4/30

  - Floor 2 ↔ Floor 3: 10.10.10.8/30

  - Floor 3 ↔ Floor 1: 10.10.10.0/30

- One switch per floor - Placed at respective floors for department connectivity

- Wi-Fi networks - Each floor has wireless access point for laptops and mobile devices

- One printer per department - Total of 8 printers

- VLAN segmentation - Each department in separate VLAN with specific network addressing

- OSPF routing protocol - For advertising routes between routers

- DHCP service - All devices obtain IP addresses dynamically from respective floor routers

- Full connectivity - All devices must communicate across VLANs and floors

- SSH remote access - Configured on all routers for secure management

- Port security - IT department switch port with MAC sticky and shutdown violation mode

VLAN and IP Addressing Scheme

Complete VLAN Table

| Floor | Department | VLAN | Network | Default Gateway |
| --- | --- | --- | --- | --- |
| First Floor | Reception | 80 | 192.168.8.0/24 | 192.168.8.1 |
| First Floor | Store | 70 | 192.168.7.0/24 | 192.168.7.1 |
| First Floor | Logistics | 60 | 192.168.6.0/24 | 192.168.6.1 |
| Second Floor | Finance | 50 | 192.168.5.0/24 | 192.168.5.1 |
| Second Floor | Human Resources (HR) | 40 | 192.168.4.0/24 | 192.168.4.1 |
| Second Floor | Sales & Marketing | 30 | 192.168.3.0/24 | 192.168.3.1 |
| Third Floor | IT | 10 | 192.168.1.0/24 | 192.168.1.1 |
| Third Floor | Administration | 20 | 192.168.2.0/24 | 192.168.2.1 |

Table 2: VLAN and IP Addressing Scheme for All Departments

Note: All department networks use /24 subnet mask (255.255.255.0), providing 254 usable host addresses per VLAN.

Required Equipment

Cisco Packet Tracer Devices

| Device Type | Quantity | Model/Notes |
| --- | --- | --- |
| Routers | 3 | Cisco 2911 |
| HWIC-2T Serial Modules | 3 | 2-port high-speed serial interface cards |
| Layer 2 Switches | 3 | Cisco 2960 (one per floor) |
| PCs | 8+ | Generic PC (one per department minimum) |
| Printers | 8 | Generic Printer (one per department) |
| Wireless Access Points | 3 | AccessPoint-PT (one per floor) |
| Laptops | 3+ | Generic Laptop with WPC300N wireless module |
| Tablets | 1+ | Generic Tablet |
| Smartphones | 1+ | Generic Smartphone |

Table 3: Network Equipment List

Part 1: Topology Design and Physical Setup

Step 1: Create New Packet Tracer Project

- Open Cisco Packet Tracer

- Create new project: File → New

- Save immediately: File → Save As → Hotel_Network_Project3.pkt

- Optional: Set background color via Options → Preferences → Interface

Step 2: Place Core Network Devices (Routers)

- From device panel, select Routers → 2911

- Place 3 routers on workspace arranged vertically or in triangle formation

- Leave adequate spacing between routers for connections

- Name routers for identification (will configure display names later):

  - Top/Left router: Floor 1 Router (F1-Router)

  - Middle router: Floor 2 Router (F2-Router)

  - Bottom/Right router: Floor 3 Router (F3-Router)

Step 3: Install Serial Interface Modules on All Routers

Critical: Cisco 2911 routers do NOT have serial interfaces by default. You must add HWIC-2T modules.

For EACH of the three routers, perform the following:

- Click on the router

- Click Physical tab

- Locate the power button (shows 1 or 0 with green/white indicator)

- Click power button to turn OFF the router (indicator turns white, wait for shutdown)

- In the Modules panel (left side), locate HWIC-2T module

- Drag and drop HWIC-2T into an empty HWIC slot (right side slots)

- Click power button to turn ON the router (indicator turns green)

- Close the physical view

Step 4: Connect Routers with Serial DCE Cables

- Click Connections icon (lightning bolt)

- Select Serial DCE cable (solid black line with clock icon)

- Create the following connections:

  - F1-Router Serial0/0/0 ↔ F2-Router Serial0/0/0

  - F2-Router Serial0/0/1 ↔ F3-Router Serial0/1/0

  - F3-Router Serial0/1/1 ↔ F1-Router Serial0/0/1

- All links will initially show orange (down), then turn green after configuration

Important: Take note of which router interfaces are DCE ends (hover over cable connection to see clock symbol). You will configure clock rate on DCE interfaces only.

Step 5: Place Switches (One Per Floor)

- Select Switches → 2960

- Place 3 switches below their respective routers

- Arrange vertically to represent floors:

  - Floor1-Switch (below F1-Router)

  - Floor2-Switch (below F2-Router)

  - Floor3-Switch (below F3-Router)

Step 6: Connect Switches to Routers

- Select Copper Straight-Through cable (or use Auto connection)

- Connect each switch to its respective router:

  - Floor1-Switch FastEthernet0/1 → F1-Router GigabitEthernet0/0

  - Floor2-Switch FastEthernet0/1 → F2-Router GigabitEthernet0/0

  - Floor3-Switch FastEthernet0/1 → F3-Router GigabitEthernet0/0

Note: Interface FastEthernet0/1 on each switch will be configured as a trunk port.

Part 2: Add End Devices by Department

Step 7: First Floor Devices (3 Departments)

For Reception Department:

- Place 1 PC and 1 Printer below Floor1-Switch

- Connect PC to Floor1-Switch FastEthernet0/2 using copper straight-through cable

- Connect Printer to Floor1-Switch FastEthernet0/3

- Label devices: Reception-PC, Reception-Printer

For Store Department:

- Place 1 PC and 1 Printer

- Connect PC to FastEthernet0/4, Printer to FastEthernet0/5

- Label: Store-PC, Store-Printer

For Logistics Department:

- Place 1 PC and 1 Printer

- Connect PC to FastEthernet0/6, Printer to FastEthernet0/7

- Label: Logistics-PC, Logistics-Printer

Step 8: Second Floor Devices (3 Departments)

For Finance Department:

- Place 1 PC and 1 Printer below Floor2-Switch

- Connect PC to Floor2-Switch FastEthernet0/2, Printer to FastEthernet0/3

- Label: Finance-PC, Finance-Printer

For HR Department:

- Place 1 PC and 1 Printer

- Connect PC to FastEthernet0/4, Printer to FastEthernet0/5

- Label: HR-PC, HR-Printer

For Sales Department:

- Place 1 PC and 1 Printer

- Connect PC to FastEthernet0/6, Printer to FastEthernet0/7

- Label: Sales-PC, Sales-Printer

Step 9: Third Floor Devices (2 Departments)

For IT Department:

- Place 1 PC (this will be the Test-PC for SSH and port security) and 1 Printer

- Connect Test-PC to Floor3-Switch FastEthernet0/2 (IMPORTANT: Note this port)

- Connect Printer to FastEthernet0/3

- Label: IT-Test-PC, IT-Printer

For Administration Department:

- Place 1 PC and 1 Printer

- Connect PC to FastEthernet0/4, Printer to FastEthernet0/5

- Label: Admin-PC, Admin-Printer

Step 10: Add Wireless Access Points (One Per Floor)

- Select Wireless Devices → AccessPoint-PT

- Place 1 access point near each floor switch

- Connect to switches:

  - Floor1-AP → Floor1-Switch FastEthernet0/8

  - Floor2-AP → Floor2-Switch FastEthernet0/8

  - Floor3-AP → Floor3-Switch FastEthernet0/6

- Label: Floor1-AP, Floor2-AP, Floor3-AP

Step 11: Add Wireless Devices

- Add at least 1 laptop per floor (3 total minimum)

- Add 1 tablet and 1 smartphone (optional, for testing)

- For laptops: Click laptop → Physical tab → Power OFF → Remove Ethernet module → Insert WPC300N wireless module → Power ON

- Tablets and smartphones have built-in wireless capability

- Place wireless devices near their respective floor access points

Part 3: Visual Organization with Boundaries

Step 12: Create Floor Boundaries

- Click Select tool, then click Rectangle shape tool

- Draw rectangles around each floor's devices

- Right-click rectangle → To Back (moves behind devices)

- Use different fill colors for each floor:

  - First Floor: Light blue

  - Second Floor: Light green

  - Third Floor: Light yellow

Step 13: Create Department Boundaries

- Use Rectangle tool with white or light-colored fill

- Draw smaller rectangles around each department's devices

- Move To Back

- This creates visual separation of departments within floors

Step 14: Add Text Labels

- Click Text (ABC) tool

- Add floor labels:

  - "FIRST FLOOR" (above Floor 1 section)

  - "SECOND FLOOR" (above Floor 2 section)

  - "THIRD FLOOR" (above Floor 3 section)

- Add department labels with VLAN information:

  - "Reception - VLAN 80 - 192.168.8.0/24"

  - "Store - VLAN 70 - 192.168.7.0/24"

  - "Logistics - VLAN 60 - 192.168.6.0/24"

  - "Finance - VLAN 50 - 192.168.5.0/24"

  - "HR - VLAN 40 - 192.168.4.0/24"

  - "Sales - VLAN 30 - 192.168.3.0/24"

  - "IT - VLAN 10 - 192.168.1.0/24"

  - "Admin - VLAN 20 - 192.168.2.0/24"

Part 4: Basic Router Configuration

Step 15: Configure Serial Interfaces and Clock Rates

For F1-Router (Floor 1 Router):

- Click F1-Router → CLI tab

- Enter the following commands:

Router> enable
Router# configure terminal
Router(config)# interface serial0/0/0
Router(config-if)# no shutdown
Router(config-if)# exit
Router(config)# interface serial0/0/1
Router(config-if)# no shutdown
Router(config-if)# exit
Router(config)# interface gigabitEthernet0/0
Router(config-if)# no shutdown
Router(config-if)# exit

Identify DCE interfaces (hover over connections to see clock symbol), then configure clock rate:

Router(config)# interface serial0/0/0
Router(config-if)# clock rate 64000
Router(config-if)# exit
Router(config)# interface serial0/0/1
Router(config-if)# clock rate 64000
Router(config-if)# exit
Router(config)# do write

For F2-Router (Floor 2 Router):

Router> enable
Router# configure terminal
Router(config)# interface serial0/0/0
Router(config-if)# no shutdown
Router(config-if)# exit
Router(config)# interface serial0/0/1
Router(config-if)# no shutdown
Router(config-if)# exit
Router(config)# interface gigabitEthernet0/0
Router(config-if)# no shutdown
Router(config-if)# exit
Router(config)# do write

Note: Check if F2-Router has DCE interfaces. If yes, add clock rate commands.

For F3-Router (Floor 3 Router):

Router> enable
Router# configure terminal
Router(config)# interface serial0/1/0
Router(config-if)# no shutdown
Router(config-if)# exit
Router(config)# interface serial0/1/1
Router(config-if)# no shutdown
Router(config-if)# clock rate 64000
Router(config-if)# exit
Router(config)# interface gigabitEthernet0/0
Router(config-if)# no shutdown
Router(config-if)# exit
Router(config)# do write

Step 16: Assign IP Addresses to Serial Interfaces

Serial Network Addressing:

- F1 ↔ F2: 10.10.10.4/30 (usable IPs: .5 and .6)

- F2 ↔ F3: 10.10.10.8/30 (usable IPs: .9 and .10)

- F3 ↔ F1: 10.10.10.0/30 (usable IPs: .1 and .2)

F1-Router:

Router(config)# interface serial0/0/0
Router(config-if)# ip address 10.10.10.5 255.255.255.252
Router(config-if)# exit
Router(config)# interface serial0/0/1
Router(config-if)# ip address 10.10.10.2 255.255.255.252
Router(config-if)# exit
Router(config)# do write

F2-Router:

Router(config)# interface serial0/0/0
Router(config-if)# ip address 10.10.10.6 255.255.255.252
Router(config-if)# exit
Router(config)# interface serial0/0/1
Router(config-if)# ip address 10.10.10.9 255.255.255.252
Router(config-if)# exit
Router(config)# do write

F3-Router:

Router(config)# interface serial0/1/0
Router(config-if)# ip address 10.10.10.10 255.255.255.252
Router(config-if)# exit
Router(config)# interface serial0/1/1
Router(config-if)# ip address 10.10.10.1 255.255.255.252
Router(config-if)# exit
Router(config)# do write

Verification: All serial links should now show green (up/up state).

Part 5: VLAN Configuration on Switches

Step 17: Configure VLANs on Floor 1 Switch

Floor 1 has 3 VLANs: 80 (Reception), 70 (Store), 60 (Logistics)

Click Floor1-Switch → CLI:

Switch> enable
Switch# configure terminal
Switch(config)# interface range fastEthernet0/2-3
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 80
Switch(config-if-range)# exit
Switch(config)# interface range fastEthernet0/4-5
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 70
Switch(config-if-range)# exit
Switch(config)# interface range fastEthernet0/6-7
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 60
Switch(config-if-range)# exit
Switch(config)# interface fastEthernet0/8
Switch(config-if)# switchport mode access
Switch(config-if)# switchport access vlan 80
Switch(config-if)# exit
Switch(config)# interface fastEthernet0/1
Switch(config-if)# switchport mode trunk
Switch(config-if)# exit
Switch(config)# do write

Step 18: Configure VLANs on Floor 2 Switch

Floor 2 has 3 VLANs: 50 (Finance), 40 (HR), 30 (Sales)

Switch> enable
Switch# configure terminal
Switch(config)# interface range fastEthernet0/2-3
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 50
Switch(config-if-range)# exit
Switch(config)# interface range fastEthernet0/4-5
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 40
Switch(config-if-range)# exit
Switch(config)# interface range fastEthernet0/6-7
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 30
Switch(config-if-range)# exit
Switch(config)# interface fastEthernet0/8
Switch(config-if)# switchport mode access
Switch(config-if)# switchport access vlan 30
Switch(config-if)# exit
Switch(config)# interface fastEthernet0/1
Switch(config-if)# switchport mode trunk
Switch(config-if)# exit
Switch(config)# do write

Step 19: Configure VLANs on Floor 3 Switch

Floor 3 has 2 VLANs: 10 (IT), 20 (Admin)

Switch> enable
Switch# configure terminal
Switch(config)# interface range fastEthernet0/2-3
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 10
Switch(config-if-range)# exit
Switch(config)# interface range fastEthernet0/4-5
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 20
Switch(config-if-range)# exit
Switch(config)# interface fastEthernet0/6
Switch(config-if)# switchport mode access
Switch(config-if)# switchport access vlan 10
Switch(config-if)# exit
Switch(config)# interface fastEthernet0/1
Switch(config-if)# switchport mode trunk
Switch(config-if)# exit
Switch(config)# do write

Part 6: Inter-VLAN Routing (Router-on-a-Stick)

Step 20: Configure Subinterfaces on F1-Router

Create subinterfaces for VLANs 80, 70, and 60:

Router(config)# interface gigabitEthernet0/0.80
Router(config-subif)# encapsulation dot1Q 80
Router(config-subif)# ip address 192.168.8.1 255.255.255.0
Router(config-subif)# exit
Router(config)# interface gigabitEthernet0/0.70
Router(config-subif)# encapsulation dot1Q 70
Router(config-subif)# ip address 192.168.7.1 255.255.255.0
Router(config-subif)# exit
Router(config)# interface gigabitEthernet0/0.60
Router(config-subif)# encapsulation dot1Q 60
Router(config-subif)# ip address 192.168.6.1 255.255.255.0
Router(config-subif)# exit
Router(config)# do write

Step 21: Configure Subinterfaces on F2-Router

Create subinterfaces for VLANs 50, 40, and 30:

Router(config)# interface gigabitEthernet0/0.50
Router(config-subif)# encapsulation dot1Q 50
Router(config-subif)# ip address 192.168.5.1 255.255.255.0
Router(config-subif)# exit
Router(config)# interface gigabitEthernet0/0.40
Router(config-subif)# encapsulation dot1Q 40
Router(config-subif)# ip address 192.168.4.1 255.255.255.0
Router(config-subif)# exit
Router(config)# interface gigabitEthernet0/0.30
Router(config-subif)# encapsulation dot1Q 30
Router(config-subif)# ip address 192.168.3.1 255.255.255.0
Router(config-subif)# exit
Router(config)# do write

Step 22: Configure Subinterfaces on F3-Router

Create subinterfaces for VLANs 10 and 20:

Router(config)# interface gigabitEthernet0/0.10
Router(config-subif)# encapsulation dot1Q 10
Router(config-subif)# ip address 192.168.1.1 255.255.255.0
Router(config-subif)# exit
Router(config)# interface gigabitEthernet0/0.20
Router(config-subif)# encapsulation dot1Q 20
Router(config-subif)# ip address 192.168.2.1 255.255.255.0
Router(config-subif)# exit
Router(config)# do write

Part 7: DHCP Server Configuration

Step 23: Configure DHCP on F1-Router

Create DHCP pools for Reception (VLAN 80), Store (VLAN 70), and Logistics (VLAN 60):

Router(config)# service dhcp
Router(config)# ip dhcp pool Reception
Router(dhcp-config)# network 192.168.8.0 255.255.255.0
Router(dhcp-config)# default-router 192.168.8.1
Router(dhcp-config)# dns-server 192.168.8.1
Router(dhcp-config)# exit
Router(config)# ip dhcp pool Store
Router(dhcp-config)# network 192.168.7.0 255.255.255.0
Router(dhcp-config)# default-router 192.168.7.1
Router(dhcp-config)# dns-server 192.168.7.1
Router(dhcp-config)# exit
Router(config)# ip dhcp pool Logistics
Router(dhcp-config)# network 192.168.6.0 255.255.255.0
Router(dhcp-config)# default-router 192.168.6.1
Router(dhcp-config)# dns-server 192.168.6.1
Router(dhcp-config)# exit
Router(config)# do write

Step 24: Configure DHCP on F2-Router

Create DHCP pools for Finance (VLAN 50), HR (VLAN 40), and Sales (VLAN 30):

Router(config)# service dhcp
Router(config)# ip dhcp pool Finance
Router(dhcp-config)# network 192.168.5.0 255.255.255.0
Router(dhcp-config)# default-router 192.168.5.1
Router(dhcp-config)# dns-server 192.168.5.1
Router(dhcp-config)# exit
Router(config)# ip dhcp pool HR
Router(dhcp-config)# network 192.168.4.0 255.255.255.0
Router(dhcp-config)# default-router 192.168.4.1
Router(dhcp-config)# dns-server 192.168.4.1
Router(dhcp-config)# exit
Router(config)# ip dhcp pool Sales
Router(dhcp-config)# network 192.168.3.0 255.255.255.0
Router(dhcp-config)# default-router 192.168.3.1
Router(dhcp-config)# dns-server 192.168.3.1
Router(dhcp-config)# exit
Router(config)# do write

Step 25: Configure DHCP on F3-Router

Create DHCP pools for IT (VLAN 10) and Admin (VLAN 20):

Router(config)# service dhcp
Router(config)# ip dhcp pool IT
Router(dhcp-config)# network 192.168.1.0 255.255.255.0
Router(dhcp-config)# default-router 192.168.1.1
Router(dhcp-config)# dns-server 192.168.1.1
Router(dhcp-config)# exit
Router(config)# ip dhcp pool Admin
Router(dhcp-config)# network 192.168.2.0 255.255.255.0
Router(dhcp-config)# default-router 192.168.2.1
Router(dhcp-config)# dns-server 192.168.2.1
Router(dhcp-config)# exit
Router(config)# do write

Step 26: Test DHCP Address Assignment

- Click on Reception-PC → Desktop tab → IP Configuration

- Select DHCP button

- Wait for IP address assignment (should receive 192.168.8.x address)

- Verify: IP Address, Subnet Mask, Default Gateway, DNS Server are all populated

- Repeat for at least one PC per department to verify all DHCP pools work

Part 8: OSPF Routing Protocol Configuration

Step 27: Configure OSPF on F1-Router

Advertise all connected networks:

Router(config)# router ospf 10
Router(config-router)# network 10.10.10.4 0.0.0.3 area 0
Router(config-router)# network 10.10.10.0 0.0.0.3 area 0
Router(config-router)# network 192.168.8.0 0.0.0.255 area 0
Router(config-router)# network 192.168.7.0 0.0.0.255 area 0
Router(config-router)# network 192.168.6.0 0.0.0.255 area 0
Router(config-router)# exit
Router(config)# do write

Step 28: Configure OSPF on F2-Router

Router(config)# router ospf 10
Router(config-router)# network 10.10.10.4 0.0.0.3 area 0
Router(config-router)# network 10.10.10.8 0.0.0.3 area 0
Router(config-router)# network 192.168.5.0 0.0.0.255 area 0
Router(config-router)# network 192.168.4.0 0.0.0.255 area 0
Router(config-router)# network 192.168.3.0 0.0.0.255 area 0
Router(config-router)# exit
Router(config)# do write

Step 29: Configure OSPF on F3-Router

Router(config)# router ospf 10
Router(config-router)# network 10.10.10.8 0.0.0.3 area 0
Router(config-router)# network 10.10.10.0 0.0.0.3 area 0
Router(config-router)# network 192.168.1.0 0.0.0.255 area 0
Router(config-router)# network 192.168.2.0 0.0.0.255 area 0
Router(config-router)# exit
Router(config)# do write

Step 30: Verify OSPF Neighbor Adjacencies

On any router, verify OSPF is working:

Router# show ip ospf neighbor

You should see two neighbors per router with state FULL.

Step 31: Test Inter-VLAN and Inter-Floor Connectivity

- From Reception-PC (VLAN 80, Floor 1), open Command Prompt

- Test connectivity to IT-Test-PC (VLAN 10, Floor 3):
ping 192.168.1.2

- Should receive successful replies (!!!)

- Test connectivity to Sales-PC (VLAN 30, Floor 2):
ping 192.168.3.2

- Test connectivity to printer in different VLAN:
ping 192.168.5.3

Part 9: Wireless Network Configuration

Step 32: Configure Access Points

For Floor1-AP:

- Click Floor1-AP → Config tab → Port 1

- Set SSID: FL1

- Set Authentication: WPA2-PSK

- Set PSK Pass Phrase: FL1@123

- Click Save Settings

For Floor2-AP:

- SSID: FL2

- PSK Pass Phrase: FL2@123

For Floor3-AP:

- SSID: FL3

- PSK Pass Phrase: FL3@123

Step 33: Connect Wireless Devices

For each laptop:

- Click laptop → Desktop tab → PC Wireless

- Click Connect tab

- Select appropriate SSID (FL1, FL2, or FL3 based on floor)

- Enter password (e.g., FL1@123)

- Click Connect

- Status should show "Successfully Connected"

- Go to IP Configuration → Select DHCP

- Verify laptop receives IP address from appropriate VLAN

Note: Assign wireless devices to specific VLANs by connecting the AP to the correct access port on the switch.

Part 10: SSH Configuration for Secure Remote Access

Step 34: Configure SSH on F1-Router

Router(config)# hostname F1-Router
F1-Router(config)# ip domain-name hotel.local
F1-Router(config)# username admin privilege 15 secret admin123
F1-Router(config)# crypto key generate rsa
How many bits in the modulus [512]: 1024
F1-Router(config)# ip ssh version 2
F1-Router(config)# line vty 0 15
F1-Router(config-line)# login local
F1-Router(config-line)# transport input ssh
F1-Router(config-line)# exit
F1-Router(config)# do write

Step 35: Configure SSH on F2-Router

Router(config)# hostname F2-Router
F2-Router(config)# ip domain-name hotel.local
F2-Router(config)# username admin privilege 15 secret admin123
F2-Router(config)# crypto key generate rsa
How many bits in the modulus [512]: 1024
F2-Router(config)# ip ssh version 2
F2-Router(config)# line vty 0 15
F2-Router(config-line)# login local
F2-Router(config-line)# transport input ssh
F2-Router(config-line)# exit
F2-Router(config)# do write

Step 36: Configure SSH on F3-Router

Router(config)# hostname F3-Router
F3-Router(config)# ip domain-name hotel.local
F3-Router(config)# username admin privilege 15 secret admin123
F3-Router(config)# crypto key generate rsa
How many bits in the modulus [512]: 1024
F3-Router(config)# ip ssh version 2
F3-Router(config)# line vty 0 15
F3-Router(config-line)# login local
F3-Router(config-line)# transport input ssh
F3-Router(config-line)# exit
F3-Router(config)# do write

Step 37: Test SSH Remote Access from IT-Test-PC

- Click IT-Test-PC → Desktop → Command Prompt

- Test SSH connection to F1-Router:
ssh -l admin 10.10.10.2

- Enter password: admin123

- You should successfully log into F1-Router CLI

- Type exit to disconnect

- Test SSH to F2-Router (10.10.10.6) and F3-Router (10.10.10.1)

Part 11: Port Security Configuration

Step 38: Configure Port Security on IT Department Switch Port

On Floor3-Switch, configure FastEthernet0/2 (where IT-Test-PC is connected):

Switch(config)# interface fastEthernet0/2
Switch(config-if)# switchport mode access
Switch(config-if)# switchport port-security
Switch(config-if)# switchport port-security maximum 1
Switch(config-if)# switchport port-security mac-address sticky
Switch(config-if)# switchport port-security violation shutdown
Switch(config-if)# exit
Switch(config)# do write

Step 39: Verify Port Security Configuration

Switch# show port-security
Switch# show port-security interface fastEthernet0/2
Switch# show port-security address

Expected output should show:

- Port Security: Enabled

- Maximum MAC Addresses: 1

- Security Violation Mode: Shutdown

- MAC Address: Sticky (learned from IT-Test-PC)

Step 40: Test Port Security Violation

- Disconnect IT-Test-PC from FastEthernet0/2

- Connect a different PC to FastEthernet0/2

- Port should go into err-disabled state (red link)

- Verify with: show interface fastEthernet0/2 status

- Interface status will show: err-disabled

To recover from err-disabled state:

Switch(config)# interface fastEthernet0/2
Switch(config-if)# shutdown
Switch(config-if)# no shutdown
Switch(config-if)# exit

Part 12: Final Verification and Testing

Step 41: Complete Connectivity Matrix

Test connectivity between selected devices from different VLANs and floors. Record results:

| Source Device | Source IP | Destination IP | Result (Success/Fail) |
| --- | --- | --- | --- |
| Reception-PC | 192.168.8.2 | 192.168.1.2 (IT-Test-PC) | ________ |
| Store-PC | 192.168.7.2 | 192.168.5.2 (Finance-PC) | ________ |
| Logistics-PC | 192.168.6.2 | 192.168.3.3 (Sales-Printer) | ________ |
| Finance-PC | 192.168.5.2 | 192.168.2.2 (Admin-PC) | ________ |
| HR-PC | 192.168.4.2 | 192.168.6.2 (Logistics-PC) | ________ |
| Sales-PC | 192.168.3.2 | 192.168.8.3 (Reception-Printer) | ________ |
| IT-Test-PC | 192.168.1.2 | 192.168.7.2 (Store-PC) | ________ |
| Admin-PC | 192.168.2.2 | 192.168.4.2 (HR-PC) | ________ |

Table 4: Inter-VLAN and Inter-Floor Connectivity Test Matrix

Step 42: Verify Routing Tables

On F1-Router, F2-Router, and F3-Router:

Router# show ip route

Verify that:

- All 8 VLAN networks (192.168.1.0 - 192.168.8.0) appear as connected or OSPF routes

- All 3 serial networks (10.10.10.0/30, 10.10.10.4/30, 10.10.10.8/30) appear as connected

- OSPF routes marked with "O" for routes learned from other routers

Step 43: Verify DHCP Bindings

On each router:

Router# show ip dhcp binding

Should show list of IP addresses assigned to devices with their MAC addresses and lease expiration times.

Step 44: Document Network Configuration

Complete the following documentation table:

| Configuration Item | F1-Router | F2-Router | F3-Router |
| --- | --- | --- | --- |
| Number of VLANs | ____ | ____ | ____ |
| Number of DHCP Pools | ____ | ____ | ____ |
| Number of Subinterfaces | ____ | ____ | ____ |
| Number of OSPF Networks Advertised | ____ | ____ | ____ |
| SSH Enabled (Yes/No) | ____ | ____ | ____ |

Table 5: Router Configuration Summary

Part 13: Troubleshooting and Documentation

Step 45: Common Troubleshooting Commands

Document the output of the following commands from each router:

Interface Status:
Router# show ip interface brief

OSPF Neighbors:
Router# show ip ospf neighbor

OSPF Database:
Router# show ip ospf database

Routing Table:
Router# show ip route

DHCP Statistics:
Router# show ip dhcp pool
Router# show ip dhcp binding
Router# show ip dhcp conflict

SSH Configuration:
Router# show ip ssh
Router# show ssh

For switches:

Switch# show vlan brief
Switch# show interfaces trunk
Switch# show port-security
Switch# show mac address-table

Troubleshooting Guide

Problem: Serial links show red/orange and won't come up

Possible Causes:

- Serial modules not installed

- Router not powered on after module installation

- Clock rate not configured on DCE end

- Interface in shutdown state

- Incorrect IP addressing

Solution:

- Verify HWIC-2T modules installed on all routers

- Check interface status: show ip interface brief

- Enable interfaces: no shutdown

- Configure clock rate on DCE ends: clock rate 64000

- Verify IP addresses match network scheme

Problem: Devices not receiving DHCP addresses

Possible Causes:

- DHCP service not enabled

- DHCP pool not configured correctly

- Incorrect default-router or network statement

- Interface not in correct VLAN

- Trunk port not configured properly

Solution:

- Verify DHCP service enabled: service dhcp

- Check DHCP pools: show ip dhcp pool

- Verify network statements match VLAN subnets

- Check switch VLAN assignments: show vlan brief

- Verify trunk on router-facing port: show interfaces trunk

Problem: Devices cannot ping across VLANs

Possible Causes:

- Inter-VLAN routing not configured

- Subinterfaces missing or misconfigured

- VLAN encapsulation incorrect

- Default gateway not set on devices

- OSPF not configured or neighbors not forming

Solution:

- Verify subinterfaces created with correct VLAN tags

- Check encapsulation: show running-config

- Verify default gateway matches subinterface IP

- Check OSPF neighbors: show ip ospf neighbor

- Verify routing table: show ip route

Problem: OSPF neighbors not forming

Possible Causes:

- Network statements incorrect or missing

- Area mismatch

- Serial interfaces down

- Network type mismatch

- Authentication mismatch (if configured)

Solution:

- Verify network statements include all interfaces

- Check wildcard masks are correct

- Ensure all networks in area 0

- Verify interfaces are up: show ip interface brief

- Check OSPF configuration: show ip protocols

Problem: SSH connection refused or times out

Possible Causes:

- SSH not configured on router

- RSA keys not generated

- VTY lines not configured for SSH

- Incorrect username/password

- IP connectivity issue

Solution:

- Verify domain name configured

- Check RSA keys exist: show crypto key mypubkey rsa

- Verify VTY lines: show running-config | section line vty

- Test IP connectivity first: ping [router-ip]

- Check SSH status: show ip ssh

Problem: Port security violation causing port shutdown

Possible Causes:

- Different device connected to secured port

- MAC address table full

- Correct device but MAC changed (rare)

Solution:

- Check port status: show interface fa0/2 status

- View violation details: show port-security interface fa0/2

- Clear violation and re-enable port:
interface fa0/2
shutdown
no shutdown

- Reconnect authorized device only

Problem: Wireless devices cannot connect to access points

Possible Causes:

- Wireless module not installed in laptop

- SSID or password incorrect

- Access point not configured

- Access point not connected to switch

- VLAN assignment issue

Solution:

- Verify WPC300N module installed in laptop

- Check SSID and password match AP configuration

- Verify AP Port 1 settings in Config tab

- Check AP physical connection to switch

- Verify switch port in correct VLAN