SOHO Network Design & Implementation Lab Worksheet

Enterprise Network Project #2

Student Name: ______________________Date: _______________

Lab Objectives

By the end of this lab, you will be able to:

- Design a Small Office/Home Office (SOHO) network topology

- Perform subnetting calculations for multiple departments

- Configure VLANs on Cisco switches

- Implement Inter-VLAN routing using "Router on a Stick"

- Configure DHCP server on a Cisco router

- Set up wireless access points for each department

- Verify network connectivity across different VLANs

Network Scenario

XYZ Company is a fast-growing company in Eastern Australia with more than 2 million customers globally. The company deals with selling and buying food items, which are operated from headquarters. The company is opening a branch near the local village of Bon Alba and requires a young IT graduate to design the network for the branch.

Important: The branch network is intended to operate separately from the HQ network.

Network Requirements

- One Cisco router (Model: 2911) and one Cisco switch for the small network

- Three departments:

  - Admin/IT

  - Finance/HR

  - Customer Service/Reception

- Each department must be in a different VLAN

- Each department requires a wireless network for users

- Host devices must obtain IPv4 addresses automatically (DHCP)

- Devices in all departments must be able to communicate with each other (Inter-VLAN routing)

- Base Network: 192.168.1.0/24 (provided by ISP)

Part 1: Network Design

Task 1.1: Create Network Topology in Cisco Packet Tracer

Equipment Required:

- 1 x Cisco 2911 Router

- 1 x Cisco Switch (2960)

- 3 x PCs (one per department)

- 3 x Printers (one per department)

- 3 x Wireless Access Points (one per department)

- Appropriate cables (straight-through and trunk)

Instructions:

- Open Cisco Packet Tracer

- Place devices on the workspace:

  - Add one 2911 Router

  - Add one 2960 Switch

  - Add three PCs (label: PC0, PC1, PC2)

  - Add three Printers (one per department)

  - Add three Wireless Access Points (one per department)

- Cable Connections:

  - Router GigabitEthernet 0/0 ↔ Switch FastEthernet 0/1 (trunk link)

  - Switch FastEthernet 0/2 ↔ PC0 (Admin/IT)

  - Switch FastEthernet 0/3 ↔ Printer0 (Admin/IT)

  - Switch FastEthernet 0/4 ↔ Access Point0 (Admin/IT)

  - Switch FastEthernet 0/5 ↔ PC1 (Finance/HR)

  - Switch FastEthernet 0/6 ↔ Printer1 (Finance/HR)

  - Switch FastEthernet 0/7 ↔ Access Point1 (Finance/HR)

  - Switch FastEthernet 0/8 ↔ PC2 (Customer Service/Reception)

  - Switch FastEthernet 0/9 ↔ Printer2 (Customer Service/Reception)

  - Switch FastEthernet 0/10 ↔ Access Point2 (Customer Service/Reception)

- Add Labels/Comments:

  - Admin/IT Department - VLAN 10

  - Finance/HR Department - VLAN 20

  - Customer Service/Reception - VLAN 30

Checkpoint Question 1: Why is the link between the router and switch configured as a trunk?

Answer:

Part 2: Subnetting Calculations

Calculate Subnet Information

| Subnet | Department | Network ID | First Usable | Last Usable | Broadcast ID | Subnet Mask |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Admin/IT (VLAN 10) | 192.168.1.0 | 192.168.1.1 | 192.168.1.62 | 192.168.1.63 | 255.255.255.192 (/26) |
| 2 | Finance/HR (VLAN 20) | 192.168.1.64 | 192.168.1.65 | 192.168.1.126 | 192.168.1.127 | 255.255.255.192 (/26) |
| 3 | Customer Service/Reception (VLAN 30) | 192.168.1.128 | 192.168.1.129 | 192.168.1.190 | 192.168.1.191 | 255.255.255.192 (/26) |

Table 1: Subnet Information Table

Checkpoint Question 2: How many usable host addresses are available per subnet?

Answer:

Part 3: Switch Configuration - VLANs

Task 3.1: Configure VLANs on Switch

Access the Switch CLI:
Switch> enable
Switch# configure terminal
Switch(config)#

Step 1: Configure VLAN 10 (Admin/IT) - Ports Fa0/2-4

Switch(config)# interface range fastEthernet 0/2-4
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 10
% Access VLAN 10 does not exist. Creating VLAN 10
Switch(config-if-range)# exit

Step 2: Configure VLAN 20 (Finance/HR) - Ports Fa0/5-7

Switch(config)# interface range fastEthernet 0/5-7
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 20
% Access VLAN 20 does not exist. Creating VLAN 20
Switch(config-if-range)# exit

Step 3: Configure VLAN 30 (Customer Service/Reception) - Ports Fa0/8-10

Switch(config)# interface range fastEthernet 0/8-10
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 30
% Access VLAN 30 does not exist. Creating VLAN 30
Switch(config-if-range)# exit

Step 4: Configure Trunk Port (Fa0/1 to Router)

Switch(config)# interface fastEthernet 0/1
Switch(config-if)# switchport mode trunk
Switch(config-if)# exit
Switch(config)# exit

Step 5: Save Configuration

Switch# copy running-config startup-config
OR
Switch# write memory

Step 6: Verify VLAN Configuration

Switch# show vlan brief
Switch# show interfaces trunk
Switch# show running-config

Part 4: Router Configuration - Inter-VLAN Routing

Task 4.1: Configure "Router on a Stick"

Access the Router CLI:
Router> enable
Router# configure terminal
Router(config)#

Step 1: Enable the Physical Interface

Router(config)# interface gigabitEthernet 0/0
Router(config-if)# no shutdown
Router(config-if)# exit

Step 2: Create Sub-interface for VLAN 10 (Admin/IT)

Router(config)# interface gigabitEthernet 0/0.10
Router(config-subif)# encapsulation dot1Q 10
Router(config-subif)# ip address 192.168.1.1 255.255.255.192
Router(config-subif)# exit

Step 3: Create Sub-interface for VLAN 20 (Finance/HR)

Router(config)# interface gigabitEthernet 0/0.20
Router(config-subif)# encapsulation dot1Q 20
Router(config-subif)# ip address 192.168.1.65 255.255.255.192
Router(config-subif)# exit

Step 4: Create Sub-interface for VLAN 30 (Customer Service/Reception)

Router(config)# interface gigabitEthernet 0/0.30
Router(config-subif)# encapsulation dot1Q 30
Router(config-subif)# ip address 192.168.1.129 255.255.255.192
Router(config-subif)# exit

Step 5: Verify Sub-interfaces

Router# show ip interface brief
Router# show running-config

Part 5: DHCP Server Configuration

Task 5.1: Configure DHCP Pools on Router

Step 1: Enable DHCP Service

Router(config)# service dhcp

Step 2: Create DHCP Pool for VLAN 10 (Admin/IT)

Router(config)# ip dhcp pool Admin_Pool
Router(dhcp-config)# network 192.168.1.0 255.255.255.192
Router(dhcp-config)# default-router 192.168.1.1
Router(dhcp-config)# dns-server 8.8.8.8
Router(dhcp-config)# exit

Step 3: Create DHCP Pool for VLAN 20 (Finance/HR)

Router(config)# ip dhcp pool Finance_Pool
Router(dhcp-config)# network 192.168.1.64 255.255.255.192
Router(dhcp-config)# default-router 192.168.1.65
Router(dhcp-config)# dns-server 8.8.8.8
Router(dhcp-config)# exit

Step 4: Create DHCP Pool for VLAN 30 (Customer Service/Reception)

Router(config)# ip dhcp pool CustomerService_Pool
Router(dhcp-config)# network 192.168.1.128 255.255.255.192
Router(dhcp-config)# default-router 192.168.1.129
Router(dhcp-config)# dns-server 8.8.8.8
Router(dhcp-config)# exit

Step 5: Exclude Gateway Addresses from DHCP

Router(config)# ip dhcp excluded-address 192.168.1.1
Router(config)# ip dhcp excluded-address 192.168.1.65
Router(config)# ip dhcp excluded-address 192.168.1.129

Step 6: Save Configuration

Router(config)# exit
Router# write memory

Step 7: Verify DHCP Configuration

Router# show ip dhcp pool
Router# show ip dhcp binding

Part 6: Wireless Access Point Configuration

Task 6.1: Configure Access Points

For Admin/IT Access Point (VLAN 10):

- Click on Access Point0

- Navigate to Config tab → Port 1

- SSID: Admin_WiFi

- Authentication: WPA2-PSK

- PSK Pass Phrase: admin@123 (minimum 8 characters)

For Finance/HR Access Point (VLAN 20):

- Click on Access Point1

- Navigate to Config tab → Port 1

- SSID: Finance_WiFi

- Authentication: WPA2-PSK

- PSK Pass Phrase: finance@123

For Customer Service Access Point (VLAN 30):

- Click on Access Point2

- Navigate to Config tab → Port 1

- SSID: Customer_WiFi

- Authentication: WPA2-PSK

- PSK Pass Phrase: customer@123

Part 7: Host Configuration & Testing

Task 7.1: Configure PCs for DHCP

For each PC (PC0, PC1, PC2):

- Click on the PC

- Navigate to Desktop tab → IP Configuration

- Select DHCP

- Wait for IP address assignment

- Record the assigned IP addresses below:

| Device | VLAN | Assigned IP Address | Default Gateway | Subnet Mask |
| --- | --- | --- | --- | --- |
| PC0 (Admin/IT) | 10 | __________________ | ________________ | ________________ |
| PC1 (Finance/HR) | 20 | __________________ | ________________ | ________________ |
| PC2 (Customer Service) | 30 | __________________ | ________________ | ________________ |

Table 2: DHCP Assignment Record Table

Task 7.2: Connectivity Testing

Test 1: Verify DHCP Assignment

- From each PC, open Command Prompt

- Type: ipconfig

- Verify IP address is within correct subnet range

Test 2: Test Default Gateway

- From PC0: ping 192.168.1.1

- From PC1: ping 192.168.1.65

- From PC2: ping 192.168.1.129

Results:

- PC0 to Gateway: Success ☐ / Fail ☐

- PC1 to Gateway: Success ☐ / Fail ☐

- PC2 to Gateway: Success ☐ / Fail ☐

Test 3: Inter-VLAN Communication

- From PC0 (VLAN 10): Ping PC1 (VLAN 20)

  - Command: ping [PC1\_IP\_Address]

  - Result: Success ☐ / Fail ☐

- From PC0 (VLAN 10): Ping PC2 (VLAN 30)

  - Command: ping [PC2\_IP\_Address]

  - Result: Success ☐ / Fail ☐

- From PC1 (VLAN 20): Ping PC2 (VLAN 30)

  - Command: ping [PC2\_IP\_Address]

  - Result: Success ☐ / Fail ☐

Test 4: Wireless Connectivity (Optional)

- Add laptops/tablets to each VLAN

- Connect to respective WiFi networks

- Verify DHCP assignment

- Test connectivity

Lab Questions

Q1: What technology allows devices in different VLANs to communicate with each other?

Answer:

Q2: Why did we create sub-interfaces on the router instead of using multiple physical interfaces?

Answer:

Q3: What is the purpose of excluding IP addresses from the DHCP pool?

Answer:

Q4: How many total usable host addresses are available across all three subnets?

Answer:

Q5: What would happen if the trunk link between the switch and router failed?

Answer:

Q6: If you wanted to add a fourth department (VLAN 40), what would you need to configure?

Answer:

Troubleshooting Guide

Issue: PC not receiving DHCP address

Solutions:

- Verify VLAN assignment on switch port

- Check DHCP pool configuration on router

- Ensure sub-interface is up (no shutdown)

- Verify trunk link between switch and router

Issue: Cannot ping between VLANs

Solutions:

- Verify Inter-VLAN routing configuration

- Check sub-interface IP addresses

- Verify encapsulation dot1Q on sub-interfaces

- Test gateway connectivity first

Issue: Wireless clients cannot connect

Solutions:

- Verify Access Point is powered on

- Check SSID and password configuration

- Ensure Access Point port is in correct VLAN

- Verify WPA2-PSK authentication settings