Enterprise pfSense Firewall Integration Lab Manual

Cisco 1911 Router and pfSense 2.8.1 Point-to-Point Configuration

Lab Objective: Deploy and integrate a pfSense firewall with Cisco enterprise infrastructure using a point-to-point link, configure static routing for VLAN segmentation, and establish complete end-to-end connectivity.

Table of Contents

- Lab Topology Overview

- Hardware Requirements and Pre-Staging

- Network Architecture and IP Addressing

- Phase 1: Hardware Assembly and pfSense Installation

- Phase 2: Cisco 1911 Router Configuration

- Phase 3: Physical Connectivity and Cabling

- Phase 4: pfSense Gateway and Static Route Configuration

- Phase 5: Firewall Rule Configuration

- Phase 6: Testing and Verification

- Phase 7: Security Hardening

- Troubleshooting Guide

- Appendix: Command Reference

Lab Topology Overview

Network Diagram

[Internet/CyberTex Network]
|
| WAN (em0)
| DHCP or Static
|
+-------+--------+
| pfSense FW |
| 10.0.0.1/30 |
+-------+--------+
| LAN (vr0)
| Bottom Port
|
| Point-to-Point Link
| 10.0.0.0/30
|
| G0/1 (10.0.0.2/30)
+-------+--------+
| Cisco 1911 RTR |
+-------+--------+
|
| Multiple VLANs
|
+-------+--------+
| Layer 2/3 SW |
+----------------+
|
[VLAN 10] [VLAN 20] [VLAN 30]

Logical Network Architecture

| Device | Interface | IP Address | Purpose |
| --- | --- | --- | --- |
| pfSense FW | em0 (WAN) | DHCP/Static | Internet connectivity |
| pfSense FW | vr0 (LAN) | 10.0.0.1/30 | P2P to router |
| Cisco 1911 | G0/1 | 10.0.0.2/30 | P2P to firewall |
| Cisco 1911 | G0/0 | TBD | VLAN trunk port |
| VLAN 10 | SVI | 192.168.10.1/24 | Management VLAN |
| VLAN 20 | SVI | 192.168.20.1/24 | User VLAN |
| VLAN 30 | SVI | 192.168.30.1/24 | Server VLAN |

Table 1: IP Addressing Scheme

Note: Adjust VLAN subnet addressing based on your specific requirements. The example uses 192.168.x.0/24 networks.

Hardware Requirements and Pre-Staging

Equipment List

- pfSense Firewall:

  - Desktop tower chassis

  - VIA VT6105 Rhine III Ethernet expansion card (installed)

  - Dual-port network configuration (em0 WAN, vr0 LAN)

  - pfSense 2.8.1-RELEASE (amd64) installed

- Cisco 1911 Router:

  - Integrated Services Router with minimum 2 GigabitEthernet ports

  - Console cable for initial configuration

  - Cisco IOS version 15.x or higher recommended

- Layer 2/3 Switch:

  - VLAN-capable switch (Cisco Catalyst recommended)

  - Minimum 3 VLANs supported

- Cabling:

  - Cat5e/Cat6 Ethernet cables (minimum 3)

  - Cisco console cable (USB or DB9)

  - Cable labels/tags for identification

- Management Workstation:

  - Laptop/desktop with terminal emulation software (PuTTY, SecureCRT, TeraTerm)

  - Web browser for pfSense GUI access

  - Network adapter for testing

Pre-Staging Status

The following components are already completed:

- VIA VT6105 Rhine III NIC physically installed in pfSense firewall

- LAN port (bottom port, expansion card) physically labeled

- Ethernet cable connected to LAN port for identification

- pfSense 2.8.1-RELEASE operating system installed

- WAN interface (em0) configured for DHCP

- LAN interface (vr0) configured with 10.0.0.1/30

- DHCP service disabled on LAN interface

- Default credentials set (admin/pfsense)

Network Architecture and IP Addressing

Point-to-Point Link Design

The pfSense firewall and Cisco router are connected via a point-to-point link using a /30 subnet (255.255.255.252), which provides exactly 2 usable host addresses[1].

Subnet Calculation:

- Network: 10.0.0.0/30

- Subnet Mask: 255.255.255.252

- Usable IPs: 10.0.0.1 and 10.0.0.2

- Broadcast: 10.0.0.3

| Parameter | Value |
| --- | --- |
| Network Address | 10.0.0.0 |
| First Usable IP (pfSense) | 10.0.0.1 |
| Second Usable IP (Cisco) | 10.0.0.2 |
| Broadcast Address | 10.0.0.3 |
| Subnet Mask | 255.255.255.252 |
| CIDR Notation | /30 |

Table 2: Point-to-Point Link Addressing

VLAN Subnet Planning

Adjust these subnets based on your organization's IP addressing scheme:

| VLAN ID | Network | Gateway | Purpose |
| --- | --- | --- | --- |
| 10 | 192.168.10.0/24 | 192.168.10.1 | Management |
| 20 | 192.168.20.0/24 | 192.168.20.1 | User Network |
| 30 | 192.168.30.0/24 | 192.168.30.1 | Server Network |

Table 3: VLAN Addressing Scheme

Routing Requirements

Traffic Flow Logic:

- Outbound (Internal → Internet):

  - Hosts in VLANs 10, 20, 30 send traffic to their default gateway (router)

  - Router forwards to pfSense (10.0.0.1) via static default route

  - pfSense NATs traffic and sends to Internet via WAN interface

- Inbound (Internet → Internal):

  - pfSense receives return traffic on WAN interface

  - pfSense checks routing table for destination VLAN networks

  - Static routes direct traffic to router (10.0.0.2)

  - Router routes to appropriate VLAN

Phase 1: Hardware Assembly and pfSense Installation

Status: ✓ COMPLETED (Pre-staged)

This phase has been completed during the initial build. The following summary documents what was performed:

Step 1.1: Physical NIC Installation

- Power down desktop chassis completely

- Open chassis side panel

- Locate available PCI expansion slot

- Install VIA VT6105 Rhine III Ethernet adapter

- Secure card with retention screw

- Close chassis and reconnect power

Step 1.2: pfSense Installation

- Download pfSense 2.8.1-RELEASE ISO from Netgate

- Create bootable USB installer

- Boot system from USB media

- Follow installation wizard:

  - Accept license agreement

  - Install pfSense on local disk

  - Select default partition scheme

  - Reboot when complete

Step 1.3: Interface Assignment

Console configuration performed during initial setup:

Available interfaces:
em0: (WAN)
vr0: (LAN - VIA VT6105 expansion card)

Do you want to configure VLANs now? n
Enter WAN interface name: em0
Enter LAN interface name: vr0

Step 1.4: IP Address Configuration

Console menu configuration:

Select option 2: Set interface IP address

LAN interface configuration:
Configure IPv4 address via DHCP? n
Enter new LAN IPv4 address: 10.0.0.1
Enter LAN subnet bit count: 30
Configure IPv6 address via DHCP6? n
Enable DHCP server on LAN? n

Step 1.5: Physical Port Labeling

- WAN port (em0): Top port on motherboard - labeled "WAN - TO INTERNET"

- LAN port (vr0): Bottom port on expansion card - labeled "LAN - TO ROUTER"

- Ethernet cable connected to LAN port for easy identification

Verification Checkpoint:

- [ ] pfSense boots successfully

- [ ] Console menu accessible

- [ ] WAN interface shows IP address (DHCP or static)

- [ ] LAN interface shows 10.0.0.1/30

- [ ] Physical ports clearly labeled

Phase 2: Cisco 1911 Router Configuration

Step 2.1: Initial Router Access

Connect to the Cisco 1911 router via console:

- Connect console cable from laptop to router console port

- Open terminal emulator (PuTTY/TeraTerm/SecureCRT)

- Settings: 9600 baud, 8 data bits, no parity, 1 stop bit, no flow control

- Power on router

- Press Enter to access console

Step 2.2: Basic Router Configuration

Enter privileged EXEC mode and global configuration:

Router> enable
Router# configure terminal
Router(config)# hostname R1
R1(config)#

Step 2.3: Configure Point-to-Point Interface (G0/1)

Configure the interface connecting to pfSense:

R1(config)# interface GigabitEthernet0/1
R1(config-if)# description *** P2P LINK TO PFSENSE FIREWALL ***
R1(config-if)# ip address 10.0.0.2 255.255.255.252
R1(config-if)# no shutdown
R1(config-if)# exit
R1(config)#

Verification:

R1# show ip interface brief
Interface IP-Address OK? Method Status Protocol
GigabitEthernet0/0 unassigned YES NVRAM administratively down down
GigabitEthernet0/1 10.0.0.2 YES manual up up

Step 2.4: Configure VLAN Trunk Interface (G0/0)

If using a Layer 2 switch with VLANs, configure G0/0 as a trunk port with sub-interfaces:

R1(config)# interface GigabitEthernet0/0
R1(config-if)# description *** TRUNK TO LAYER 2 SWITCH ***
R1(config-if)# no shutdown
R1(config-if)# exit
R1(config)#

! Configure VLAN 10 sub-interface
R1(config)# interface GigabitEthernet0/0.10
R1(config-subif)# description *** MANAGEMENT VLAN ***
R1(config-subif)# encapsulation dot1Q 10
R1(config-subif)# ip address 192.168.10.1 255.255.255.0
R1(config-subif)# exit

! Configure VLAN 20 sub-interface
R1(config)# interface GigabitEthernet0/0.20
R1(config-subif)# description *** USER VLAN ***
R1(config-subif)# encapsulation dot1Q 20
R1(config-subif)# ip address 192.168.20.1 255.255.255.0
R1(config-subif)# exit

! Configure VLAN 30 sub-interface
R1(config)# interface GigabitEthernet0/0.30
R1(config-subif)# description *** SERVER VLAN ***
R1(config-subif)# encapsulation dot1Q 30
R1(config-subif)# ip address 192.168.30.1 255.255.255.0
R1(config-subif)# exit
R1(config)#

Alternative: Layer 3 Switch Configuration

If using a Layer 3 switch for inter-VLAN routing instead of router sub-interfaces:

R1(config)# interface GigabitEthernet0/0
R1(config-if)# description *** ROUTED PORT TO L3 SWITCH ***
R1(config-if)# ip address 192.168.100.1 255.255.255.252
R1(config-if)# no shutdown
R1(config-if)# exit

For this lab, we'll proceed with the router-on-a-stick configuration using sub-interfaces.

Step 2.5: Configure Default Route to pfSense

Add a default route pointing all Internet-bound traffic to the pfSense firewall:

R1(config)# ip route 0.0.0.0 0.0.0.0 10.0.0.1
R1(config)# exit
R1#

Verification:

R1# show ip route
Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP
D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

Gateway of last resort is 10.0.0.1 to network 0.0.0.0

S* 0.0.0.0/0 [1/0] via 10.0.0.1
10.0.0.0/8 is variably subnetted, 2 subnets, 2 masks
C 10.0.0.0/30 is directly connected, GigabitEthernet0/1
L 10.0.0.2/32 is directly connected, GigabitEthernet0/1
192.168.10.0/24 is variably subnetted, 2 subnets, 2 masks
C 192.168.10.0/24 is directly connected, GigabitEthernet0/0.10
L 192.168.10.1/32 is directly connected, GigabitEthernet0/0.10
192.168.20.0/24 is variably subnetted, 2 subnets, 2 masks
C 192.168.20.0/24 is directly connected, GigabitEthernet0/0.20
L 192.168.20.1/32 is directly connected, GigabitEthernet0/0.20
192.168.30.0/24 is variably subnetted, 2 subnets, 2 masks
C 192.168.30.0/24 is directly connected, GigabitEthernet0/0.30
L 192.168.30.1/32 is directly connected, GigabitEthernet0/0.30

Step 2.6: Enable IP Routing (Verification)

Ensure IP routing is enabled (default on Cisco routers):

R1# show running-config | include ip routing
ip routing

If not present, enable it:

R1(config)# ip routing

Step 2.7: Save Configuration

CRITICAL: Save the configuration to prevent loss on reboot:

R1# copy running-config startup-config
Destination filename [startup-config]? [Press Enter]
Building configuration...
[OK]
R1#

Verification Checkpoint:

- [ ] GigabitEthernet0/1 configured with 10.0.0.2/30

- [ ] GigabitEthernet0/0 sub-interfaces configured for VLANs 10, 20, 30

- [ ] Default route points to 10.0.0.1 (pfSense)

- [ ] All interfaces show "up/up" status

- [ ] Configuration saved to startup-config

Phase 3: Physical Connectivity and Cabling

Step 3.1: Power Down Equipment (Safety)

- Gracefully shut down pfSense firewall (console option 6)

- Power off Cisco router

- Disconnect power from all devices

Step 3.2: Cable Connection - Firewall to Router

- Locate the labeled cable on pfSense LAN port (bottom port, vr0)

- Connect the cable to Cisco 1911 GigabitEthernet0/1 port

- Ensure connection is secure

- Verify cable label: "PFSENSE LAN TO ROUTER G0/1"

Step 3.3: Cable Connection - Router to Switch

- Connect Ethernet cable from Cisco 1911 GigabitEthernet0/0 to switch trunk port

- Label cable: "ROUTER G0/0 TO SWITCH TRUNK"

- Ensure connection is secure

Step 3.4: Cable Connection - WAN Uplink

- Connect Ethernet cable from pfSense WAN port (em0, top port) to upstream network

- Label cable: "PFSENSE WAN TO CYBERTEX NETWORK"

- Ensure connection is secure

Step 3.5: Power Up Sequence

Power devices in the following order to prevent routing loops:

- Switch: Power on Layer 2/3 switch first, wait 2-3 minutes for boot

- Router: Power on Cisco 1911, wait for boot completion (watch console)

- Firewall: Power on pfSense, wait for boot completion

- Test Device: Connect laptop to VLAN 10 switch port for testing

Step 3.6: Verify Physical Link Status

On Cisco Router:

R1# show interfaces GigabitEthernet0/1 | include line protocol
GigabitEthernet0/1 is up, line protocol is up

R1# show interfaces status
Port Name Status Vlan Duplex Speed Type
Gi0/0 TRUNK TO SWITCH connected trunk a-full a-1000 RJ45
Gi0/1 P2P TO PFSENSE connected routed a-full a-1000 RJ45

On pfSense Console:

Select option 7: Ping host

Enter hostname or IP: 10.0.0.2

PING 10.0.0.2 (10.0.0.2): 56 data bytes
64 bytes from 10.0.0.2: icmp_seq=0 ttl=255 time=1.234 ms
64 bytes from 10.0.0.2: icmp_seq=1 ttl=255 time=0.987 ms

If ping succeeds, physical connectivity is established.

Verification Checkpoint:

- [ ] All cables securely connected and labeled

- [ ] Link lights active on all ports

- [ ] Router shows interfaces "up/up"

- [ ] pfSense can ping 10.0.0.2 (router)

- [ ] Router can ping 10.0.0.1 (pfSense)

Phase 4: pfSense Gateway and Static Route Configuration

Step 4.1: Access pfSense Web Interface

- Connect management laptop to VLAN 10 switch port

- Configure laptop with static IP:

  - IP Address: 192.168.10.100

  - Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.10.1

- Open web browser

- Navigate to: https://10.0.0.1

- Accept self-signed certificate warning

- Login credentials:

  - Username: admin

  - Password: pfsense

Alternative Access Method:

If direct access to 10.0.0.1 is not possible from VLAN 10, temporarily connect laptop directly to pfSense LAN port using a second cable, configure laptop with 10.0.0.10/30, then access https://10.0.0.1.

Step 4.2: Add Gateway for Router

Before adding static routes, you must define the router as a gateway[2]:

- Navigate to: System > Routing

- Click the Gateways tab

- Click + Add button

- Configure the gateway with the following settings:

| Parameter | Value |
| --- | --- |
| Interface | LAN |
| Address Family | IPv4 |
| Name | CISCO_ROUTER_GW |
| Gateway | 10.0.0.2 |
| Gateway Monitoring | Unchecked (optional) |
| Description | Gateway to Cisco 1911 Router for VLAN routing |

Table 4: Gateway Configuration Parameters

- Click Save

- Click Apply Changes

Verification:

Gateway should appear in the gateway list with status "Online" (if monitoring enabled) or "Pending" (if monitoring disabled).

Step 4.3: Add Static Route for VLAN 10

- Navigate to: System > Routing

- Click the Routes tab

- Click + Add button

- Configure static route:

| Parameter | Value |
| --- | --- |
| Destination Network | 192.168.10.0/24 |
| Gateway | CISCO_ROUTER_GW - 10.0.0.2 |
| Disabled | Unchecked |
| Description | Route to Management VLAN 10 |

Table 5: VLAN 10 Static Route

- Click Save

- Click Apply Changes

Step 4.4: Add Static Route for VLAN 20

Repeat the process for VLAN 20:

- Navigate to: System > Routing > Routes

- Click + Add

- Configure:

  - Destination Network: 192.168.20.0/24

  - Gateway: CISCO_ROUTER_GW - 10.0.0.2

  - Description: Route to User VLAN 20

- Click Save

- Click Apply Changes

Step 4.5: Add Static Route for VLAN 30

Repeat the process for VLAN 30:

- Navigate to: System > Routing > Routes

- Click + Add

- Configure:

  - Destination Network: 192.168.30.0/24

  - Gateway: CISCO_ROUTER_GW - 10.0.0.2

  - Description: Route to Server VLAN 30

- Click Save

- Click Apply Changes

Step 4.6: Verify Routing Table

- Navigate to: Diagnostics > Routes

- Verify the following routes are present:

| Destination | Gateway | Flags |
| --- | --- | --- |
| 192.168.10.0/24 | 10.0.0.2 | UGS |
| 192.168.20.0/24 | 10.0.0.2 | UGS |
| 192.168.30.0/24 | 10.0.0.2 | UGS |
| 10.0.0.0/30 | link#2 | U |
| default | [WAN Gateway] | UGS |

Table 6: Expected Routing Table Entries

Alternative Verification via Console:

Select option 8: Shell

% netstat -rn | grep 192.168
192.168.10.0/24 10.0.0.2 UGS 0 0 vr0
192.168.20.0/24 10.0.0.2 UGS 0 0 vr0
192.168.30.0/24 10.0.0.2 UGS 0 0 vr0

Step 4.7: Test Gateway Connectivity

- Navigate to: Diagnostics > Ping

- Enter hostname: 10.0.0.2

- Click Ping

- Verify successful responses

Expected output:

PING 10.0.0.2 (10.0.0.2): 56 data bytes
64 bytes from 10.0.0.2: icmp_seq=0 ttl=255 time=1.234 ms
64 bytes from 10.0.0.2: icmp_seq=1 ttl=255 time=0.987 ms
64 bytes from 10.0.0.2: icmp_seq=2 ttl=255 time=1.056 ms

--- 10.0.0.2 ping statistics ---
3 packets transmitted, 3 packets received, 0.0% packet loss

Verification Checkpoint:

- [ ] Gateway "CISCO_ROUTER_GW" created and online

- [ ] Three static routes added (VLANs 10, 20, 30)

- [ ] Routes visible in routing table

- [ ] Successful ping to 10.0.0.2 from pfSense

- [ ] Successful ping to 192.168.10.1, 192.168.20.1, 192.168.30.1 from pfSense

Phase 5: Firewall Rule Configuration

By default, pfSense blocks all traffic not explicitly allowed[3]. You must create firewall rules to permit traffic between VLANs and to the Internet.

Step 5.1: Understanding pfSense Firewall Rule Logic

Key Concepts:

- Rules are processed top-down (first match wins)

- Rules are applied on the inbound interface

- Default behavior: deny all unless explicitly allowed

- LAN interface has default "allow all" rule (can be modified)

- WAN interface blocks all inbound by default (good security practice)

Traffic Flow Through pfSense:

- Packet arrives on interface (e.g., LAN)

- pfSense evaluates rules on that interface from top to bottom

- First matching rule is applied (allow or block)

- If packet is allowed, state is created for return traffic

- NAT is applied if configured (outbound to WAN)

Step 5.2: Configure LAN Interface Firewall Rules

Navigate to: Firewall > Rules > LAN

By default, you should see a rule allowing all traffic from LAN to any destination. This rule permits traffic from the 10.0.0.0/30 point-to-point link.

Verify Default Rule:

| Action | Protocol | Source | Destination | Description |
| --- | --- | --- | --- | --- |
| Pass | Any | LAN net | Any | Default allow LAN to any |

Table 7: Default LAN Rule

This rule is sufficient for basic connectivity. For enhanced security, you can create more specific rules.

Step 5.3: Create Specific Rules for VLAN Networks (Optional)

For more granular control, create explicit rules for each VLAN network:

Rule 1: Allow VLAN 10 to Internet

- Click + Add (add rule to top)

- Configure:

  - Action: Pass

  - Interface: LAN

  - Address Family: IPv4

  - Protocol: Any

  - Source: Network - 192.168.10.0/24

  - Destination: Any

  - Description: Allow VLAN 10 Management to Internet

- Click Save

Rule 2: Allow VLAN 20 to Internet

Repeat with:

- Source: 192.168.20.0/24

- Description: Allow VLAN 20 Users to Internet

Rule 3: Allow VLAN 30 to Internet

Repeat with:

- Source: 192.168.30.0/24

- Description: Allow VLAN 30 Servers to Internet

Click Apply Changes after creating all rules.

Step 5.4: Configure Outbound NAT (Verification)

Verify that Outbound NAT includes your VLAN networks:

- Navigate to: Firewall > NAT

- Click the Outbound tab

- Mode should be: Automatic outbound NAT rule generation

If using Manual Outbound NAT, ensure rules exist for:

- 192.168.10.0/24

- 192.168.20.0/24

- 192.168.30.0/24

Step 5.5: Handle Asymmetric Routing (Advanced)

Because traffic from VLANs to each other does not pass through pfSense (handled by the Cisco router), you may encounter asymmetric routing issues[2].

Option A: Enable Automatic Bypass (Recommended for Lab)

- Navigate to: System > Advanced

- Click the Firewall & NAT tab

- Scroll to Firewall Optimization Options

- Check: Bypass firewall rules for traffic on the same interface

- Click Save

This automatically creates rules to handle asymmetric routing for statically routed networks.

Option B: Manual Firewall Rules (Production Environment)

If you need specific control, create manual rules:

Floating Rule for Asymmetric Traffic:

- Navigate to: Firewall > Rules > Floating

- Click + Add

- Configure:

  - Action: Pass

  - Quick: Checked

  - Interface: LAN

  - Direction: Out

  - Protocol: TCP

  - Source: 192.168.10.0/24, 192.168.20.0/24, 192.168.30.0/24 (use aliases)

  - Destination: 192.168.10.0/24, 192.168.20.0/24, 192.168.30.0/24

  - Advanced: TCP Flags - Any flags

  - Advanced: State Type - Sloppy State

  - Description: Allow asymmetric VLAN-to-VLAN traffic

- Click Save

- Click Apply Changes

Step 5.6: Create Aliases for Simplified Management (Best Practice)

Using aliases makes rule management easier and more scalable[3]:

- Navigate to: Firewall > Aliases

- Click + Add

- Create alias for internal networks:

  - Name: RFC1918_Internal_VLANs

  - Type: Networks

  - Network 1: 192.168.10.0/24 - Description: VLAN 10

  - Network 2: 192.168.20.0/24 - Description: VLAN 20

  - Network 3: 192.168.30.0/24 - Description: VLAN 30

- Click Save

- Click Apply Changes

Now you can use this alias in firewall rules instead of typing individual networks.

Verification Checkpoint:

- [ ] LAN interface has appropriate firewall rules

- [ ] Outbound NAT configured for VLAN networks

- [ ] Asymmetric routing handled (bypass enabled or manual rules)

- [ ] Aliases created for simplified management

- [ ] All changes applied

Phase 6: Testing and Verification

Step 6.1: End-to-End Connectivity Tests

Test 1: VLAN Device to Internet

- Connect test PC to VLAN 10 switch port

- Configure PC with DHCP or static IP (192.168.10.100/24, gateway 192.168.10.1)

- Test DNS resolution:

ping 8.8.8.8
ping google.com

Expected: Successful replies

Test 2: VLAN Device to pfSense

ping 10.0.0.1

Expected: Successful replies (if firewall rules permit ICMP)

Test 3: Inter-VLAN Connectivity

Connect PC to VLAN 10, ping device in VLAN 20:

ping 192.168.20.100

Expected: Successful replies (traffic routed by Cisco router, not pfSense)

Step 6.2: Verify NAT Functionality

- From VLAN device, browse to: https://whatismyip.com

- Verify displayed IP is pfSense WAN address (not internal VLAN IP)

- This confirms NAT is working correctly

Step 6.3: Check pfSense State Table

- Navigate to: Diagnostics > States

- Filter by source: 192.168.10.100

- Verify states are created for outbound connections

Example state entry:

192.168.10.100:54321 -> 8.8.8.8:443 (WAN:1.2.3.4:54321)

This shows:

- Internal IP: 192.168.10.100

- Destination: 8.8.8.8 (Google DNS)

- NATted to: WAN IP 1.2.3.4

Step 6.4: Monitor Firewall Logs

- Navigate to: \textbf{Status > System Logs > Firewall**

- Generate test traffic from VLAN device

- Verify traffic is being passed (green indicators)

- Look for any blocked traffic (red indicators) that should be allowed

Step 6.5: Verify Routing from Cisco Router

On Cisco router console:

R1# show ip route
Codes: L - local, C - connected, S - static

Gateway of last resort is 10.0.0.1 to network 0.0.0.0

S* 0.0.0.0/0 [1/0] via 10.0.0.1
10.0.0.0/8 is variably subnetted, 2 subnets, 2 masks
C 10.0.0.0/30 is directly connected, GigabitEthernet0/1
L 10.0.0.2/32 is directly connected, GigabitEthernet0/1
192.168.10.0/24 is variably subnetted, 2 subnets, 2 masks
C 192.168.10.0/24 is directly connected, GigabitEthernet0/0.10
L 192.168.10.1/32 is directly connected, GigabitEthernet0/0.10

Step 6.6: Traceroute Analysis

From VLAN device, perform traceroute to external destination:

traceroute google.com

Expected path:

- 192.168.10.1 (default gateway - router)

- 10.0.0.1 (pfSense firewall)

- [WAN gateway] (upstream router)

- [Internet hops...]

This confirms traffic flow: VLAN → Router → pfSense → Internet

Step 6.7: Bandwidth and Performance Testing

- Use iperf3 or similar tool to test throughput

- From VLAN device to Internet server

- Monitor pfSense CPU/memory usage: Status > Dashboard

- Verify acceptable performance for your requirements

Step 6.8: Failover Testing (Optional)

Test routing behavior during failures:

Test A: Disconnect Router Link

- Disconnect cable from G0/1 on Cisco router

- Verify VLAN devices lose Internet connectivity (expected)

- Verify pfSense detects gateway down (if monitoring enabled)

- Reconnect cable, verify automatic recovery

Test B: pfSense WAN Failure Simulation

- Disconnect WAN cable from pfSense

- Verify internal routing still works (VLAN to VLAN)

- Verify Internet connectivity fails (expected)

- Reconnect, verify recovery

Verification Checkpoint:

- [ ] VLAN devices can reach Internet

- [ ] DNS resolution working

- [ ] NAT functioning correctly

- [ ] State table shows active connections

- [ ] Traceroute shows correct path

- [ ] Firewall logs show passed traffic

- [ ] Inter-VLAN routing works (router-handled)

- [ ] Performance acceptable

Phase 7: Security Hardening

Step 7.1: Change Default Credentials

CRITICAL SECURITY STEP:

- Navigate to: System > User Manager

- Click pencil icon next to admin user

- Click + Add under User Certificates (optional, for key-based auth)

- Change password:

  - Password: [Strong password, minimum 12 characters]

  - Confirm Password: [Same]

- Click Save

Document the new password in a secure password manager.

Step 7.2: Disable SSH on WAN Interface

- Navigate to: System > Advanced

- Click Admin Access tab

- Uncheck: Enable Secure Shell (if not needed)

- Or configure SSH to listen only on LAN:

  - Check Enable Secure Shell

  - SSH Interfaces: Select LAN only (not WAN)

- Click Save

Step 7.3: Enable HTTPS Only for WebGUI

- Navigate to: System > Advanced > Admin Access

- Protocol: HTTPS

- Uncheck: Disable HTTP Redirect (keep HTTP redirect enabled for security)

- WebGUI listen interfaces: Select LAN only

- Click Save

Step 7.4: Configure Logging

- Navigate to: Status > System Logs > Settings

- Configure:

  - Log Message Format: syslog (RFC 5424, with RFC 3339 microsecond-precision timestamps)

  - Log Firewall Default Blocks: Checked

  - Log packets matched from the default pass rules: Checked

  - GUI Log Entries to Display: 500

- Optional: Configure remote syslog server for centralized logging

- Click Save

Step 7.5: Enable Automatic Updates

- Navigate to: System > Update

- Review current version: 2.8.1-RELEASE

- Enable: Dashboard check for updates

- Configure update branch: Stable

- Regularly check for security updates

Step 7.6: Configure Backup Schedule

- Navigate to: Diagnostics > Backup & Restore

- Click Download configuration as XML

- Save to secure location

- Schedule regular backups (weekly recommended)

- Optional: Configure AutoConfigBackup for cloud backup

Step 7.7: Implement Two-Factor Authentication (Recommended)

- Navigate to: System > User Manager

- Click Authentication Servers tab

- Click + Add

- Configure TOTP (Time-Based One-Time Password):

  - Type: TOTP

  - Descriptive name: 2FA_TOTP

- Edit admin user, enable 2FA

- Scan QR code with authenticator app (Google Authenticator, Authy)

Step 7.8: Review and Optimize Firewall Rules

Best practices for rule management[3]:

- Top-down order: Place most specific/restrictive rules at top

- Use aliases: Don't hardcode IPs in rules

- Document rules: Every rule should have descriptive comment

- Disable unused rules: Don't delete, disable for audit trail

- Log critical rules: Enable logging for important allow/deny rules

- Regular audits: Review rules quarterly

Step 7.9: Configure DHCP Security (If DHCP Enabled)

If enabling DHCP on VLANs:

- Reserve IPs for critical devices (servers, printers, APs)

- Configure DHCP static mappings based on MAC addresses

- Set appropriate lease times (4 hours for user devices, 24 hours for servers)

- Configure DNS servers (internal or public like 1.1.1.1, 8.8.8.8)

Step 7.10: Harden Cisco Router

On Cisco router console:

R1# configure terminal

! Secure console access
R1(config)# line console 0
R1(config-line)# exec-timeout 5 0
R1(config-line)# logging synchronous
R1(config-line)# exit

! Secure VTY access
R1(config)# line vty 0 4
R1(config-line)# transport input ssh
R1(config-line)# exec-timeout 5 0
R1(config-line)# logging synchronous
R1(config-line)# exit

! Enable password encryption
R1(config)# service password-encryption

! Set enable secret
R1(config)# enable secret [strong-password]

! Disable unused services
R1(config)# no ip http server
R1(config)# no ip http secure-server
R1(config)# no cdp run

! Configure logging
R1(config)# logging buffered 16384
R1(config)# logging console warnings

! Save configuration
R1(config)# exit
R1# copy running-config startup-config

Verification Checkpoint:

- [ ] Default admin password changed

- [ ] SSH configured securely

- [ ] HTTPS-only WebGUI access

- [ ] Logging configured and tested

- [ ] Configuration backup created

- [ ] Automatic updates enabled

- [ ] Two-factor authentication configured

- [ ] Firewall rules reviewed and documented

- [ ] Cisco router hardened

Troubleshooting Guide

Problem: Cannot Access pfSense Web Interface

Symptoms:

- Browser cannot reach https://10.0.0.1

- Connection timeout or "Unable to connect"

Diagnostic Steps:

- Verify laptop IP configuration:

ipconfig /all (Windows)
ip addr show (Linux/Mac)

Ensure IP is in 10.0.0.0/30 range or on VLAN network that can route to 10.0.0.1.

- Test connectivity:

ping 10.0.0.1

- Check pfSense interface status on console:

Select option 1: Assign interfaces
Review interface assignments

- Verify LAN IP address:

Select option 2: Set interface IP address
Select LAN interface
Verify IP is 10.0.0.1/30

Resolution:

- Connect laptop directly to pfSense LAN port

- Configure laptop with 10.0.0.10/30

- Access https://10.0.0.1 directly

Problem: VLANs Cannot Reach Internet

Symptoms:

- VLAN devices can ping default gateway (router)

- Cannot ping 10.0.0.1 or external IPs

- No Internet connectivity

Diagnostic Steps:

- Test from router to pfSense:

R1# ping 10.0.0.1

Type escape sequence to abort.
Sending 5, 100-byte ICMP Echos to 10.0.0.1, timeout is 2 seconds:
!!!!!
Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/2 ms

If this fails, physical connectivity issue between router and firewall.

- Verify router default route:

R1# show ip route | include 0.0.0.0
S* 0.0.0.0/0 [1/0] via 10.0.0.1

If missing, add: ip route 0.0.0.0 0.0.0.0 10.0.0.1

- Check pfSense static routes:

On pfSense WebGUI: Diagnostics > Routes

Verify routes for 192.168.10.0/24, 192.168.20.0/24, 192.168.30.0/24 via 10.0.0.2.

If missing, add via System > Routing > Routes.

- Check firewall rules:

Firewall > Rules > LAN

Ensure rules allow traffic from VLAN networks to any destination.

- Verify NAT configuration:

Firewall > NAT > Outbound

Mode should be "Automatic outbound NAT" or manual rules for VLAN networks exist.

- Check pfSense firewall logs:

Status > System Logs > Firewall

Look for blocked packets from VLAN IPs. Create rules to allow if needed.

Resolution:

- Add missing static routes

- Create firewall allow rules

- Verify NAT configuration

Problem: Asymmetric Routing - Connections Drop

Symptoms:

- Initial connection succeeds then stops

- Intermittent connectivity

- TCP connections work briefly then fail

Diagnostic Steps:

- Check firewall logs for TCP state violations:

Status > System Logs > Firewall

Look for messages: "TCP INVALID STATE"

- Verify traffic flow:

VLAN-to-VLAN traffic should NOT go through pfSense (router handles it directly).

Resolution:

- Enable bypass for same-interface traffic:

System > Advanced > Firewall & NAT

Check: "Bypass firewall rules for traffic on the same interface"

Or create manual floating rules with "Sloppy State" as described in Phase 5.5.

Problem: Cannot Ping Between VLANs

Symptoms:

- VLAN 10 device cannot ping VLAN 20 device

- Each VLAN can reach Internet

- Router is configured correctly

Diagnostic Steps:

- Verify router has sub-interfaces for all VLANs:

R1# show ip interface brief | include Gigabit
GigabitEthernet0/0 unassigned YES NVRAM up up
GigabitEthernet0/0.10 192.168.10.1 YES manual up up
GigabitEthernet0/0.20 192.168.20.1 YES manual up up
GigabitEthernet0/0.30 192.168.30.1 YES manual up up

- Verify IP routing is enabled:

R1# show running-config | include ip routing
ip routing

- Check switch VLAN configuration:

Ensure switch has:

- VLANs 10, 20, 30 created

- Trunk port to router (G0/0) allows all VLANs

- Access ports assigned to correct VLANs

- Test routing from router:

R1# ping 192.168.10.100 source 192.168.20.1

If successful, routing works. If not, issue with router or switch configuration.

Resolution:

- This is normal - inter-VLAN routing handled by Cisco router, not pfSense

- pfSense only routes traffic to/from Internet

- Verify router and switch VLAN configuration

Problem: pfSense Cannot Reach VLAN Gateways

Symptoms:

- pfSense cannot ping 192.168.10.1, 192.168.20.1, 192.168.30.1

- Router can ping pfSense (10.0.0.1)

- VLANs have partial Internet connectivity

Diagnostic Steps:

- Verify router sub-interfaces are up:

R1# show ip interface brief
GigabitEthernet0/0.10 192.168.10.1 YES manual up up

- Check pfSense static routes:

System > Routing > Routes

Verify routes for all VLAN networks via gateway 10.0.0.2.

- Test from pfSense Diagnostics:

Diagnostics > Ping

Try: 10.0.0.2 (router P2P interface) - should work
Try: 192.168.10.1 (router VLAN interface) - may fail

- Check router ACLs or firewall features:

R1# show ip access-lists

Ensure no ACLs blocking ICMP from 10.0.0.1.

Resolution:

- This is partially expected - pfSense routes packets to 10.0.0.2, router delivers to VLANs

- Not critical for functionality, but useful for diagnostics

- If needed, ensure router allows ICMP from pfSense

Problem: High Latency or Packet Loss

Symptoms:

- Slow Internet connectivity from VLANs

- High ping times (>50ms to nearby destinations)

- Intermittent packet drops

Diagnostic Steps:

- Check interface errors on router:

R1# show interfaces GigabitEthernet0/1 | include errors
0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored
0 output errors, 0 collisions, 0 interface resets

Errors indicate cable, duplex, or hardware issues.

- Verify duplex/speed settings:

R1# show interfaces GigabitEthernet0/1 | include duplex
Full-duplex, 1000Mb/s, media type is RJ45

Mismatched duplex causes packet loss.

- Check pfSense system resources:

Status > Dashboard

- CPU usage should be <50% under normal load

- Memory usage should be <80%

- States should be <50% of maximum

- Monitor queue drops:

Diagnostics > Traffic Shaper

Look for queue drops indicating congestion.

Resolution:

- Replace suspect cables

- Force interface speed/duplex if auto-negotiation fails

- Upgrade pfSense hardware if CPU/memory constrained

- Implement traffic shaping/QoS if needed

Problem: DNS Resolution Fails

Symptoms:

- Can ping 8.8.8.8 but not google.com

- nslookup/dig commands fail

- Browsers show "DNS probe failed"

Diagnostic Steps:

- Test DNS from VLAN device:

nslookup google.com 8.8.8.8

If this works, DNS servers are reachable but not configured on device.

- Check DHCP configuration (if used):

On Cisco router:

R1# show running-config | section dhcp
ip dhcp pool VLAN10
network 192.168.10.0 255.255.255.0
default-router 192.168.10.1
dns-server 8.8.8.8 1.1.1.1

Ensure DNS servers are configured in DHCP pool.

- Test DNS resolution from pfSense:

Diagnostics > DNS Lookup

Enter: google.com

Should resolve successfully.

- Verify pfSense DNS configuration:

System > General Setup

Ensure DNS servers are configured (8.8.8.8, 1.1.1.1, or internal DNS).

Resolution:

- Configure DNS servers in DHCP pools

- Manually configure DNS on client devices

- Use pfSense DNS Resolver/Forwarder for centralized DNS

Appendix: Command Reference

pfSense Console Commands

| Option | Description |
| --- | --- |
| 0 | Logout |
| 1 | Assign interfaces |
| 2 | Set interface IP address |
| 3 | Reset webConfigurator password |
| 4 | Reset to factory defaults |
| 5 | Reboot system |
| 6 | Halt system |
| 7 | Ping host |
| 8 | Shell |
| 9 | pfTop (process monitor) |
| 10 | Filter logs |
| 11 | Restart webConfigurator |
| 12 | PHP shell + pfSense tools |
| 13 | Update from console |
| 14 | Disable Secure Shell (SSH) |
| 15 | Restore recent configuration |
| 16 | Restart PHP-FPM |

Table 8: pfSense Console Menu Options

Common pfSense Shell Commands

From pfSense shell (option 8):

View routing table

netstat -rn

View ARP table

arp -a

View active states

pfctl -s states

View firewall rules

pfctl -s rules

View NAT rules

pfctl -s nat

Restart firewall

pfctl -d && pfctl -e

View interface statistics

ifconfig -a

Cisco Router Commands

Interface Configuration:

! Show interface status
show ip interface brief
show interfaces status
show interfaces GigabitEthernet0/1

! Configure interface
configure terminal
interface GigabitEthernet0/1
ip address 10.0.0.2 255.255.255.252
description P2P to pfSense
no shutdown
exit

Routing Commands:

! View routing table
show ip route

! Add static route
ip route [destination] [mask] [next-hop]
ip route 0.0.0.0 0.0.0.0 10.0.0.1

! Remove static route
no ip route 0.0.0.0 0.0.0.0 10.0.0.1

VLAN/Sub-interface Commands:

! Create sub-interface
interface GigabitEthernet0/0.10
encapsulation dot1Q 10
ip address 192.168.10.1 255.255.255.0
exit

! Verify sub-interfaces
show vlans
show running-config interface GigabitEthernet0/0.10

Diagnostic Commands:

! Ping with source interface
ping 10.0.0.1 source 10.0.0.2
ping 192.168.10.100 source 192.168.20.1

! Traceroute
traceroute 8.8.8.8

! Show ARP table
show ip arp

! Clear ARP cache
clear ip arp

! Show CDP neighbors (if enabled)
show cdp neighbors detail

Configuration Management:

! View running configuration
show running-config

! Save configuration
copy running-config startup-config
write memory

! View startup configuration
show startup-config

! Backup configuration (via TFTP)
copy running-config tftp://[server-ip]/config-backup.txt