# Lab: Multi-Site Enterprise Network with Dynamic Routing (OSPF)

## Scenario

"Northwind Logistics" is a growing company with a headquarters and two branch offices. They currently use static routes between sites, which breaks every time a link changes or a new subnet is added. Your job is to redesign their WAN using OSPF multi-area routing, configure route summarization at each site boundary, and connect the network to a simulated ISP with a default route advertised to the whole enterprise.

Tools: Cisco Packet Tracer, GNS3, or EVE-NG (any platform with Cisco IOS or IOS-like routers). Commands below are IOS syntax.

## 1. Learning Objectives

By the end of this lab you will be able to:

- Design a hierarchical, summarizable IP addressing scheme for a multi-site network

- Configure multi-area OSPF (backbone + 2 non-backbone areas)

- Summarize routes at Area Border Routers (ABRs) to reduce routing table size

- Inject a default route from an edge router into OSPF so every site can reach "the internet"

- Verify routing behavior and troubleshoot common OSPF adjacency issues

## 2. Topology

ISP-R1

(203.0.113.1)

Loopback0: 8.0.0.1/32

(simulates "the internet")

|

203.0.113.0/30

|

Gi0/0 .2

+-----------+

|   HQ-R1   |   Area 0 (Backbone)

| RID 1.1.1.1|

+-----------+

Gi0/1 |         | Gi0/2

10.10.100.0/30   10.10.100.4/30

|         |

Se0/0 |         | Se0/0

+-----------+   +-----------+

| BR1-R1    |   | BR2-R1    |

| RID 2.2.2.2 |  | RID 3.3.3.3 |

+-----------+   +-----------+

Area 1  |            |  Area 2

Gi0/1  |            |  Gi0/1

|            |

10.20.1.0/24   10.30.1.0/24

(Branch1 LAN)  (Branch2 LAN)

|            |

SW-BR1        SW-BR2

|            |

PC-BR1        PC-BR2

HQ LAN side (attached to HQ-R1, also Area 0):

Gi0/0 (HQ-R1) --- SW-HQ --- PC-HQ1 / Server-HQ

10.10.1.0/24  and  10.10.2.0/24

Sites:

| Site | Router | Area | Role |
| --- | --- | --- | --- |
| HQ | HQ-R1 | 0 (backbone) | Core router, connects to ISP, holds Area 0 WAN links |
| Branch 1 | BR1-R1 | 1 | ABR between Area 1 (its LAN) and Area 0 |
| Branch 2 | BR2-R1 | 2 | ABR between Area 2 (its LAN) and Area 0 |
| ISP (simulated) | ISP-R1 | N/A (not in OSPF) | Represents the internet edge |

Why this area design? Putting the HQ-to-branch WAN links in Area 0 and each branch's LAN in its own area makes each branch router a natural ABR, which is exactly where you want to summarize — at the boundary between an area and the backbone.

## 3. IP Addressing Plan

Each site gets its own /16 block so its subnets summarize into one clean route. This is the key design decision that makes summarization possible — don't skip this step in real designs.

| Site | Site Block | Subnet | Purpose |
| --- | --- | --- | --- |
| HQ | 10.10.0.0/16 | 10.10.1.0/24 | HQ user LAN |
| HQ | 10.10.0.0/16 | 10.10.2.0/24 | HQ server LAN |
| Branch 1 | 10.20.0.0/16 | 10.20.1.0/24 | Branch1 LAN |
| Branch 2 | 10.30.0.0/16 | 10.30.1.0/24 | Branch2 LAN |

WAN links (Area 0 backbone):

| Link | Subnet |
| --- | --- |
| HQ-R1 <-> BR1-R1 | 10.10.100.0/30 |
| HQ-R1 <-> BR2-R1 | 10.10.100.4/30 |

ISP edge (outside OSPF):

| Link | Subnet |
| --- | --- |
| HQ-R1 <-> ISP-R1 | 203.0.113.0/30 |
| ISP-R1 Loopback0 | 8.0.0.1/32 (represents "the internet") |

Router IDs (set via Loopback0 on each internal router):

- HQ-R1: 1.1.1.1

- BR1-R1: 2.2.2.2

- BR2-R1: 3.3.3.3

## 4. Base Configuration — HQ-R1

hostname HQ-R1

!

interface Loopback0

ip address 1.1.1.1 255.255.255.255

!

interface GigabitEthernet0/0

description Link to ISP-R1

ip address 203.0.113.2 255.255.255.252

no shutdown

!

interface GigabitEthernet0/1

description LAN - HQ Users

ip address 10.10.1.1 255.255.255.0

no shutdown

!

interface GigabitEthernet0/2

description WAN to BR1-R1

ip address 10.10.100.1 255.255.255.252

no shutdown

!

interface GigabitEthernet0/3

description WAN to BR2-R1

ip address 10.10.100.5 255.255.255.252

no shutdown

!

! Second HQ LAN (servers) - add a sub-interface or second physical NIC as needed

interface GigabitEthernet0/4

description LAN - HQ Servers

ip address 10.10.2.1 255.255.255.0

no shutdown

!

! --- Default route out to the simulated ISP ---

ip route 0.0.0.0 0.0.0.0 203.0.113.1

!

router ospf 1

router-id 1.1.1.1

network 10.10.1.0 0.0.0.255 area 0

network 10.10.2.0 0.0.0.255 area 0

network 10.10.100.0 0.0.0.3 area 0

network 10.10.100.4 0.0.0.3 area 0

default-information originate

default-information originate tells OSPF: "I have a static default route — advertise 0.0.0.0/0 to everyone else in the OSPF domain." Without this, branches would have no way to reach the ISP/internet even though HQ does.

## 5. Base Configuration — BR1-R1 (Branch 1, Area 1 ABR)

hostname BR1-R1

!

interface Loopback0

ip address 2.2.2.2 255.255.255.255

!

interface GigabitEthernet0/0

description WAN to HQ-R1

ip address 10.10.100.2 255.255.255.252

no shutdown

!

interface GigabitEthernet0/1

description Branch1 LAN

ip address 10.20.1.1 255.255.255.0

no shutdown

!

router ospf 1

router-id 2.2.2.2

network 10.10.100.0 0.0.0.3 area 0

network 10.20.1.0 0.0.0.255 area 1

!

! --- Route summarization: advertise the whole 10.20.0.0/16 block into Area 0 ---

! instead of every individual /24 behind this branch

area 1 range 10.20.0.0 255.255.0.0

## 6. Base Configuration — BR2-R1 (Branch 2, Area 2 ABR)

hostname BR2-R1

!

interface Loopback0

ip address 3.3.3.3 255.255.255.255

!

interface GigabitEthernet0/0

description WAN to HQ-R1

ip address 10.10.100.6 255.255.255.252

no shutdown

!

interface GigabitEthernet0/1

description Branch2 LAN

ip address 10.30.1.1 255.255.255.0

no shutdown

!

router ospf 1

router-id 3.3.3.3

network 10.10.100.4 0.0.0.3 area 0

network 10.30.1.0 0.0.0.255 area 2

!

! --- Route summarization for Branch 2 ---

area 2 range 10.30.0.0 255.255.0.0

## 7. Simulated ISP — ISP-R1

This router is deliberately kept outside OSPF — that's the point. It represents an external network HQ reaches only via a default/static route, just like a real ISP handoff.

hostname ISP-R1

!

interface Loopback0

description Represents "the internet"

ip address 8.0.0.1 255.255.255.255

!

interface GigabitEthernet0/0

description Link to HQ-R1

ip address 203.0.113.1 255.255.255.252

no shutdown

!

! Static route back to the enterprise supernet (optional, for return traffic)

ip route 10.0.0.0 255.0.0.0 203.0.113.2

## 8. Verification Checklist

Run these on each router and confirm the expected result:

| Command | What to check |
| --- | --- |
| show ip ospf neighbor | HQ-R1 sees BR1-R1 (2.2.2.2) and BR2-R1 (3.3.3.3) as FULL neighbors |
| show ip route ospf on BR1-R1 | Should show a single summarized route for 10.30.0.0/16 (Branch2), not individual /24s |
| show ip route ospf on BR2-R1 | Should show a single summarized route for 10.20.0.0/16 (Branch1) |
| show ip route on BR1-R1 / BR2-R1 | Should show O*E2 0.0.0.0/0 learned from HQ-R1 |
| show ip protocols | Confirms area assignments and router ID on each box |
| ping 8.0.0.1 from PC-BR1 or PC-BR2 | Confirms branch-to-"internet" reachability via the default route |
| traceroute 8.0.0.1 from a branch PC | Should hop: Branch LAN → BR-R1 → HQ-R1 → ISP-R1 |
| show ip ospf database | Confirms Type-3 summary LSAs (10.20.0.0/16, 10.30.0.0/16) instead of per-subnet Type-1/2 entries flooding across areas |

What "success" looks like: if you add a new subnet behind Branch 1 (say 10.20.2.0/24) and put it in Area 1, HQ and Branch 2 should not see a new route appear — they should still only see the single 10.20.0.0/16 summary. That's the entire point of summarization: local changes stay local.

## 9. Troubleshooting Tips

- Neighbors stuck in EXSTART/EXCHANGE: usually an MTU mismatch on the WAN link. Check mtu settings match on both ends.

- No neighbor forming at all: confirm both sides are in the same area on that link (WAN links must both say area 0), and that network statements' wildcard masks actually cover the interface IP.

- Summarization not appearing: area <id> range is configured on the ABR, not on the router that owns the LAN behind it (in this design that happens to be the same router, but in bigger designs it might not be).

- Default route not propagating: default-information originate only works if HQ-R1 actually has a route to 0.0.0.0/0 in its own routing table (the static route). If HQ-R1 has no default route itself, add default-information originate always to force-advertise it anyway (useful in labs where the ISP link might be down).

## 10. Challenge Extensions

Once the base lab works, try these to go further:

- Add a 3rd branch (Branch 3, Area 3, block 10.40.0.0/16) and summarize it the same way.

- Add a redundant WAN link between BR1-R1 and BR2-R1 directly, and observe OSPF choosing paths by cost — then tune bandwidth or ip ospf cost to influence path selection.

- Convert to EIGRP instead of OSPF: use router eigrp 100, network statements per interface, and eigrp stub on branch routers (a good compare/contrast exercise — EIGRP has no concept of areas, so summarization uses ip summary-address eigrp 100 ... directly on the interface instead of an ABR "range" command).

- Simulate a WAN outage: shut down the HQ-BR1 link and confirm Branch 1 loses its default route and Area 0 reconverges, while Branch 2 is unaffected.

- Add authentication (ip ospf authentication message-digest) to the WAN links — a good segue into a security-focused follow-up lab.

## 11. Why This Beats Static Routing (for your writeup/demo)

- Scalability: adding a 4th branch means configuring one new router, not editing static routes on every existing router.

- Convergence: a link failure is detected and rerouted automatically (seconds), instead of waiting for someone to notice and fix a static route.

- Smaller routing tables: summarization means HQ only ever sees 10.20.0.0/16 and 10.30.0.0/16, not every /24 behind each branch — this matters a lot once branches have 5-10 subnets each.

- Single point of internet exit: default-information originate lets you change ISPs or add a second ISP link at HQ without touching a single branch router's configuration.