# Packet Tracer Build Guide: Home Network Redesign (Home / IoT / Guest VLANs)

Devices used:

- 1x Router (e.g., Cisco 4331 or 2911 — anything with two Gig interfaces)

- 1x Layer 2 Switch (e.g., Cisco 2960)

- 1x Wireless Router/AP with multiple SSID support (e.g., WRT300N, or use 3 separate access points if your PT version doesn't support multi-SSID)

- 6x End devices (2 laptops/PCs for Home, 2 IoT-style devices — smart TV or a PC standing in for a smart plug/camera, 2 guest laptops/phones)

If your Packet Tracer version's home wireless router doesn't support multiple SSIDs on separate VLANs, substitute 3 access points, one per VLAN, each connected to its own trunk-carried VLAN on the switch. The logic is identical either way.

## Part 1: Physical Placement and Cabling

- Drag the Router onto the canvas — this represents your gateway/firewall.

- Drag a 2960 Switch and connect it to the router's GigabitEthernet0/0 using a straight-through cable to the router, into the switch's GigabitEthernet0/1.

- Drag your wireless router/AP and connect it to the switch — pick another switch port, e.g. FastEthernet0/2.

- Add your 6 end devices:

  - PC-1, PC-2 → connect via Ethernet to switch ports Fa0/3 and Fa0/4 (Home VLAN)

  - IoT-1, IoT-2 → connect to switch ports Fa0/5 and Fa0/6 (IoT VLAN) — or via Wi-Fi to the IoT SSID if you're modeling them wireless

  - Guest-1, Guest-2 → connect wirelessly to the Guest SSID on the AP

## Part 2: Create VLANs on the Switch

Click the switch → CLI tab.

Switch> enable

Switch# configure terminal

Switch(config)# vlan 1

Switch(config-vlan)# name HOME

Switch(config-vlan)# exit

Switch(config)# vlan 20

Switch(config-vlan)# name IOT

Switch(config-vlan)# exit

Switch(config)# vlan 30

Switch(config-vlan)# name GUEST

Switch(config-vlan)# exit

## Part 3: Assign Access Ports

! Home devices

Switch(config)# interface range fastEthernet 0/3 - 4

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 1

Switch(config-if-range)# exit

! IoT devices

Switch(config)# interface range fastEthernet 0/5 - 6

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 20

Switch(config-if-range)# exit

## Part 4: Trunk the Links to the Router and AP

Both the router-facing link and the AP-facing link need to carry all three VLANs, so both become trunks.

! Link to router

Switch(config)# interface gigabitEthernet 0/1

Switch(config-if)# switchport trunk encapsulation dot1q

Switch(config-if)# switchport mode trunk

Switch(config-if)# switchport trunk allowed vlan 1,20,30

Switch(config-if)# exit

! Link to wireless AP (carries Guest + IoT SSIDs mapped to VLANs)

Switch(config)# interface fastEthernet 0/2

Switch(config-if)# switchport trunk encapsulation dot1q

Switch(config-if)# switchport mode trunk

Switch(config-if)# switchport trunk allowed vlan 1,20,30

Switch(config-if)# exit

If your AP model in Packet Tracer doesn't support trunk mode (some simplified wireless devices don't), use three separate access points instead — one per VLAN, each connected to an access port on the switch configured for that VLAN, each broadcasting one SSID.

## Part 5: Router Subinterfaces (Router-on-a-Stick)

Click the router → CLI.

Router> enable

Router# configure terminal

Router(config)# interface gigabitEthernet 0/0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface gigabitEthernet 0/0.1

Router(config-subif)# encapsulation dot1Q 1 native

Router(config-subif)# ip address 192.168.1.1 255.255.255.0

Router(config-subif)# exit

Router(config)# interface gigabitEthernet 0/0.20

Router(config-subif)# encapsulation dot1Q 20

Router(config-subif)# ip address 192.168.20.1 255.255.255.0

Router(config-subif)# exit

Router(config)# interface gigabitEthernet 0/0.30

Router(config-subif)# encapsulation dot1Q 30

Router(config-subif)# ip address 192.168.30.1 255.255.255.0

Router(config-subif)# exit

Note: VLAN 1 is marked native on the trunk since it's the default VLAN — this avoids a common dot1q native VLAN mismatch error between the router and switch. Make sure the switch trunk ports also treat VLAN 1 as native (this is the default behavior, so usually no extra command is needed).

## Part 6: DHCP Pools (So Devices Get Addressed Automatically)

Still on the router:

Router(config)# ip dhcp excluded-address 192.168.1.1 192.168.1.9

Router(config)# ip dhcp excluded-address 192.168.20.1 192.168.20.9

Router(config)# ip dhcp excluded-address 192.168.30.1 192.168.30.9

Router(config)# ip dhcp pool HOME-POOL

Router(dhcp-config)# network 192.168.1.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.1.1

Router(dhcp-config)# dns-server 8.8.8.8

Router(dhcp-config)# exit

Router(config)# ip dhcp pool IOT-POOL

Router(dhcp-config)# network 192.168.20.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.20.1

Router(dhcp-config)# dns-server 8.8.8.8

Router(dhcp-config)# exit

Router(config)# ip dhcp pool GUEST-POOL

Router(dhcp-config)# network 192.168.30.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.30.1

Router(dhcp-config)# dns-server 8.8.8.8

Router(dhcp-config)# exit

On each end device, set IP configuration to DHCP (Desktop tab > IP Configuration) rather than static — this matches the "IP Addressing Scheme" table from the redesign lab, where only the low end of each range is reserved for static use.

## Part 7: Access Control — Enforce the Isolation

This is the step that actually makes the "IoT/Guest can't reach Home" improvement real, rather than just a diagram claim. Without this, all three VLANs can freely route to each other — routing alone doesn't isolate anything, it just makes cross-VLAN reachability possible, so ACLs are what turn "possible" into "controlled."

! Block Guest from reaching Home and IoT, allow it out to the internet

Router(config)# access-list 101 deny ip 192.168.30.0 0.0.0.255 192.168.1.0 0.0.0.255

Router(config)# access-list 101 deny ip 192.168.30.0 0.0.0.255 192.168.20.0 0.0.0.255

Router(config)# access-list 101 permit ip any any

! Block IoT from reaching Home and Guest, allow it out to the internet

Router(config)# access-list 102 deny ip 192.168.20.0 0.0.0.255 192.168.1.0 0.0.0.255

Router(config)# access-list 102 deny ip 192.168.20.0 0.0.0.255 192.168.30.0 0.0.0.255

Router(config)# access-list 102 permit ip any any

! Apply the ACLs inbound on the respective subinterfaces

Router(config)# interface gigabitEthernet 0/0.30

Router(config-subif)# ip access-group 101 in

Router(config-subif)# exit

Router(config)# interface gigabitEthernet 0/0.20

Router(config-subif)# ip access-group 102 in

Router(config-subif)# exit

Home VLAN (VLAN 1) gets no restrictive ACL — it's the trusted/admin network and can reach everything, including IoT devices (useful for managing smart home apps from a phone/laptop on the Home VLAN).

## Part 8: Save Everything

Router# copy running-config startup-config

Switch# copy running-config startup-config

## Part 9: Verification

| Check | Command | Where | Expected Result |
| --- | --- | --- | --- |
| VLANs exist and ports assigned correctly | show vlan brief | Switch | VLAN 1, 20, 30 active with correct ports |
| Trunk links up | show interfaces trunk | Switch | Gi0/1 and Fa0/2 trunking, VLANs 1,20,30 allowed |
| Subinterfaces up with correct IPs | show ip interface brief | Router | All three .1/.20/.30 subinterfaces up/up |
| DHCP working | Check IP Configuration tab on each PC | End devices | Each device gets an address in its VLAN's subnet |
| Home → IoT reachable | ping 192.168.20.x from a Home PC | PC-1/PC-2 | Success (Home is trusted, no ACL blocking it) |
| IoT → Home blocked | ping 192.168.1.x from an IoT device | IoT-1/IoT-2 | Fails — blocked by ACL 102 |
| Guest → Home blocked | ping 192.168.1.x from a Guest device | Guest-1/Guest-2 | Fails — blocked by ACL 101 |
| Guest → IoT blocked | ping 192.168.20.x from a Guest device | Guest-1/Guest-2 | Fails — blocked by ACL 101 |
| Guest/IoT → internet still works | Ping an external IP or simulated server | Guest/IoT devices | Success — ACLs only block internal cross-VLAN traffic |

If any "should fail" ping actually succeeds, double check:

- The ACL is applied in on the correct subinterface (not out, not on the wrong VLAN)

- The permit ip any any line is last in each ACL (order matters — first match wins)

- The device actually pulled an address from the right DHCP pool (wrong VLAN membership is the most common root cause)

## Part 10: Optional Extensions

- Add a DHCP relay/helper if you move to SVIs on a Layer 3 switch instead of router subinterfaces.

- Add port security on the access ports (limit to 1–2 MAC addresses) to prevent someone plugging in an unauthorized device.

- Simulate the "before" state first (no ACLs) and run the same ping tests, so you have a clear before/after contrast to include in your writeup from Section 5 of the redesign lab.