Baseline Switch Configuration

1. Physical and console access

- Verify hardware:

  - Check model, fan and status LEDs, and that all modules/SFPs are seated properly.

- Power on the switch and wait for it to fully boot.

- Connect to the console:

  - Use a console/USB-to-serial cable to your laptop and a terminal emulator (PuTTY, Tera Term, SecureCRT).

  - Typical serial settings: 9600 bps, 8 data bits, no parity, 1 stop bit, no flow control.

Example (user PC side, conceptually): connect to COM port at 9600, open terminal, you should see the switch prompt after pressing Enter.

2. Erase any old configuration

Run these from the initial CLI prompt:

- Enter privileged EXEC mode:

  - Switch> enable

- Erase startup configuration:

  - Switch# write erase or Switch# erase startup-config (platform dependent).

- Erase VLAN database (for reused switches):

  - Switch# delete flash:vlan.dat then confirm.

- Reload the switch:

  - Switch# reload

  - Answer “no” if asked to save configuration.

After reload, you should get the default prompt like Switch> and possibly the “Initial configuration dialog”; answer no to use the CLI manually.

3. Basic identity and management settings

From the default prompt:

- Go to global configuration:

  - Switch> enable

  - Switch# configure terminal

- Set hostname:

  - Switch(config)# hostname SW-Access-01

- Set a domain name (needed for SSH later):

  - SW-Access-01(config)# ip domain-name lab.local

- Secure privileged (enable) mode:

  - Legacy way (not recommended in prod, ok in lab):

    - SW-Access-01(config)# enable secret MyEnableP@ss1

- Configure a local admin user:

  - SW-Access-01(config)# username admin privilege 15 secret Adm1nP@ss!

4. Management interface IP and default gateway

For a typical L2 access switch using VLAN 1 (or a dedicated management VLAN):

- Create or use the management SVI:

  - SW-Access-01(config)# interface vlan 1

  - SW-Access-01(config-if)# description Management_VLAN

  - SW-Access-01(config-if)# ip address 192.168.10.10 255.255.255.0

  - SW-Access-01(config-if)# no shutdown

- Configure default gateway:

  - SW-Access-01(config)# ip default-gateway 192.168.10.1

- (Optional) For out-of-band mgmt port (e.g., FastEthernet0 or GigabitEthernet0 on some models) configure IP similarly under that interface instead.

You should now be able to ping this IP from a device in the same subnet once a port is cabled and up.

5. Secure console, vty, and enable SSH

- Console line:

  - SW-Access-01(config)# line console 0

  - SW-Access-01(config-line)# logging synchronous

  - SW-Access-01(config-line)# exec-timeout 10 0 (10 minutes)

  - SW-Access-01(config-line)# login local

  - SW-Access-01(config-line)# exit

- VTY (Telnet/SSH) lines:

  - SW-Access-01(config)# line vty 0 4

  - SW-Access-01(config-line)# transport input ssh

  - SW-Access-01(config-line)# login local

  - SW-Access-01(config-line)# exec-timeout 10 0

  - SW-Access-01(config-line)# exit

- Generate RSA keys for SSH:

  - SW-Access-01(config)# crypto key generate rsa modulus 2048

  - SW-Access-01(config)# ip ssh version 2

From your PC, you should then be able to SSH to the management IP and log in with the admin account.

6. Common “baseline” switch settings

These are typical defaults you probably want on every lab switch:

- Disable unnecessary services:

  - SW-Access-01(config)# no ip http server

  - SW-Access-01(config)# no ip http secure-server

- Enable logging and basic security:

  - SW-Access-01(config)# service password-encryption

  - SW-Access-01(config)# login block-for 60 attempts 5 within 60 (if supported)

- Configure banner (legal warning):

  - SW-Access-01(config)# banner motd #Unauthorized access prohibited#

- Time settings (for logging):

  - SW-Access-01(config)# clock timezone CST -6

  - SW-Access-01(config)# clock summer-time CDT recurring

  - Optionally NTP:

    - SW-Access-01(config)# ntp server 192.0.2.10

7. Verify and save

Run these checks:

- SW-Access-01# show running-config | include hostname|ip domain-name

- SW-Access-01# show ip interface brief (verify VLAN 1 up/up with correct IP).

- SW-Access-01# show interfaces status (ports up/down as expected).

- SW-Access-01# show ssh (ensure SSH is enabled).

- SW-Access-01# show version (model, IOS, uptime).

When satisfied:

- SW-Access-01# write memory or SW-Access-01# copy running-config startup-config