Cisco Packet Tracer Lab

Building a 6-Switch / 6-PC Full-Mesh Network

Student Worksheet

Name: ______________________________     Date: ________________

# Lab Goal

In this lab you will build a full-mesh topology of 6 switches and 6 PCs in Cisco Packet Tracer, assign IP addresses, verify connectivity with ping, and demonstrate the redundancy a mesh provides. By the end you should be able to:

- Build a full-mesh topology with 6 2960-24TT switches and 6 PCs.

- Configure static IPv4 addresses on all PCs in a single /24 subnet.

- Verify connectivity using ping and a Simple PDU in Simulation mode.

- Explain why the mesh stays loop-free and survives a link failure.

# Step 1 — Create the Workspace and Place Devices

- Open Cisco Packet Tracer and create a new blank workspace.

- From the End Devices panel, drag 6 PC-PT devices onto the canvas. Label them PC1–PC6.

- From Network Devices → Switches, drag 6 of the 2960-24TT switches. Place each near a PC and label them SW1–SW6.

- (Optional) Rename each device using Config → Display Name so the diagram is readable.

# Step 2 — Connect PCs to Switches

- Select the Connections tool, then choose the Copper Straight-Through cable.

- Connect each PC to its own switch: PC1 → SW1, PC2 → SW2, … PC6 → SW6 (PC FastEthernet0 to any free Fa port on the switch, e.g. Fa0/1).

Note: The Auto-Connect (lightning-bolt) tool can pick ports for you, but choose cables manually here for skills practice.

# Step 3 — Build the Full Mesh Between Switches

Goal: every switch must have a direct link to every other switch. Use the systematic pattern below so you don't create duplicate links.

- SW1 connects to SW2, SW3, SW4, SW5, SW6

- SW2 connects to SW3, SW4, SW5, SW6

- SW3 connects to SW4, SW5, SW6

- SW4 connects to SW5, SW6

- SW5 connects to SW6

This yields 15 unique switch-to-switch links — a full mesh for 6 nodes.

Use a single subnet: 192.168.10.0 / 24, mask 255.255.255.0. The default gateway can be left blank for this lab.

Record each link you create in the table below.

Inter-Switch Link Table

| Link # | From Switch / Port | To Switch / Port | Link OK? (✓) |
| --- | --- | --- | --- |
| 1 | SW1 Fa0/____ | SW2 Fa0/____ |  |
| 2 | SW1 Fa0/____ | SW3 Fa0/____ |  |
| 3 | SW1 Fa0/____ | SW4 Fa0/____ |  |
| 4 | SW1 Fa0/____ | SW5 Fa0/____ |  |
| 5 | SW1 Fa0/____ | SW6 Fa0/____ |  |
| 6 | SW2 Fa0/____ | SW3 Fa0/____ |  |
| 7 | SW2 Fa0/____ | SW4 Fa0/____ |  |
| 8 | SW2 Fa0/____ | SW5 Fa0/____ |  |
| 9 | SW2 Fa0/____ | SW6 Fa0/____ |  |
| 10 | SW3 Fa0/____ | SW4 Fa0/____ |  |
| 11 | SW3 Fa0/____ | SW5 Fa0/____ |  |
| 12 | SW3 Fa0/____ | SW6 Fa0/____ |  |
| 13 | SW4 Fa0/____ | SW5 Fa0/____ |  |
| 14 | SW4 Fa0/____ | SW6 Fa0/____ |  |
| 15 | SW5 Fa0/____ | SW6 Fa0/____ |  |

Step 4 — Configure IP Addresses on PCs

Use a single subnet: 192.168.10.0 / 24, mask 255.255.255.0. The default gateway can be left blank for this lab.

For each PC: click the PC → Desktop tab → IP Configuration → enter the values below.

| Device | IP Address | Subnet Mask | Configured? (✓) |
| --- | --- | --- | --- |
| PC1 | 192.168.10.11 | 255.255.255.0 |  |
| PC2 | 192.168.10.12 | 255.255.255.0 |  |
| PC3 | 192.168.10.13 | 255.255.255.0 |  |
| PC4 | 192.168.10.14 | 255.255.255.0 |  |
| PC5 | 192.168.10.15 | 255.255.255.0 |  |
| PC6 | 192.168.10.16 | 255.255.255.0 |  |

# Step 5 — Test Connectivity (Real-Time Mode)

- Wait until all link lights turn green (amber first while STP converges).

- On PC1: Desktop → Command Prompt → ping each of the other PCs (192.168.10.12 through .16).

- Repeat from every PC and fill in the ping matrix. Mark ✓ for a reply, ✗ for a timeout.

Tip: The first ping to a host may time out (ARP + STP convergence); run it again before recording a result.

Ping Matrix – Success/Failed

| From \ To | PC1 | PC2 | PC3 | PC4 | PC5 | PC6 |
| --- | --- | --- | --- | --- | --- | --- |
| PC1 | — |  |  |  |  |  |
| PC2 |  | — |  |  |  |  |
| PC3 |  |  | — |  |  |  |
| PC4 |  |  |  | — |  |  |
| PC5 |  |  |  |  | — |  |
| PC6 |  |  |  |  |  | — |

# Step 6 — Failure and Redundancy Test

- In Real-Time mode, open one switch (e.g. SW2) and disable one inter-switch link — use Config → Interface → turn the port Off, or the CLI command shutdown.

- Re-run pings between the PCs. Connectivity should still succeed because alternate paths exist.

- Return to Simulation mode and send another Simple PDU; note the new path taken.

Which link did you disable? ________________________________________

Did all PCs still reach each other? Why?  ______________________________

______________________________

______________________________