Lab: VLANs and Access Ports

Objective:
Configure VLANs and access ports on a Cisco switch to see how VLANs segment broadcast domains and why routing is required for communication between VLANs.

Topology (simplified)

- 1 Cisco switch (e.g., Catalyst 2960).

- PC1 → connected to a switch port assigned to VLAN 10 (e.g., 192.168.10.0/24).

- PC2 → connected to a switch port assigned to VLAN 20 (e.g., 192.168.20.0/24).

Step 1: Configure the PCs and connect to the switch

- Assign IP addresses in subnets to PC1 and PC2:

  - PC1: IP = 192.168.10.10, Subnet mask = 255.255.255.0

  - PC2: IP = 192.168.20.20, Subnet mask = 255.255.255.0

- Connect:

  - PC1 → Switch port Gi0/1 (example).

  - PC2 → Switch port Gi0/2 (example).

Step 2: Create VLAN 10 and VLAN 20 on the switch

- Access the switch CLI (console or SSH) and enter privileged exec mode:

Switch> enable

Switch#

- Enter global configuration mode:

Switch# configure terminal

Switch(config)#

- Create VLAN 10:

Switch(config)# vlan 10

Switch(config-vlan)# name HR_VLAN

Switch(config-vlan)# exit

- Create VLAN 20:

Switch(config)# vlan 20

Switch(config-vlan)# name IT_VLAN

Switch(config-vlan)# exit

Step 3: Assign ports as access ports in VLAN 10 and VLAN 20

- Configure Gi0/1 as an access port in VLAN 10:

Switch(config)# interface gi0/1

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

Switch(config-if)# no shutdown

Switch(config-if)# end

- Configure Gi0/2 as an access port in VLAN 20:

text

Switch(config)# interface gi0/2

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 20

Switch(config-if)# no shutdown

Switch(config-if)# end

Note:

  - switchport mode access sets the port for end-device connectivity (not a trunk).

- switchport access vlan X assigns the port to that specific VLAN.

Step 4: Verify VLAN and switchport configuration

- On the switch, issue:

Switch# show vlan brief

  - Confirm that:

    - VLAN 10 is present and Gi0/1 appears listed under it.

    - VLAN 20 is present and Gi0/2 appears listed under it.

- Check the switchport mode for each PC’s port:

Switch# show interfaces gi0/1 switchport

Switch# show interfaces gi0/2 switchport

  - Verify:

    - Administrative mode = access.

    - Access VLAN = 10 (for Gi0/1) and 20 (for Gi0/2).

Step 5: Test connectivity between PCs

- From PC1 (192.168.10.10), try to ping PC2 (192.168.20.20):

- From PC2, try to ping PC1:

Questions

- In this lab, PC1 and PC2 are connected to the same physical switch but cannot ping each other. Explain why they cannot communicate.

- What is the purpose of the command switchport mode access on an interface in this lab? Describe what it does and why it is needed.

- What does the command switchport access vlan 10 do on interface Gi0/1? Include what happens to traffic coming from PC1 in your explanation.

- a) What information should you see for VLAN 10 and VLAN 20 if the lab was configured correctly?

b) Why is this command useful during troubleshooting?

- a) Which two key fields should you check to verify that Gi0/1 is properly configured for PC1 in VLAN 10?

b) What values do you expect for those fields?

- Suppose show vlan brief does not list interface Gi0/2 under VLAN 20. List two possible configuration mistakes that could cause this.

- A student misconfigures Gi0/1 with switchport access vlan 20 instead of VLAN 10.

a) What IP network is PC1 in?

b) What VLAN is the port in?

c) What kind of connectivity problems might this cause and why?

Task X: Connect the router to port Gi0/3 for future inter-VLAN routing and management.

- Physically connect the router’s G0/0 interface to switch port Gi0/3.

- On the switch, configure Gi0/3 either:

  - As a trunk port to carry VLANs 10 and 20 (future router-on-a-stick), or

  - As an access port in a new management VLAN 99.

- Use show interfaces switchport and show vlan brief or show interfaces trunk to verify your configuration.

- Answer: “Why would you want a separate management VLAN instead of managing the switch from VLAN 10 or VLAN 20?”

Extended lab: You can extend Lab 3 with a short add-on task: connecting the router on Gi0/3 and preparing that port either as a future trunk or as an access port in a management VLAN.

1. Add router connection to Gi0/3

Topology addition:

- Connect router GigabitEthernet0/0 to switch GigabitEthernet0/3 (or FastEthernet0/3 if that’s what your switch has).

- This link will initially be used either:

  - As a trunk (future inter-VLAN routing lab), or

  - As an access port in a dedicated management VLAN (e.g., VLAN 99).

Tell students to document the cable type (straight-through copper) and interfaces used.

2. Option A – Prepare Gi0/3 as a trunk (for future labs)

This option sets up the switch side now; you can do router-on-a-stick later.

On the switch:

bash

S1(config)# interface GigabitEthernet0/3

S1(config-if)# switchport mode trunk

! Optional but recommended: restrict allowed VLANs

S1(config-if)# switchport trunk allowed vlan 10,20

! (Later you can add 99 or others as needed)

S1(config-if)# exit

Key teaching points:

- switchport mode trunk makes Gi0/3 carry tagged frames for multiple VLANs.

- switchport trunk allowed vlan 10,20 limits which VLANs can cross the link, which is good practice in real networks.

- You are not yet configuring subinterfaces on the router; that will be part of the inter-VLAN routing lab.

Verification on the switch:

bash

S1# show interfaces trunk

S1# show interfaces GigabitEthernet0/3 switchport

Students should see:

- Administrative/operational mode: trunk.

- Allowed VLANs: 10,20 (plus native VLAN 1 unless changed).

You can simply state that “this trunk will later let the router see traffic from multiple VLANs on one physical link.”

3. Option B – Gi0/3 as access port in a management VLAN

If you also want to introduce a management VLAN early, keep Gi0/3 as an access port in a dedicated management VLAN (e.g., VLAN 99).

Step 1 – Create the management VLAN

On the switch:

bash

S1(config)# vlan 99

S1(config-vlan)# name MANAGEMENT

S1(config-vlan)# exit

Step 2 – Make Gi0/3 an access port in VLAN 99

bash

S1(config)# interface GigabitEthernet0/3

S1(config-if)# switchport mode access

S1(config-if)# switchport access vlan 99

S1(config-if)# exit

Key points:

- An access port carries only one VLAN, untagged.

- Here, VLAN 99 is reserved for managing the switch and router, not for user PCs.

Verification:

bash

S1# show vlan brief

S1# show interfaces GigabitEthernet0/3 switchport

Students should confirm:

- VLAN 99 exists and includes Gi0/3.

- Gi0/3 switchport mode is static access, access VLAN 99.

Step 3 – (Optional) Give switch an IP in VLAN 99

You can optionally show how to give the switch a management IP:

bash

S1(config)# interface vlan 99

S1(config-if)# ip address 192.168.99.2 255.255.255.0

S1(config-if)# no shutdown

S1(config-if)# exit

On the router interface connected to Gi0/3 (e.g., G0/0):

bash

R1(config)# interface GigabitEthernet0/0

R1(config-if)# ip address 192.168.99.1 255.255.255.0

R1(config-if)# no shutdown

R1(config-if)# exit

Now the router and switch share the management subnet 192.168.99.0/24 on VLAN 99.

Students can then:

- Ping 192.168.99.2 from the router.

- Later, use telnet 192.168.99.2 or SSH for real management (future lab).