Lab: Router on a Stick (Inter-VLAN Routing)

This hands-on lab will teach you how to configure a single router interface to route traffic between multiple VLANs using the Router-on-a-Stick method with 802.1Q VLAN tagging.

Topology Overview

- Router: 1 Cisco Router (1921 or similar) with G0/0 interface

- Switch: 1 Cisco Catalyst Switch (2960 or similar)

- PCs: PC1 in VLAN 10, PC2 in VLAN 20

- Connection: Router G0/0 connected to switch trunk port

Equipment Required

- 1 Cisco Router (1921, 2900 series, or compatible)

- 1 Cisco Catalyst Switch (2960, 2960X, or compatible)

- 2 PCs or laptop computers

- Console cables for device configuration

- Ethernet cables as per topology

Part 1: Initial Setup and Cable the Network

Step 1: Physical Connections

- Connect router interface G0/0 to switch port Fa0/5 (or Fa0/1) using an Ethernet cable

- Connect PC1 to switch port Fa0/6 using an Ethernet cable

- Connect PC2 to switch port Fa0/18 using an Ethernet cable

- Connect console cables to router and switch for configuration

Step 2: Power on Devices

- Power on the router and wait for boot sequence to complete

- Power on the switch and wait for boot sequence to complete

- Power on both PCs

Part 2: Configure the Switch

Step 1: Basic Switch Configuration

Access the switch via console connection and enter the following commands:

Switch> enable

Switch# configure terminal

Step 2: Create VLANs on the Switch

Create VLAN 10 and VLAN 20 with descriptive names:

S1(config)# vlan 10

S1(config-vlan)# name Sales

S1(config-vlan)# exit

S1(config)# vlan 20

S1(config-vlan)# name Engineering

S1(config-vlan)# exit

Step 3: Assign Access Ports to VLANs

Configure the ports connecting to PCs as access ports:

For PC1 (VLAN 10):

S1(config)# interface fa0/6

S1(config-if)# switchport mode access

S1(config-if)# switchport access vlan 10

S1(config-if)# no shutdown

S1(config-if)# exit

For PC2 (VLAN 20):

S1(config)# interface fa0/18

S1(config-if)# switchport mode access

S1(config-if)# switchport access vlan 20

S1(config-if)# no shutdown

S1(config-if)# exit

Step 4: Configure Trunk Port to Router

Configure the interface connecting to the router as a trunk port:

S1(config)# interface fa0/5

S1(config-if)# switchport mode trunk

S1(config-if)# switchport trunk allowed vlan 10,20

S1(config-if)# no shutdown

S1(config-if)# exit

S1(config)# end

S1# write memory

Note: On some switches, you may need to configure trunk encapsulation first:

S1(config-if)# switchport trunk encapsulation dot1q

Part 3: Configure the Router

Step 1: Basic Router Configuration

Access the router via console and configure basic settings:

Router> enable

Router# configure terminal

Router(config) hostname R1

Step 2: Create Subinterfaces for Each VLAN

The router-on-a-stick method requires creating a subinterface for each VLAN. Configure subinterface G0/0.10 for VLAN 10:

R1(config)# interface g0/0.10

R1(config-subif)# description Gateway for VLAN 10

R1(config-subif)# encapsulation dot1Q 10

R1(config-subif)# ip address 192.168.10.1 255.255.255.0

R1(config-subif)# exit

Configure subinterface G0/0.20 for VLAN 20:

R1(config)# interface g0/0.20

R1(config-subif)# description Gateway for VLAN 20

R1(config-subif)# encapsulation dot1Q 20

R1(config-subif)# ip address 192.168.20.1 255.255.255.0

R1(config-subif)# exit

Step 3: Enable the Physical Interface

Enable the main physical interface to activate all subinterfaces:

R1(config)# interface g0/0

R1(config-if)# description Trunk link to S1

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# end

R1# write memory

Important: All subinterfaces depend on the physical interface being up.

Part 4: Configure PC Hosts

PC1 Configuration (VLAN 10)

Configure PC1 with the following network settings:

- IP Address: 192.168.10.10

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.10.1

PC2 Configuration (VLAN 20)

Configure PC2 with the following network settings:

- IP Address: 192.168.20.10

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.20.1

Windows Configuration Steps:

- Open Control Panel → Network and Internet → Network Connections

- Right-click Ethernet adapter → Properties

- Select Internet Protocol Version 4 (TCP/IPv4) → Properties

- Select "Use the following IP address" and enter values above

- Click OK to save

Part 5: Verification and Testing

Step 1: Verify Switch Configuration

On the switch, verify VLAN configuration:

text

S1# show vlan brief

Expected output should show VLAN 10 and VLAN 20 with assigned ports.

Verify trunk configuration:

text

S1# show interfaces trunk

This should display Fa0/5 as trunking with VLANs 10 and 20 allowed.

Step 2: Verify Router Configuration

On the router, verify subinterfaces are operational:

R1# show ip interface brief

Expected output should show:

- G0/0 status: up/up

- G0/0.10 with IP 192.168.10.1, status: up/up

- G0/0.20 with IP 192.168.20.1, status: up/up

Verify routing table:

text

R1# show ip route

You should see directly connected routes (C) for 192.168.10.0/24 and 192.168.20.0/24.

Display detailed subinterface information:

R1# show interfaces g0/0.10

R1# show interfaces g0/0.20

Verify that "Encapsulation 802.1Q Virtual LAN" appears with correct VLAN ID.

Verify the running configuration for G0/0:

R1# show run interface g0/0

Step 3: Connectivity Testing from PCs

From PC1:

- Verify IP configuration:

C:\> ipconfig

- Ping the default gateway (router subinterface):

C:\> ping 192.168.10.1

Expected: Successful replies

- Ping PC2 in the different VLAN:

C:\> ping 192.168.20.10

Expected: Successful replies (proves inter-VLAN routing works)

From PC2:

- Verify IP configuration:

C:\> ipconfig

- Ping the default gateway:

C:\> ping 192.168.20.1

- Ping PC1:

C:\> ping 192.168.10.10

C:\> tracert 192.168.10.10

1. First quick checks on PCs (Troubleshooting steps)

On both PCs, from the command prompt:

- ipconfig

  - PC1 must show: IP 192.168.10.10, mask 255.255.255.0, gateway 192.168.10.1.

  - PC2 must show: IP 192.168.20.10, mask 255.255.255.0, gateway 192.168.20.1.

- Basic pings

  - From PC1:

    - ping 192.168.10.1 → must work.

    - ping 192.168.20.1 → should also work if inter-VLAN routing is correct.

  - From PC2:

    - ping 192.168.20.1 → must work.

    - ping 192.168.10.1 → should also work.

Tell me exactly which of these succeed/fail; that narrows it fast.

2. Verify VLANs and ports on switch

On S1:

- show vlan brief
Confirm:

  - VLAN 10 exists, Fa0/6 is in VLAN 10.

  - VLAN 20 exists, Fa0/18 is in VLAN 20.

- show interfaces fa0/6 switchport and show interfaces fa0/18 switchport

  - Both should be Administrative Mode: static access and Access Mode VLAN: 10 / 20 respectively.

If PC2’s port is in the wrong VLAN or still in VLAN 1, PC1 will never reach it.

3. Verify the trunk to the router

On S1:

- show interfaces trunk
Check for Fa0/5:

  - “Mode: on” or “desirable/auto” with trunking ON.

  - “Allowed VLANs: 10,20” (or “1-4094” including 10 and 20).

- show interfaces fa0/5 switchport

  - Administrative Mode: trunk

  - Operational Mode: trunk

  - Trunking VLANs Enabled: must include 10 and 20.

If VLAN 20 is not in the allowed list on the trunk, PC1 can ping its own gateway but packets to PC2 will be dropped on the trunk.

4. Verify router subinterfaces

On R1:

- show ip interface brief
You should see:

  - G0/0: up/up

  - G0/0.10: 192.168.10.1, up/up

  - G0/0.20: 192.168.20.1, up/up

- show run interface g0/0.10 and show run interface g0/0.20
Must show:

  - encapsulation dot1Q 10 on G0/0.10

  - encapsulation dot1Q 20 on G0/0.20

  - Correct IPs as above.

If you accidentally put the wrong VLAN ID under encapsulation (e.g., dot1Q 1 instead of 20, or missing command), the router will not route VLAN 20 traffic properly.

Also confirm the physical interface:

- show run interface g0/0

  - Should have no shutdown.

  - No IP address configured directly on G0/0 (IPs only on subinterfaces).

5. Common concrete root causes in this exact scenario

Given you can ping the gateway from PC1 but not PC2, typical issues are:

- PC2:

  - Wrong IP, mask, or gateway (e.g., 192.168.10.x instead of 192.168.20.x, or gateway 192.168.10.1).

- Switch:

  - Fa0/18 not in VLAN 20 (still VLAN 1, or typo in VLAN).

  - Trunk Fa0/5 not actually trunking, or VLAN 20 not allowed on the trunk.

- Router:

  - Missing or wrong encapsulation dot1Q 20 on G0/0.20.

  - G0/0.20 administratively down.

Any one of these lets PC1 reach its own gateway (192.168.10.1) but prevents reaching PC2.

6. Simple step-by-step test sequence

Run these in order and tell me where it breaks:

- From R1:

  - ping 192.168.10.10 (PC1)

  - ping 192.168.20.10 (PC2)
If R1 cannot ping 192.168.20.10, the problem is between R1 and PC2 (VLAN 20 side).

- From PC2:

  - ping 192.168.20.1
If this fails, focus on VLAN 20 port config, trunk, subinterface G0/0.20, or PC2 IP config.

- If both PCs can ping both gateways but still cannot ping each other, suspect a local firewall on the PCs blocking ICMP. Windows Defender Firewall often does this.