Lab : DHCP from the Router for a Single VLAN

1. Lab overview

In this lab you will configure a Cisco router to act as a DHCP server for a single LAN/VLAN and verify that PCs receive IP addresses automatically.

- Topology: PC(s) — Switch — Router (single VLAN on the switch).

- IP plan: 192.168.10.0/24, default gateway 192.168.10.1.

- DHCP server: The router’s G0/0 interface.

2. Pre-lab: Topology and basic wiring

You must complete this physical/logical setup before configuring DHCP.

- Cabling

  - Connect PC1’s NIC to any access port on the switch with a straight-through Ethernet cable.

  - Connect the switch uplink (e.g., F0/1) to the router interface G0/0.

- Switch basic configuration (console into the switch)

  - Enter privileged EXEC mode:

    - Switch> enable

  - Enter global configuration mode:

    - Switch# configure terminal

  - (Optional, if you want to be explicit) Ensure access ports are in VLAN 1:

    - Switch(config)# interface range fa0/1 - 24

    - Switch(config-if-range)# switchport mode access

    - Switch(config-if-range)# switchport access vlan 1

    - Switch(config-if-range)# exit

  - Save configuration:

    - Switch# write memory or Switch# copy running-config startup-config.

- PC initial settings

  - For now, leave PCs with whatever default IP settings they have; you will change them to DHCP later in the lab.

3. Configure the router interface for the LAN

You will configure the router’s G0/0 interface with the default gateway IP address for the LAN.

- Access the router

  - Connect to the router console.

  - Enter privileged EXEC mode:

    - Router> enable

  - Enter global configuration mode:

    - Router# configure terminal

- Configure interface G0/0

  - Enter interface configuration mode:

    - Router(config)# interface gigabitEthernet0/0

  - Assign the IP address and subnet mask:

    - Router(config-if)# ip address 192.168.10.1 255.255.255.0

  - Enable the interface:

    - Router(config-if)# no shutdown

  - Exit interface mode:

    - Router(config-if)# exit

- Verify interface status

  - From privileged EXEC mode:

    - Router# show ip interface brief

  - Confirm that G0/0 shows:

    - IP address: 192.168.10.1

    - Status/Protocol: up/up.

4. Configure the DHCP server on the router

You will now configure the router’s IOS DHCP server for the 192.168.10.0/24 network.

4.1 Plan excluded addresses

Decide which addresses will be reserved (e.g., for servers, printers, static devices). Example: reserve 192.168.10.1–192.168.10.20.

- From global config mode:

  - Router(config)# ip dhcp excluded-address 192.168.10.1 192.168.10.20

Explain to students: Excluded addresses will never be given out by DHCP; they can be used for statics like the router interface itself.

4.2 Create the DHCP pool

- Create a DHCP pool named LAN-POOL:

  - Router(config)# ip dhcp pool LAN-POOL

- Specify the network for this pool:

  - Router(dhcp-config)# network 192.168.10.0 255.255.255.0

- Configure the default gateway (default-router) that clients will use:

  - Router(dhcp-config)# default-router 192.168.10.1

- Configure a DNS server (choose a public DNS such as 8.8.8.8 or 1.1.1.1):

  - Example:

    - Router(dhcp-config)# dns-server 8.8.8.8

- (Optional) Configure a lease time if desired; otherwise default lease is used:

  - Example (1 day 2 hours):

    - Router(dhcp-config)# lease 1 2

- Exit pool configuration mode:

  - Router(dhcp-config)# exit

This sequence (exclude, ip dhcp pool, network, default-router, dns-server) is the typical IOS DHCP server workflow.

5. Configure PCs to obtain addresses via DHCP

Now you will set the PCs to request an IP address automatically from the router’s DHCP server.

- On each PC (Packet Tracer or physical):

  - Open the PC configuration window.

  - Go to the IP Configuration or Network Settings area.

  - Set “IP Address Assignment” to DHCP or “Obtain an IP address automatically”.

- Verify IP information on each PC

  - The PC should obtain:

    - IP address in the range 192.168.10.21–192.168.10.254 (based on your exclusions).

    - Subnet mask: 255.255.255.0.

    - Default gateway: 192.168.10.1.

    - DNS server: 8.8.8.8 (or whatever you configured).

- If using Windows PCs

  - Open Command Prompt.

  - Use ipconfig /release then ipconfig /renew to force DHCP if needed.

  - Use ipconfig to confirm the assigned address and gateway.

This demonstrates how DHCP clients automatically receive full TCP/IP configuration without manual addressing.

6. Verify DHCP operation on the router

You will use IOS show commands to confirm that the router is assigning addresses.

- Show DHCP bindings

  - On the router (privileged EXEC):

    - Router# show ip dhcp binding

  - Confirm that:

    - Each PC’s MAC address appears.

    - Each PC has a leased IP address from the pool (e.g., 192.168.10.21, .22, etc.).

- Show DHCP pool statistics

  - Router# show ip dhcp pool

- Show running configuration for DHCP

  - Router# show running-config | section dhcp

7. Test connectivity from PCs

- Ping the default gateway

  - On each PC, open a command prompt/terminal.

  - Run:

    - ping 192.168.10.1

  - Successful replies confirm:

    - Correct IP addressing.

    - Layer 2 connectivity PC ↔ Switch ↔ Router.

    - Router interface G0/0 is up.

- Ping between PCs (optional)

  - From PC1 (e.g., 192.168.10.21) ping PC2 (e.g., 192.168.10.22):

    - ping 192.168.10.22

  - Successful ping shows that hosts within the same VLAN can communicate.

Router Interface Configuration Questions

What command would you use to verify that interface G0/0 is administratively enabled and operationally functioning?

Why must the router's G0/0 interface be assigned 192.168.10.1 before configuring DHCP? What role does this IP address serve?

If the output of show ip interface brief shows G0/0 as "up/down", what layer of the OSI model is the problem occurring at, and what are two possible causes?

DHCP Configuration Questions

What is the purpose of the ip dhcp excluded-address command? Why is it configured before creating the DHCP pool?

Write the complete sequence of commands (starting from global configuration mode) to create a DHCP pool named "SALES-POOL" for network 10.50.0.0/24 with excluded addresses 10.50.0.1-10.50.0.50, default gateway 10.50.0.1, and DNS server 1.1.1.1.

If you configure a lease time of lease 2 12 30, what does each number represent? How long will clients keep their IP addresses?

What configuration parameter tells DHCP clients which IP address to use as their gateway to reach other networks?

Can the excluded address range (192.168.10.1-192.168.10.20) include the router's G0/0 IP address (192.168.10.1)? Why or why not?

PC Configuration Questions

On a Windows PC, what two commands would you use to release the current DHCP lease and request a new IP address from the DHCP server?

List the four pieces of IP configuration information that a PC should receive from the DHCP server in this lab.

Based on the lab configuration, what is the first IP address that could be assigned to a PC? What is the last possible IP address?

Verification and Testing Questions

What command displays statistics about the DHCP pool, including the number of addresses in use and available?

Write the complete command to display only the DHCP-related configuration from the router's running configuration.

When you ping 192.168.10.1 from PC1, what does a successful reply confirm about your network? List at least three things.

If PC1 (192.168.10.21) can ping PC2 (192.168.10.22) but cannot ping the router at 192.168.10.1, what could be wrong?