OSPF Configuration Between 3 Routers and 3 Laptops

Lab overview

This lab guides students through building a three-router topology with three laptops and configuring OSPF so all LANs can communicate across the routed network.

Objectives

By the end of this lab, students should be able to:

- Build a three-router topology using physical hardware.

- Configure IPv4 addressing on router interfaces and laptop NICs.

- Enable OSPF and advertise the required networks in Area 0.

- Verify OSPF neighbor relationships and end-to-end connectivity.

Devices and topology

Use the following devices for the lab:

- 3 routers labeled R1, R2, and R3.

- 3 laptops labeled PC1, PC2, and PC3.

- Ethernet cables as needed for router-to-router and laptop-to-router or laptop-to-switch connections.

- Console cables for local router configuration.

Connect the topology as follows:

- R1 to R2.

- R2 to R3.

- PC1 to R1.

- PC3 to R2.

- PC2 to R3.

If the router model uses FastEthernet instead of GigabitEthernet, substitute the equivalent interface numbers while keeping the same IP addressing plan.

Addressing plan

Use this addressing scheme for the lab.[cite:1]

| Device | Interface | IP address | Subnet mask |
| --- | --- | --- | --- |
| R1 | G0/0 | 10.0.0.1 | 255.0.0.0 |
| R1 | G0/1 | 192.168.1.1 | 255.255.255.0 |
| R2 | G0/0 | 10.0.0.2 | 255.0.0.0 |
| R2 | G0/1 | 11.0.0.2 | 255.0.0.0 |
| R2 | G0/2 | 192.168.2.1 | 255.255.255.0 |
| R3 | G0/0 | 11.0.0.1 | 255.0.0.0 |
| R3 | G0/1 | 12.0.0.1 | 255.0.0.0 |
| PC1 | NIC | 192.168.1.10 | 255.255.255.0 |
| PC3 | NIC | 192.168.2.10 | 255.255.255.0 |
| PC2 | NIC | 12.0.0.10 | 255.0.0.0 |

Default gateways:

- PC1: 192.168.1.1

- PC3: 192.168.2.1

- PC2: 12.0.0.1

Part 1: Build the topology

- Place the three routers on the bench and label them R1, R2, and R3.

- Place the three laptops near the routers and label them PC1, PC2, and PC3.

- Connect R1 to R2 using the interfaces assigned to the 10.0.0.0 network.

- Connect R2 to R3 using the interfaces assigned to the 11.0.0.0 network.

- Connect PC1 to the LAN interface of R1.

- Connect PC3 to the LAN interface of R2.

- Connect PC2 to the LAN interface of R3.

- Power on all devices and wait for link lights to stabilize before starting configuration.

Part 2: Configure router interfaces

Configure R1

Enter the following commands on R1:

enable
configure terminal
interface gigabitEthernet0/0
 ip address 10.0.0.1 255.0.0.0
 no shutdown
exit
interface gigabitEthernet0/1
 ip address 192.168.1.1 255.255.255.0
 no shutdown
exit
end
write memory

Configure R2

Enter the following commands on R2:

enable
configure terminal
interface gigabitEthernet0/0
 ip address 10.0.0.2 255.0.0.0
 no shutdown
exit
interface gigabitEthernet0/1
 ip address 11.0.0.2 255.0.0.0
 no shutdown
exit
interface gigabitEthernet0/2
 ip address 192.168.2.1 255.255.255.0
 no shutdown
exit
end
write memory

Configure R3

Enter the following commands on R3:

enable
configure terminal
interface gigabitEthernet0/0
 ip address 11.0.0.1 255.0.0.0
 no shutdown
exit
interface gigabitEthernet0/1
 ip address 12.0.0.1 255.0.0.0
 no shutdown
exit
end
write memory

After configuring the interfaces, verify interface status on each router with the following command:

show ip interface brief

Part 3: Configure laptop IP settings

Configure each laptop NIC with the following settings:

- PC1: IP address 192.168.1.10, subnet mask 255.255.255.0, default gateway 192.168.1.1.

- PC3: IP address 192.168.2.10, subnet mask 255.255.255.0, default gateway 192.168.2.1.

- PC2: IP address 12.0.0.10, subnet mask 255.0.0.0, default gateway 12.0.0.1.

Verify the laptop addressing with the operating system network tools after configuration.

Part 4: Configure OSPF

OSPF is configured on each router to advertise directly connected networks in Area 0.
The OSPF process IDs can be different on each router because they are locally significant.

Configure OSPF on R1

configure terminal
router ospf 1
 network 10.0.0.0 0.255.255.255 area 0
 network 192.168.1.0 0.0.0.255 area 0
end
write memory

Configure OSPF on R2

configure terminal
router ospf 2
 network 10.0.0.0 0.255.255.255 area 0
 network 11.0.0.0 0.255.255.255 area 0
 network 192.168.2.0 0.0.0.255 area 0
end
write memory

Configure OSPF on R3

configure terminal
router ospf 3
 network 11.0.0.0 0.255.255.255 area 0
 network 12.0.0.0 0.255.255.255 area 0
end
write memory

Part 5: Verify routing

Use the following commands to confirm OSPF is working correctly:

- Check OSPF neighbors on each router:

show ip ospf neighbor

- Check the routing table on each router:

show ip route

- Confirm that OSPF-learned routes appear with the code O in the routing table.

Part 6: Test connectivity

From each laptop, open the command prompt and test connectivity to the other LANs.

Suggested ping tests:

- From PC1: ping 192.168.2.10 and ping 12.0.0.10.

- From PC2: ping 192.168.1.10 and ping 192.168.2.10.

- From PC3: ping 192.168.1.10 and ping 12.0.0.10.

Successful replies confirm that OSPF exchanged routes correctly across the topology.

Questions

Answer the following after completing the lab:

- Which routes on R1 were learned through OSPF?

- What command is used to verify OSPF neighbor relationships?

- Why do the routers need matching OSPF area numbers on connected links?

- What happens if a LAN network is omitted from the OSPF network statements?

- Were all ping tests successful? If not, what troubleshooting steps were used?

- What is the difference between a directly connected route and an OSPF-learned route?