Lab 5: DHCP from the Router for a Single VLAN

Objective

Configure a Cisco router to act as a DHCP server for a single LAN so that PCs connected through a switch receive IP addresses automatically.[cite:7][cite:9]

Topology

PC(s) connect to a switch, and the switch connects to the router using one LAN interface.[cite:1][cite:8]

Devices and addressing plan

- 1 Cisco router

- 1 Cisco switch

- 1 to 3 PCs or Packet Tracer PCs

- Router LAN interface: 192.168.10.1/24

- DHCP network: 192.168.10.0/24

- DHCP pool name: LAN-POOL[cite:8][cite:9]

Learning outcomes

After completing this lab, students should be able to:

- Explain why DHCP reduces manual IP configuration and helps avoid addressing errors.[cite:7][cite:11]

- Configure a Cisco router as a DHCP server for one VLAN or one flat LAN.[cite:7][cite:10]

- Verify leases with Cisco IOS show commands.[cite:7][cite:9]

- Test end-to-end connectivity from a client to the default gateway.[cite:8][cite:9]

Task summary

Students will complete the following tasks:

- Configure a DHCP pool named LAN-POOL on the router.[cite:7][cite:10]

- Exclude reserved addresses using ip dhcp excluded-address.[cite:7][cite:9]

- Configure router interface G0/0 with IP address 192.168.10.1/24 and enable the interface.[cite:8][cite:9]

- Set the PCs to obtain addressing automatically through DHCP.[cite:8]

- Verify DHCP bindings and pool usage with show ip dhcp binding and show ip dhcp pool.[cite:7][cite:9]

- Ping the default gateway and, if available later, test connectivity beyond the router.[cite:8]

Physical setup

- Connect the router LAN interface G0/0 to a switch access port using a straight-through Ethernet cable.

- Connect each PC to an access port on the switch.

- Power on the router, switch, and PCs.[cite:1][cite:8]

Step 1: Configure the router interface

Enter privileged EXEC mode and then global configuration mode.

Router> enable
Router# configure terminal
Router(config)# hostname R1
R1(config)# interface g0/0
R1(config-if)# ip address 192.168.10.1 255.255.255.0
R1(config-if)# no shutdown
R1(config-if)# exit

This interface becomes the default gateway for all DHCP clients on the LAN.[cite:8][cite:9]

Verify the interface

Use the following command to confirm the interface is up and has the correct IP address.

R1# show ip interface brief

G0/0 should display the address 192.168.10.1 and an up/up status when cabling is correct.[cite:9]

Step 2: Exclude reserved addresses

Exclude addresses that should not be leased dynamically, such as the router address and any future static devices.[cite:7][cite:11]

R1(config)# ip dhcp excluded-address 192.168.10.1 192.168.10.20

This example reserves the first 20 addresses in the subnet, including the router gateway address.[cite:9]

Step 3: Create the DHCP pool

Create a DHCP pool and define the network, default gateway, and DNS server values that clients will receive.[cite:7][cite:10]

R1(config)# ip dhcp pool LAN-POOL
R1(dhcp-config)# network 192.168.10.0 255.255.255.0
R1(dhcp-config)# default-router 192.168.10.1
R1(dhcp-config)# dns-server 8.8.8.8
R1(dhcp-config)# exit

The network command defines the scope, default-router provides the gateway, and dns-server supplies name-resolution information to clients.[cite:7][cite:11]

Step 4: Configure switch ports for a single VLAN

In a single-VLAN lab, all PC-facing ports and the router-facing port should be configured as access ports in the same VLAN.[cite:1][cite:2]

Example switch configuration:

Switch> enable
Switch# configure terminal
Switch(config)# interface range f0/1 - 3
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 1
Switch(config-if-range)# no shutdown
Switch(config-if-range)# exit
Switch(config)# interface f0/24
Switch(config-if)# switchport mode access
Switch(config-if)# switchport access vlan 1
Switch(config-if)# no shutdown

Any single VLAN may be used, but every attached device must remain in the same VLAN for this flat-LAN lab.[cite:1]

Step 5: Configure the PCs for DHCP

On each PC, open the network settings and select DHCP or automatic IP addressing.[cite:8]

In Packet Tracer, use the following path:

- Desktop

- IP Configuration

- DHCP[cite:8]

After a few seconds, each PC should receive:

- An IP address from the 192.168.10.0/24 network

- Subnet mask 255.255.255.0

- Default gateway 192.168.10.1

- DNS server 8.8.8.8[cite:9][cite:11]

Because addresses 192.168.10.1 through 192.168.10.20 were excluded, clients should normally receive addresses starting at 192.168.10.21 or higher.[cite:9]

Step 6: Verify DHCP operation on the router

Use the following commands to verify that the router is leasing addresses correctly.[cite:7][cite:9]

View active bindings

R1# show ip dhcp binding

This command displays leased IP addresses and the client identifiers associated with those leases.[cite:7]

View pool statistics

R1# show ip dhcp pool

This command shows total addresses, leased addresses, and available addresses in the pool.[cite:7][cite:9]

Optional conflict check

R1# show ip dhcp conflict

This command helps identify duplicate or conflicting addresses detected by the router DHCP service.[cite:7]

Step 7: Test connectivity

From a PC command prompt, ping the default gateway.

ping 192.168.10.1

Successful replies confirm that the PC received correct addressing information and can reach the router at Layer 3.[cite:8][cite:9]

If the router is later connected to an upstream network and has a default route, students can also test connectivity beyond the local gateway.[cite:7][cite:8]

Troubleshooting tips

- If the PC does not receive an IP address, verify that the router interface is up and not administratively down.[cite:9]

- If the DHCP pool does not work, verify the network, default-router, and dns-server entries under the DHCP pool.[cite:7][cite:10]

- If the PC and router cannot communicate, confirm that the switch ports are in the same VLAN and are enabled.[cite:1][cite:2]

- If bindings do not appear, renew the PC address request and check show ip dhcp binding again.[cite:7][cite:8]

Questions for students

- What are two benefits of DHCP compared with manual static addressing?[cite:7][cite:11]

- Why is the router interface address excluded from the DHCP pool?[cite:7][cite:9]

- Which command shows current DHCP leases on the router?[cite:7]

- Which command shows DHCP pool statistics?[cite:7]

- What should the PC default gateway be in this lab?[cite:8][cite:9]

Expected result

By the end of the lab, PCs should automatically obtain valid IP configuration from the router DHCP service and successfully ping 192.168.10.1 through the switch in a single-VLAN LAN.[cite:8][cite:9]