Lab 6: Security Basics (Passwords, SSH, and Port Security Lite)

Lab overview

This lab introduces basic management-plane security on Cisco routers and switches by using local user accounts, SSH-only remote access, login protections, and simple switch port security.[cite:1][cite:12][cite:10]
Students will configure a router and a switch, test secure remote access from a PC, and observe how a switch reacts when a protected access port sees an unexpected MAC address.[cite:1][cite:12][cite:10]

Objective

By the end of this lab, students should be able to:

- Configure local user accounts with privilege levels on a router and a switch.

- Enable SSH by setting a domain name, generating RSA keys, and limiting VTY access to SSH with local login.[cite:1][cite:14]

- Test SSH access from a PC using PuTTY or OpenSSH.[cite:1]

- Configure login block-for and exec-timeout on the switch to improve management access security.[cite:12][cite:6]

- Configure basic port security on an access port using a maximum of one MAC address and sticky learning.[cite:7][cite:10]

- Trigger and verify a port-security violation by moving the PC or changing the NIC.[cite:7][cite:10]

Learning points

- Basic management-plane hardening reduces unnecessary exposure of the device management interface by using local authentication and SSH instead of Telnet.[cite:1][cite:14]

- login block-for helps slow repeated failed login attempts, while exec-timeout automatically closes idle management sessions.[cite:12][cite:6]

- Switch port security is a simple Layer 2 control that limits which MAC addresses can use an access port.[cite:7][cite:10]

Suggested topology

Use one router, one switch, and one PC connected as follows:

- PC to switch access port, such as FastEthernet0/1 or GigabitEthernet0/1.

- Switch to router using any available Ethernet uplink.

- Console access available to both devices for initial setup.

Example addressing plan:

| Device | Interface | IP address | Subnet mask | Default gateway |
| --- | --- | --- | --- | --- |
| Router | G0/0 or F0/0 | 192.168.10.1 | 255.255.255.0 | N/A |
| Switch | VLAN 1 | 192.168.10.2 | 255.255.255.0 | 192.168.10.1 |
| PC | NIC | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |

Required equipment

- 1 Cisco router with IOS support for SSH.[cite:1]

- 1 Cisco switch that supports SSH and port security on access ports.[cite:14][cite:10]

- 1 Windows or Linux PC with PuTTY or OpenSSH client.

- Console cables and Ethernet patch cables.

Before you begin

- Erase old configurations if instructed by the instructor.

- Confirm physical cabling and interface status.

- Assign the IP addresses from the addressing plan.

- Verify the PC can ping the router and switch management IP addresses before starting SSH configuration.

Part 1: Base IP configuration

Configure basic connectivity first so SSH testing can work.

Router base setup

Enter the following on the router, adjusting interface names if needed:

enable
configure terminal
hostname R1
interface g0/0
 ip address 192.168.10.1 255.255.255.0
 no shutdown
exit

Switch base setup

Enter the following on the switch, adjusting interface names if needed:

enable
configure terminal
hostname S1
interface vlan 1
 ip address 192.168.10.2 255.255.255.0
 no shutdown
exit
ip default-gateway 192.168.10.1

PC configuration

Set the PC NIC to:

- IP address: 192.168.10.10

- Subnet mask: 255.255.255.0

- Default gateway: 192.168.10.1

Verification

Run these checks:

- From the PC, ping 192.168.10.1.

- From the PC, ping 192.168.10.2.

- On each Cisco device, use show ip interface brief to verify interface status.

Part 2: Configure local users

Local usernames can be used for device management authentication, and privilege levels can control user access after login.[cite:1]

Router local user

configure terminal
username admin privilege 15 secret Cisco123!
username student privilege 1 secret Student123!
end
write memory

Switch local user

configure terminal
username admin privilege 15 secret Cisco123!
username student privilege 1 secret Student123!
end
write memory

Verification

Use:

show running-config | include username

Record the configured usernames and privilege levels in your notes.

Part 3: Enable SSH on the router

Cisco SSH setup requires a hostname, a domain name, RSA key generation, and VTY line configuration for SSH with local login.[cite:1][cite:14]

Router SSH configuration

configure terminal
ip domain-name lab.local
crypto key generate rsa
1024
ip ssh version 2
line vty 0 4
 login local
 transport input ssh
 exec-timeout 5 0
exit
end
write memory

Explanation

- ip domain-name lab.local is required before generating RSA keys on Cisco IOS.[cite:1][cite:14]

- crypto key generate rsa creates the key pair used by SSH.[cite:1][cite:14]

- transport input ssh disables Telnet on the VTY lines so only SSH is allowed.[cite:1]

- login local tells the VTY lines to use the local user database for authentication.[cite:1]

- exec-timeout 5 0 disconnects idle sessions after 5 minutes.[cite:6]

Verify router SSH

Use these commands:

show ip ssh
show crypto key mypubkey rsa
show running-config | section line vty

Part 4: Enable SSH on the switch

Switch SSH configuration follows the same general process: set a domain name, generate RSA keys, and configure the VTY lines for SSH-only remote access with local login.[cite:1][cite:14]

Switch SSH configuration

configure terminal
ip domain-name lab.local
crypto key generate rsa
1024
ip ssh version 2
line vty 0 4
 login local
 transport input ssh
 exec-timeout 5 0
exit
end
write memory

Add login protections on the switch

The login block-for feature temporarily blocks further login attempts after too many failures within a specified time window, and exec-timeout closes idle sessions automatically.[cite:12][cite:6]

configure terminal
login block-for 120 attempts 3 within 60
line console 0
 exec-timeout 5 0
exit
line vty 0 4
 exec-timeout 5 0
exit
end
write memory

Verify switch SSH and login protection

Use these commands:

show ip ssh
show running-config | include login block-for
show running-config | section line vty
show running-config | section line con

Part 5: Test SSH from the PC

After SSH is enabled, students should confirm that Telnet is not allowed and that SSH authentication works with local usernames.[cite:1]

Using PuTTY on Windows

- Open PuTTY.

- In Host Name, enter 192.168.10.1 to test the router.

- Confirm port 22 and SSH are selected.

- Click Open.

- Log in with admin and the configured password.

- Repeat for 192.168.10.2 to test the switch.

Using OpenSSH from Windows Terminal, PowerShell, or Linux

ssh admin@192.168.10.1
ssh admin@192.168.10.2

What to observe

- The device should prompt for a username and password.

- Login should succeed with the local account.

- A Telnet attempt should fail because VTY transport is limited to SSH only.[cite:1]

Suggested verification commands after login

show users
show privilege

Part 6: Configure simple port security on the switch

Port security on an access interface can limit the number of MAC addresses, learn the attached device MAC address with sticky learning, and take action when a violation occurs.[cite:7][cite:10]

Use the switch port connected to the PC. The example below uses FastEthernet0/1.

Port security configuration

configure terminal
interface f0/1
 switchport mode access
 switchport port-security
 switchport port-security maximum 1
 switchport port-security mac-address sticky
 switchport port-security violation shutdown
 no shutdown
exit
end
write memory

Explanation

- switchport port-security enables port security on the access port.[cite:7]

- maximum 1 allows only one source MAC address on the port.[cite:7]

- mac-address sticky tells the switch to learn the current device MAC address and retain it in the running configuration.[cite:7]

- violation shutdown places the port into an error-disabled state when an unauthorized MAC address appears; this is the common default violation behavior discussed in Cisco training references.[cite:7][cite:10]

Verify the secured port

Use these commands:

show port-security
show port-security interface f0/1
show running-config interface f0/1
show mac address-table interface f0/1

Record:

- Maximum MAC allowed.

- Current learned sticky MAC address.

- Violation count.

- Port status.

Part 7: Create a port-security violation

A violation can be triggered by moving the PC to another port and connecting a different device to the secured port, or by changing the NIC or MAC identity seen on that interface.[cite:7][cite:10]

Method A: Move the original PC

- Disconnect the current PC from F0/1.

- Connect a different PC or device to F0/1.

- Wait a few seconds.

- Check the port status.

Method B: Change the device on the secured port

- Leave the original cabling in place.

- Replace the NIC identity if your lab platform allows it, or connect a different device.

- Observe the interface state and security counters.

Verification commands

show port-security
show port-security interface f0/1
show interfaces status
show interfaces f0/1

Expected results

- The violation counter should increase.[cite:7]

- In shutdown mode, the interface may enter an error-disabled or down state after the violation.[cite:7][cite:10]

- The interface status and port-security output should clearly indicate that a security violation occurred.[cite:7][cite:10]

Part 8: Recover the secured port

If the interface shuts down because of a violation, it can typically be restored by clearing the condition and re-enabling the interface.[cite:10]

Recovery steps

configure terminal
interface f0/1
 shutdown
 no shutdown
exit
end

If needed, remove sticky entries and re-learn the correct MAC:

clear port-security sticky interface f0/1

Then reconnect the correct PC and verify the learned MAC address again.

Student tasks checklist

Complete each task and get instructor initials if required.

| Task | Done |
| --- | --- |
| Configure router hostname and IP address |  |
| Configure switch management IP and default gateway |  |
| Create local users on router and switch |  |
| Enable SSH on router |  |
| Enable SSH on switch |  |
| Configure SSH-only VTY access with local login |  |
| Configure login block-for on switch |  |
| Configure exec-timeout on console and VTY lines |  |
| Test SSH from the PC to router and switch |  |
| Configure access-port security with sticky MAC |  |
| Trigger a port-security violation |  |
| Verify violation using show commands |  |
| Recover the interface |  |

Show commands reference

Use these commands during the lab:

show ip interface brief
show running-config | section line vty
show running-config | section line con
show running-config | include username
show ip ssh
show crypto key mypubkey rsa
show users
show privilege
show port-security
show port-security interface f0/1
show interfaces status
show mac address-table interface f0/1

Reflection questions

- Why is SSH preferred over Telnet for remote management?[cite:1]

- What is the purpose of RSA keys in SSH configuration?[cite:1][cite:14]

- What does login local do on the VTY lines?[cite:1]

- How does login block-for 120 attempts 3 within 60 improve security?[cite:12]

- What does exec-timeout protect against?[cite:6]

- What happened to the switch port during the port-security violation?[cite:7][cite:10]

- Why is sticky MAC useful on end-user access ports?[cite:7]

Troubleshooting tips

- If RSA key generation fails, verify that both the hostname and domain name are configured first.[cite:1][cite:14]

- If SSH fails from the PC, verify IP connectivity, VTY settings, and local usernames.[cite:1]

- If the switch does not show a violation, verify that port security is enabled on an access port and that a different MAC address was introduced.[cite:7][cite:10]

- If the port stays down after a violation, check interface status and use shutdown followed by no shutdown to recover it.[cite:10]

Instructor notes

- The instructor may choose whether students use admin only or both admin and student accounts.

- The instructor may keep the port-security section optional if lab time is short.

- Interface names may differ by hardware model, such as Fa0/1, Gi0/1, or Gi1/0/1.

- Key modulus size may vary by IOS version and platform requirements, although RSA key generation is the required step for enabling SSH.[cite:1][cite:14]