Router-on-a-Stick (Inter-VLAN Routing) Lab Guide

Introduction

Inter-VLAN routing enables communication between devices on different VLANs. This lab guide demonstrates the Router-on-a-Stick method, which uses a single physical link between a switch and router, divided into multiple logical subinterfaces to handle multiple VLANs.

Objectives

By completing this lab, you will be able to:

- Understand why inter-VLAN routing is necessary

- Configure VLANs on a Cisco switch

- Create subinterfaces on a router

- Configure encapsulation (802.1Q tagging)

- Assign IP addresses to logical subinterfaces

- Test connectivity between devices on different VLANs

- Troubleshoot inter-VLAN routing issues

Prerequisites

Equipment Required:

- Cisco Switch (e.g., Catalyst 2960)

- Cisco Router (e.g., 1811 or higher)

- 2 Personal Computers

- Ethernet cables

- Console cable for device configuration

- Terminal emulation software (PuTTY, SecureCRT)

Physical Connections

- Router Fa0/0 (FastEthernet 0/0) → Switch Fa0/3 (trunk link)

- Switch Fa0/1 (VLAN 1) → PC1

- Switch Fa0/2 (VLAN 2) → PC2

Configuration Steps

Step 1: Switch Configuration

1.1 Create VLANs

Log into the switch and enter configuration mode:

Switch> enable
Switch# configure terminal
Switch(config)# vlan 10
Switch(config-vlan)# exit
Switch(config)# vlan 20
Switch(config-vlan)# exit

1.2 Assign Ports to VLANs

Configure switch ports to belong to specific VLANs:

Switch(config)# interface range fastethernet 0/2-12
Switch(config-if)# switchport mode access
Switch(config-if)# switchport access vlan 10
Switch(config-if)# exit

Switch(config)# interface range fastethernet 0/13-24
Switch(config-if)# switchport mode access
Switch(config-if)# switchport access vlan 20
Switch(config-if)# exit

1.3 Configure Trunk Port

Configure the port connected to the router as a trunk:

Switch(config)# interface fastethernet 0/1

Switch(config-if)# switchport trunk encapsulation dot1q
Switch(config-if)# switchport mode trunk
Switch(config-if)# switchport trunk allowed vlan 10,20
Switch(config-if)# exit
Switch(config)# exit
Switch# copy running-config startup-config

Verification:

Switch# show vlan brief

Switch# show interface fastethernet 0/1 switchport

Step 2: Router Configuration

2.1 Create Subinterface for VLAN 10

Access the router and configure the first subinterface:

Router> enable
Router# configure terminal
Router(config)# interface fastethernet 0/0.1
Router(config-subif)# encapsulation dot1q 10
Router(config-subif)# ip address 192.168.1.1 255.255.255.0
Router(config-subif)# exit

Command Explanation:

- interface fastethernet 0/0.1 = Physical interface (Fa0/0) with subinterface number (1)

- encapsulation dot1q 10 = Uses 802.1Q tagging for VLAN 10

- ip address 192.168.1.1 255.255.255.0 = Gateway IP for VLAN 10

2.2 Create Subinterface for VLAN 20

Router(config)# interface fastethernet 0/0.2
Router(config-subif)# encapsulation dot1q 20
Router(config-subif)# ip address 192.168.2.1 255.255.255.0
Router(config-subif)# exit

2.3 Activate the Physical Interface

The physical interface must be enabled to support the subinterfaces:

Router(config)# interface fastethernet 0/0
Router(config-if)# no shutdown
Router(config-if)# exit
Router(config)# exit
Router# copy running-config startup-config

Verification:

Router# show ip route
Router# show interface fastethernet 0/0
Router# show interface fastethernet 0/0.1
Router# show interface fastethernet 0/0.2

Step 3: Configure End Devices

3.1 Configure PC1 (VLAN 10)

- IP Address: 192.168.1.100

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

- DNS (optional): 8.8.8.8

3.2 Configure PC2 (VLAN 20)

- IP Address: 192.168.2.100

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.2.1

- DNS (optional): 8.8.8.8

Testing and Verification

Test 1: Different VLAN Communication

Test communication between VLANs through the router: PC1 to PC2

PC1> ping 192.168.2.100

Expected Result: Replies received (routing through router)

Test communication between VLANs through the router: PC2 to PC1

PC2> ping 192.168.1.100

Expected Result: Replies received (routing through router)

Test 3: Gateway Reachability

Verify that each device can reach its default gateway:

PC1> ping 192.168.1.1
PC2> ping 192.168.2.1

Expected Result: All replies successful

Router Verification Commands

Verify the router configuration:

Router# show running-config
Router# show ip interface brief
Router# show interfaces trunk
Router# show vlan-switch brief
Router# show ip route

Debug Connectivity Issues

If communication fails between VLANs, use these troubleshooting commands:

Router# debug ip packet
Router# ping 192.168.2.100
Router# undebug all

Alternatively, check ARP tables:

Router# show arp
Router# show ip arp