LAB WORKSHEET

Basic Router Security Configuration (Cisco IOS)

| Name: | Date: | Period: |
| --- | --- | --- |

# Overview

In this lab you will configure fundamental security settings on Cisco routers using the IOS command-line interface (CLI). You will protect access to the device using passwords, encrypt those passwords, and set up basic console, privileged-EXEC, and remote-access protections. This worksheet follows three short video labs by Jeremy's IT Lab.

▶ Lab 1 — Basic Router Security Configuration (Part 1)

▶ Lab 2 — Basic Router Security Configuration (Part 2)

▶ Lab 3 — Basic Router Security Configuration (Part 3)

## Objectives

- Enter and move between the IOS command modes (user EXEC, privileged EXEC, global config).

- Set a device hostname and secure privileged-EXEC mode with an enable secret.

- Secure the console line and the virtual terminal (VTY) lines with passwords.

- Encrypt plaintext passwords and apply a minimum password length.

- Verify the configuration and save it to the startup configuration.

## Materials

- Cisco Packet Tracer (or real/simulated Cisco router).

- The Packet Tracer lab files from Jeremy's IT Lab (jeremysitlab.com).

- This worksheet and a pen for recording answers.

## Key IOS Command Modes

| Mode | Prompt looks like | How you get there |
| --- | --- | --- |
| User EXEC | Router> | Default when you log in |
| Privileged EXEC | Router# | Type: enable |
| Global Config | Router(config)# | Type: configure terminal |
| Line Config | Router(config-line)# | Type: line console 0 (or) line vty 0 4 |

# Part 1 — Hostname and Enable Secret

Watch Lab 1, then complete the steps below on your own device.

## Procedure

- Enter privileged EXEC mode, then global configuration mode.

- Set the router's hostname to a name your instructor assigns (e.g., R1).

- Configure an enable secret password to protect privileged EXEC mode.

- Exit to privileged EXEC and verify the prompt shows your new hostname.

## Commands — fill in the blanks

Router> ______________            ! enter privileged EXEC

Router# ______________            ! enter global config

Router(config)# hostname ________

R1(config)# enable secret ________

R1(config)# ______  ! return to privileged EXEC

Q1. What is the difference between the 'enable password' and the 'enable secret' command? Which is more secure and why?

Q2. After setting the enable secret, what happens when you type 'enable' at the Router> prompt?

# Part 2 — Securing the Console and Password Encryption

Watch Lab 2, then complete the steps below.

## Procedure

- Enter line configuration mode for the console (line console 0).

- Set a console password and enable login so the password is required.

- Return to global config and apply 'service password-encryption'.

- Run 'show running-config' and observe how the passwords now appear.

## Commands — fill in the blanks

R1(config)# line console 0

R1(config-line)# password ________

R1(config-line)# ________            ! require the password at login

R1(config-line)# exit

R1(config)# ________________________  ! encrypt plaintext passwords

Q3. Before running 'service password-encryption', how did the console password appear in the running-config? How did it appear afterward?

Q4. Does 'service password-encryption' use strong encryption? Explain why an administrator should still use 'enable secret' for the privileged password.

# Part 3 — VTY Lines, Minimum Password Length, and Saving

Watch Lab 3, then complete the steps below.

## Procedure

- Set a minimum password length using 'security passwords min-length'.

- Enter line configuration mode for the VTY lines (line vty 0 4).

- Set a VTY password and enable login for remote access.

- Verify your configuration with 'show running-config'.

- Save the configuration to the startup configuration file.

## Commands — fill in the blanks

R1(config)# security passwords min-length ____

R1(config)# line vty 0 4

R1(config-line)# password ________

R1(config-line)# login

R1(config-line)# end

R1# ______________________        ! save config (copy run start)

Q5. What do the numbers '0 4' represent in the command 'line vty 0 4'?

Q6. Why must you set a VTY password before a router will accept Telnet/SSH connections?

Q7. What is the difference between the running-configuration and the startup-configuration? What happens to unsaved changes if the router reboots?

# Verification Checklist

Check off each item once you have confirmed it on your device.

| ✓ | Item |
| --- | --- |
| ☐ | Hostname is set correctly and shows in the prompt. |
| ☐ | enable secret is configured (try 'enable' from user EXEC). |
| ☐ | Console line requires a password at login. |
| ☐ | service password-encryption is applied (passwords are scrambled in running-config). |
| ☐ | Minimum password length is enforced. |
| ☐ | VTY lines have a password and login enabled. |
| ☐ | Configuration is saved to startup-config. |

# Conclusion

In 3–4 complete sentences, summarize what you configured in this lab and explain why each security measure matters in a real network.