Routing Protocol Labs for 1 Router + 1 Switch + 2 Laptops

1. RIP v2 Lab — Basic Dynamic Routing

Goal: Teach students how dynamic routing works using the simplest protocol.

How it works with one router: You create multiple loopback interfaces on the router, each representing a different “remote network.”

Students learn:

- Enabling RIP v2

- Advertising networks

- Verifying with show ip route

- Testing reachability from laptops

Why it’s great: Perfect intro to routing protocols — very visual, very easy.

2. EIGRP Lab — Classic Cisco Routing Protocol

Goal: Show how a distance-vector protocol with advanced features behaves.

How it works:

- Create 2–4 loopbacks on the router

- Advertise them using EIGRP

- Laptops ping all loopback networks

- Observe DUAL behavior with show ip eigrp topology

Students learn:

- Autonomous system numbers

- EIGRP metrics

- Feasible successors

- Passive vs active routes

Why it’s great: EIGRP gives you richer output and deeper learning than RIP.

3. OSPF Single-Area Lab

Goal: Teach link-state routing fundamentals.

How it works:

- Router has multiple loopbacks

- Switch + laptops form the “LAN segment”

- Router advertises loopbacks into OSPF Area 0

- Laptops test connectivity

Students learn:

- OSPF neighbor formation (even if only one neighbor)

- LSAs and SPF tree

- Router ID selection

- OSPF cost and interface tuning

Why it’s great: OSPF is required for CCNA — this lab gives a clean, simple setup.

4. OSPF Multi-Area Simulation (Using Loopbacks)

Goal: Teach multi-area design using only one router.

How it works:

- Assign loopbacks to different areas

  - Loopback0 → Area 0

  - Loopback1 → Area 1

  - Loopback2 → Area 2

- Router becomes an ABR

- Laptops test reachability to all areas

Students learn:

- ABR behavior

- Inter-area routes

- Type 3 LSAs

- Area design concepts

Why it’s great: You simulate a multi-router OSPF network with zero extra hardware.

5. EIGRP vs OSPF Comparison Lab

Goal: Show how different routing protocols choose routes.

How it works:

- Advertise the same loopback networks under EIGRP and OSPF

- Use administrative distance to see which protocol wins

- Change AD to flip the winner

Students learn:

- Administrative distance

- Protocol preference

- How routing tables resolve conflicts

Why it’s great: This is a powerful conceptual lab that students love.

6. Passive Interface Lab (RIP, EIGRP, or OSPF)

Goal: Teach how to prevent routing updates on certain interfaces.

How it works:

- Router advertises loopbacks

- LAN interface is set to passive

- Laptops still reach all networks

- Students observe that routing updates stop on that interface

Students learn:

- Security best practices

- Control-plane traffic

- Why passive interfaces matter