Lab Overview

In this lab, you will build a simple network: PC — Switch — Router, then configure static IP addresses so the PC can reach its default gateway (the router). You will also verify switch Layer 2 information and test connectivity with ping and traceroute.

Topology and Addressing

Physical/logical topology:

- PC ↔ Switch ↔ Router G0/0

Suggested addressing (you can change per class):

- Router G0/0: 192.168.10.1 / 255.255.255.0

- PC: 192.168.10.10 / 255.255.255.0

- PC default gateway: 192.168.10.1

Note: the router interface IP is the default gateway for the PC.

Step 1 – Basic Device Setup

- Power on and cable the devices:

  - Connect PC NIC to any access port on the switch (for example, F0/1 or G0/1 depending on model).

  - Connect switch port (for example, F0/24 or G0/24) to router interface G0/0.

- On the router:

  - Enter privileged EXEC mode:

    - Router> enable

  - Enter global configuration mode:

    - Router# configure terminal

- On the switch:

  - Enter privileged EXEC mode:

    - Switch> enable

  - Enter global configuration mode:

    - Switch# configure terminal

Step 2 – Configure Router Interface IP

- From global configuration mode on the router, go to interface G0/0:

  - Router(config)# interface gigabitEthernet0/0

- Assign IP address and subnet mask:

  - Router(config-if)# ip address 192.168.10.1 255.255.255.0

- Bring the interface up:

  - Router(config-if)# no shutdown

- Verify the interface status and IP:

  - Exit to privileged EXEC: Router(config-if)# end

  - Check interface:

    - Router# show ip interface brief

  - Confirm G0/0 is up/up and shows 192.168.10.1.

Step 3 – Configure PC with Static IP and Default Gateway

If using Packet Tracer or a real PC, have students set IP manually, not via DHCP.

- On the PC:

  - Open the network settings / IP configuration window.

  - Select IPv4 settings.

- Configure:

  - IP address: 192.168.10.10

  - Subnet mask: 255.255.255.0

  - Default gateway: 192.168.10.1

- Apply/save the settings.

- (Optional check on Windows-like CLI if available):

  - Use a command similar to:
ipconfig to view current IP settings.

Step 4 – Verify Switch Port Status

Students will confirm Layer 2 connectivity between PC and router via the switch.

- On the switch (privileged EXEC mode):

  - View interface status:

    - Switch# show interfaces status

Have students locate:

  - The port connected to the PC (e.g., F0/1): verify it shows connected.

  - The port connected to the router (e.g., F0/24): verify it shows connected.

- View MAC address table:

  - Switch# show mac address-table

Discuss:

  - Dynamically learned MAC addresses (Type = DYNAMIC).

  - Which VLAN the PC and router MAC addresses are in.

Ask: “Are we looking at Layer 2 or Layer 3 information here?” (Answer: Layer 2 on the switch.)

Step 5 – Test Connectivity from the PC

- From the PC, open Command Prompt / terminal.

- Ping the default gateway:

  - ping 192.168.10.1

- Observe results:

  - Successful replies indicate end-to-end connectivity PC → Switch → Router on this LAN.

  - If pings fail, students should:

    - Re-check PC IP, mask, gateway.

    - Re-check cabling and switch port status.

    - Re-check router G0/0 status (must be up/up).

Step 6 – Test Connectivity and Tools from the Router

- From the router, ping the PC:

  - Router# ping 192.168.10.10

  - Successful replies confirm bidirectional connectivity on the LAN.

- Use traceroute from the router to the PC:

  - Router# traceroute 192.168.10.10

  - In a single-LAN topology, traceroute should typically show just one hop or stay local, illustrating there is no other router in the path.

- Have students also ping their own interface:

  - Router# ping 192.168.10.1

  - This should always succeed if the interface is up and correctly configured.

Questions based on this lab

- What is the role of the switch in this topology — Layer 2 or Layer 3? Explain your answer.

- What is the role of the router in this topology — Layer 2 or Layer 3? Explain your answer.

- Why does the PC need a default gateway, and which device provides it?

- What IP address is used as the default gateway in this lab, and why?

- Why can a PC communicate only with devices in its own subnet if the default gateway is missing or incorrect?

- Which command is used on the router to assign an IP address to interface G0/0?

- After configuring the router interface, which command verifies that G0/0 is up/up?

- When configuring the PC, which three IPv4 settings must be entered manually?

- What subnet mask is used in this lab, and what does it represent?

- Why must the router interface be brought up with no shutdown?

- Which switch command displays the status of all switch ports?

- Which switch command shows the dynamically learned MAC addresses?

- When viewing the MAC address table, what does the Type = DYNAMIC entry mean?

- What information should you expect to see for the port connected to the PC?

- What information should you expect to see for the port connected to the router?

- What does a successful ping from the PC to 192.168.10.1 confirm?

- What does a successful ping from the router to 192.168.10.10 confirm?

- Why does traceroute show only one hop (or stay local) in this topology?

- What protocol does ping use, and at which OSI layer does it operate?

- Why is pinging the router’s own interface (192.168.10.1) always expected to succeed?

- If the PC cannot ping the router, list three things you should check first.

- If the switch does not show the PC’s MAC address, what might be wrong?

- If the router interface shows administratively down, what command fixes it?

- Why is verifying cabling important before checking IP configuration?

- Which three commands from this lab are most useful for verifying network connectivity, and what does each one show?