Step guide on how to do initial configuration on a router before we use this for setup.

1. Console access and cleanup

- Rack and power on the router.

- Connect your PC to the console port using console cable + terminal emulator (9600-8-N-1, no flow control).

- When you see the prompt, enter privileged EXEC:

  - Router> enable

- (Optional but recommended in lab) Erase any old config:

  - Router# write erase

  - Confirm, then Router# reload and say no to saving configuration.

- If you hit the “initial configuration dialog”, answer no so you can configure manually.

Example:

text

Router> enable

Router# write erase

Router# reload

System configuration has been modified. Save? [yes/no]: no

2. Basic identity and secure access

- Enter global config mode:

  - Router# configure terminal

- Set hostname:

  - Router(config)# hostname R1

- Set an encrypted enable secret:

  - R1(config)# enable secret <STRONG_PASSWORD>

- Turn on service password-encryption (so line passwords don’t appear in clear):

  - R1(config)# service password-encryption

- Disable DNS lookup to avoid long hangs when you mistype commands:

  - R1(config)# no ip domain-lookup

- (Optional) Set a domain name if you plan to use SSH:

  - R1(config)# ip domain-name lab.local

3. Secure console and vty lines

- Protect console access:

  - R1(config)# line console 0

  - R1(config-line)# password <CONSOLE_PASS>

  - R1(config-line)# login

  - R1(config-line)# exec-timeout 10 0 (10 minutes)

  - R1(config-line)# logging synchronous

  - R1(config-line)# exit

- Protect vty (Telnet/SSH) lines:

  - R1(config)# line vty 0 4

  - R1(config-line)# password <VTY_PASS> (or use local user, below)

  - R1(config-line)# login

  - R1(config-line)# transport input ssh (or ssh telnet in lab)

  - R1(config-line)# exec-timeout 10 0

  - R1(config-line)# logging synchronous

  - R1(config-line)# exit

- (Recommended) Use local user accounts instead of line passwords:

  - R1(config)# username admin privilege 15 secret <USER_PASS>

  - R1(config)# line vty 0 4

  - R1(config-line)# login local

  - R1(config-line)# transport input ssh

  - R1(config-line)# exit

If using SSH, also generate keys:

text

R1(config)# crypto key generate rsa modulus 2048

R1(config)# ip ssh version 2

4. Management interface and IP addressing

Decide which interface will be your management / upstream interface (e.g., G0/0 to ISP or lab switch).

- Show interface status to identify ports:

  - R1# show ip interface brief

- Go to the interface that should be active:

  - R1(config)# interface GigabitEthernet0/0

- Assign IP address and mask:

  - R1(config-if)# ip address 192.168.10.1 255.255.255.0

- Enable the interface:

  - R1(config-if)# no shutdown

- (Optional) Add a description:

  - R1(config-if)# description Link to LAN switch

- Repeat for WAN/other interfaces as needed.

If this router will be a DHCP client on its WAN (common with ISP CPE):

text

R1(config)# interface GigabitEthernet0/0

R1(config-if)# ip address dhcp

R1(config-if)# no shutdown

5. Default gateway / routing basics

For a router, you usually configure a default route, not a default gateway.

- If the router has a single upstream next hop:

text

R1(config)# ip route 0.0.0.0 0.0.0.0 <NEXT_HOP_IP>

- If the WAN interface gets IP via DHCP, you can use an interface default route:

text

R1(config)# ip route 0.0.0.0 0.0.0.0 GigabitEthernet0/0

- Verify routing:

text

R1# show ip route

You should see a S* 0.0.0.0/0 entry pointing to the configured next hop.

6. Basic services and logging

- Configure a local clock (or use NTP later):

text

R1# clock set 12:00:00 21 April 2026

- Set logging to buffer and timestamp messages:

text

R1(config)# service timestamps log datetime msec

R1(config)# logging buffered 8192 debugging

- (Optional) Configure NTP and syslog later once you have those servers.

7. Basic security hardening (pre-deployment)

For students, you can make this a checklist they always run:

- Disable unused services that are often on by default in labs:

  - R1(config)# no cdp run (if not needed)

  - R1(config)# no ip http server

  - R1(config)# no ip http secure-server

- Enable SSH only on management VLAN if appropriate (using access-class on vty).

- Plan and apply basic infrastructure ACLs once IP plan is known.

Many best-practice guides for Cisco routers recommend at least changing default credentials, disabling unused management protocols, and using encrypted access.

8. Save and verify

- Save the running configuration to NVRAM:

  - R1# copy running-config startup-config

- Verify:

  - R1# show running-config | include hostname|enable secret|line vty|interface

  - R1# show ip interface brief

  - R1# show version (to document IOS, uptime, etc.)

- Test basic reachability from the router:

  - R1# ping <DEFAULT_GATEWAY_OR_NEXT_HOP>

  - R1# ping 8.8.8.8 (if it has internet)