Cisco Router and Switch Hands-On Lab Worksheet

Lab Title

Single Router and Single Switch Lab: Basic Configuration, VLANs, Trunking, and Router-on-a-Stick

Lab Overview

This lab uses one Cisco router and one Cisco switch to teach foundational configuration, VLAN segmentation, trunking, and inter-VLAN routing concepts that are common in introductory routing and switching courses.[cite:19][cite:21] The Cisco Catalyst 2960-S is well suited for Layer 2 switching labs, while Cisco ISR routers such as the 1900, 1921, and 2911 support common branch-routing tasks used in hands-on training.[cite:19][cite:28]

Recommended Devices

- Switch: Cisco Catalyst 2960-S

- Router: Cisco 1900, 1921, or 2911 series router

- End devices: 2 PCs or 2 laptops

- Cables: 1 console cable, 3 Ethernet cables

Learning Objectives

By the end of this lab, students should be able to:

- Perform basic IOS setup on a router and a switch.

- Configure VLAN 10 and VLAN 20 on a Catalyst switch.

- Configure access ports and a trunk port.

- Configure router subinterfaces for router-on-a-stick.

- Verify inter-VLAN connectivity using show commands and ping tests.

- Troubleshoot common configuration mistakes using verification commands.[cite:19][cite:26]

Topology

- PC1 connected to Switch FastEthernet 0/1

- PC2 connected to Switch FastEthernet 0/2

- Switch GigabitEthernet 0/1 connected to Router GigabitEthernet 0/0

Addressing Plan

| Device | Interface | VLAN | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- | --- |
| Router | G0/0.10 | 10 | 192.168.10.1 | 255.255.255.0 | N/A |
| Router | G0/0.20 | 20 | 192.168.20.1 | 255.255.255.0 | N/A |
| PC1 | NIC | 10 | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| PC2 | NIC | 20 | 192.168.20.10 | 255.255.255.0 | 192.168.20.1 |

Part 1: Basic Device Setup

Tasks

- Connect to the router and switch through the console port.

- Enter privileged EXEC mode.

- Configure the hostname on each device.

- Disable DNS lookup.

- Configure the enable secret.

- Configure console password and enable login.

- Configure a message-of-the-day banner.

- Save the configuration.

Router Commands

enable
configure terminal
hostname R1
no ip domain-lookup
enable secret class
banner motd # Authorized Access Only #
line console 0
password cisco
login
exit
service password-encryption
end
copy running-config startup-config

Switch Commands

enable
configure terminal
hostname S1
no ip domain-lookup
enable secret class
banner motd # Authorized Access Only #
line console 0
password cisco
login
exit
service password-encryption
end
copy running-config startup-config

Verification

Use the following commands on both devices:

show running-config
show startup-config
show ip interface brief

Part 2: Create VLANs and Assign Access Ports

Tasks

- Create VLAN 10 named Faculty.

- Create VLAN 20 named Students.

- Assign FastEthernet 0/1 to VLAN 10.

- Assign FastEthernet 0/2 to VLAN 20.

- Verify VLAN membership.

Switch Commands

enable
configure terminal
vlan 10
name Faculty
exit
vlan 20
name Students
exit
interface fastethernet 0/1
switchport mode access
switchport access vlan 10
description PC1_in_VLAN10
no shutdown
exit
interface fastethernet 0/2
switchport mode access
switchport access vlan 20
description PC2_in_VLAN20
no shutdown
exit
end
copy running-config startup-config

Verification

show vlan brief
show interfaces fastethernet 0/1 switchport
show interfaces fastethernet 0/2 switchport

Part 3: Configure the Trunk to the Router

Tasks

- Configure the switch port connected to the router as a trunk.

- Allow VLANs 10 and 20 on the trunk.

- Verify trunk status.

Switch Commands

enable
configure terminal
interface gigabitethernet 0/1
description Link_to_R1
switchport mode trunk
switchport trunk allowed vlan 10,20
no shutdown
exit
end
copy running-config startup-config

Verification

show interfaces trunk
show interfaces gigabitethernet 0/1 switchport

Part 4: Configure Router-on-a-Stick

Tasks

- Enable the physical router interface.

- Create subinterface G0/0.10 for VLAN 10.

- Create subinterface G0/0.20 for VLAN 20.

- Apply 802.1Q encapsulation.

- Assign IP addresses to each subinterface.

- Verify interface status.

Router Commands

enable
configure terminal
interface gigabitethernet 0/0
no shutdown
exit
interface gigabitethernet 0/0.10
encapsulation dot1Q 10
ip address 192.168.10.1 255.255.255.0
description Gateway_for_VLAN10
exit
interface gigabitethernet 0/0.20
encapsulation dot1Q 20
ip address 192.168.20.1 255.255.255.0
description Gateway_for_VLAN20
exit
end
copy running-config startup-config

Verification

show ip interface brief
show running-config interface gigabitethernet 0/0.10
show running-config interface gigabitethernet 0/0.20

Part 5: Configure the PCs

PC1 Settings

- IP address: 192.168.10.10

- Subnet mask: 255.255.255.0

- Default gateway: 192.168.10.1

PC2 Settings

- IP address: 192.168.20.10

- Subnet mask: 255.255.255.0

- Default gateway: 192.168.20.1

Part 6: Connectivity Testing

Test Sequence

- From PC1, ping 192.168.10.1.

- From PC2, ping 192.168.20.1.

- From PC1, ping 192.168.20.10.

- From PC2, ping 192.168.10.10.

- On the switch, verify MAC learning.

- On the router, verify subinterface status.

Useful Verification Commands

show mac address-table
show vlan brief
show interfaces trunk
show ip interface brief
ping 192.168.10.10
ping 192.168.20.10

Expected Results

- PC1 should reach gateway 192.168.10.1.

- PC2 should reach gateway 192.168.20.1.

- PC1 and PC2 should successfully ping each other after router-on-a-stick is configured correctly.

- VLAN 10 and VLAN 20 should appear in the switch VLAN database.

- The switch uplink should show trunk mode and allow VLANs 10 and 20.[cite:21][cite:26]

Troubleshooting Checklist

- Check whether the router physical interface is shut down.

- Check whether subinterfaces have the correct encapsulation.

- Check whether the switch uplink is trunking.

- Check whether the PC default gateway matches the router subinterface address.

- Check whether the access ports are in the correct VLAN.

- Check cabling and interface status using show ip interface brief and show interfaces trunk.[cite:25][cite:26]

Student Questions

- Why can hosts in different VLANs not communicate without a Layer 3 device?

- What is the purpose of encapsulation dot1Q on router subinterfaces?

- Why must the switch port connected to the router be configured as a trunk?

- What would happen if PC1 used the wrong default gateway?

- Which commands help verify VLAN membership and trunk operation?

Answer Key

Part 1 Answers

- Hostnames should be R1 and S1.

- no ip domain-lookup prevents delays from mistyped commands being resolved as DNS names.

- copy running-config startup-config saves the current configuration so it remains after reload.

Part 2 Answers

- FastEthernet 0/1 should belong to VLAN 10.

- FastEthernet 0/2 should belong to VLAN 20.

- show vlan brief should display VLAN 10 and VLAN 20 with the assigned access ports.

Part 3 Answers

- The uplink port should be GigabitEthernet 0/1 on the switch.

- The trunk should allow VLANs 10 and 20.

- show interfaces trunk should confirm trunking status and allowed VLANs.

Part 4 Answers

- Router subinterface G0/0.10 should have encapsulation dot1Q 10 and IP 192.168.10.1/24.

- Router subinterface G0/0.20 should have encapsulation dot1Q 20 and IP 192.168.20.1/24.

- The physical interface G0/0 must be up for the subinterfaces to function.

Part 5 Answers

- PC1 default gateway should be 192.168.10.1.

- PC2 default gateway should be 192.168.20.1.

Part 6 Answers

- If all configurations are correct, both PCs should be able to ping their gateways and each other.

- If PCs can ping their gateways but not each other, verify subinterfaces and trunking first.

- If a VLAN is missing from show vlan brief, the access port assignment or VLAN creation is incomplete.

Instructor Notes

This worksheet is aligned to introductory router and switch training objectives commonly practiced on Cisco ISR routers and Catalyst switching platforms used in classroom and lab environments.[cite:19][cite:28] The same workflow can be adapted to a Cisco 1811, 1900, 1921, or 2911 router with minor interface-name adjustments depending on the installed hardware.[cite:23][cite:28]