Lab Worksheet: Two Laptops to One Switch Ping Test

Lab overview

This hands-on lab has students connect two end devices to one switch, assign IPv4 addresses in the same subnet, and verify connectivity with ping. On Cisco switches, access ports carry traffic for one VLAN, and by default an access port carries traffic for VLAN 1, so two hosts connected to default access ports can communicate at Layer 2 when their IP settings are in the same subnet [cite:16][cite:23].

Lab title

Simple Lab: Two Laptops -> One Switch -> Ping Test

Expected outcome

By the end of this lab, both laptops should successfully ping each other when they are connected to the same switch, use unique IP addresses, and share the same subnet .

Learning objectives

After completing this lab, students should be able to:

- Identify the devices needed for a basic LAN.

- Connect two laptops or PCs to a switch using Ethernet cables.

- Assign IPv4 addresses and subnet masks to end devices in Packet Tracer.

- Test connectivity using the ping command.

- Explain why hosts on the same switch and same subnet can communicate without a router [cite:22][cite:25].

Devices and materials

Use the following:

- 1 switch, such as a Cisco 2960 in Packet Tracer or real hardware.

- 2 laptops or PCs.

- 2 copper straight-through cables.

- Packet Tracer, or access to a real lab bench.

Suggested topology

Use this simple topology:

Laptop0/PC0 -------- Switch -------- Laptop1/PC1

Both end devices connect to regular switch access ports. By default, Cisco access ports belong to VLAN 1 unless changed, which is suitable for this basic lab [cite:16][cite:23].

Addressing table

Use the following IP plan:

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| Laptop0 / PC0 | NIC | 192.168.1.10 | 255.255.255.0 | Not required |
| Laptop1 / PC1 | NIC | 192.168.1.11 | 255.255.255.0 | Not required |

A default gateway is not required for hosts that only need to reach another host in the same subnet because their traffic does not need to be routed [cite:22].

Pre-lab questions

Answer these before starting:

- What is the purpose of a switch in a LAN?

- Why must each host have a unique IP address?

- Why do both hosts need to be in the same subnet for this lab to work?

- Is a router required for two hosts to communicate on the same subnet? Explain.

Lab tasks

Part 1: Build the topology

- Open Packet Tracer.

- Place one switch in the workspace.

- Place two laptops or PCs in the workspace.

- Use copper straight-through cables to connect each laptop or PC to the switch.

Part 2: Configure IP addressing

- Click Laptop0/PC0.

- Open Desktop > IP Configuration.

- Configure the following:

  - IP address: 192.168.1.10

  - Subnet mask: 255.255.255.0

- Click Laptop1/PC1.

- Open Desktop > IP Configuration.

- Configure the following:

  - IP address: 192.168.1.11

  - Subnet mask: 255.255.255.0

In Packet Tracer, IP addressing for a PC can be set from Desktop > IP Configuration, and connectivity can then be tested from the command prompt with ping [cite:25].

Part 3: Verify physical connectivity

Check the link lights on both switch ports.

- Are both links up?

- If no, check cable type and port connection.

Part 4: Test connectivity with ping

- Open Laptop0/PC0.

- Go to Desktop > Command Prompt.

- Enter:

ping 192.168.1.11

- Record the result.

- Open Laptop1/PC1.

- Go to Desktop > Command Prompt.

- Enter:

ping 192.168.1.10

- Record the result.

Packet Tracer labs commonly verify host communication by sending ICMP echo requests with the ping command after assigning valid IP settings [cite:25].

Student results table

| Test | Source Device | Destination IP | Success/Fail | Notes |
| --- | --- | --- | --- | --- |
| Ping 1 | Laptop0 / PC0 | 192.168.1.11 |  |  |
| Ping 2 | Laptop1 / PC1 | 192.168.1.10 |  |  |

Why it works

This lab works because both hosts are attached to the same switch and are in the same Layer 2 broadcast domain through default access-port behavior on VLAN 1 unless the switch configuration has been changed [cite:16][cite:23]. Since both hosts use addresses in the same IPv4 subnet, they can communicate directly without sending traffic to a default gateway [cite:22][cite:27]. Each host also needs a unique IP address so the network can identify the correct destination device [cite:25].

Common mistakes

If the ping fails, check these items:

- Incorrect IP address on one host.

- Incorrect subnet mask on one host.

- Duplicate IP address.

- Wrong cable or disconnected cable.

- Port not active.

- Host firewall enabled on real laptops.

- Ports placed in different VLANs on a non-default switch configuration.

When hosts on the same VLAN and same subnet cannot ping, common causes include incorrect subnet masks, host filtering, or the hosts not actually being in the same broadcast domain [cite:24][cite:14].

Reflection questions

- What would happen if one laptop used 192.168.2.11 instead?

- What would happen if both laptops used the same IP address?

- Why is a switch enough for this lab, but not enough for communication between different subnets?

- What command did you use to test connectivity?

Instructor notes

Possible extensions for the next activity:

- VLAN version of this lab, place each PC in a different VLAN and observe that ping fails until proper Layer 3 routing is added.

- Router-on-a-stick version, use a router and trunk link for inter-VLAN communication.

- Troubleshooting version, intentionally misconfigure IP addressing, VLAN membership, or cabling and have students diagnose the fault.

Completion checklist

- Topology created.

- Both devices connected to the switch.

- IP addresses assigned correctly.

- Subnet masks assigned correctly.

- Ping from Laptop0/PC0 successful.

- Ping from Laptop1/PC1 successful.

- Reflection questions completed.